ANNEX A - Specifications

UNHCR UKRAINE Invitation to Bid ITB 2021-02

for procurement and supply of construction materials for the purpose of rehabilitation (repair) of 90 damaged private houses in Donetsk region, NGCA

- QUANTITY AND SPECIFICATION OF PRODUCTS

Bids should confirm ability to supply goods to the following quantities by category. The offer can be provided and evaluated for whole range within the category:

| # | Product category | Specification | Unit of measure | Appr. annual q-ty |
| --- | --- | --- | --- | --- |
| LOT #1. Construction materials |  |  |  |  |
| 1 | Roofing sheet | Dimensions: 1130х1750 mm Material: Asbestos free cement (meeting European quality standards) Corrugation: Compliance with GOST 30340-95 | Pcs (sheet) | 4291 |
| 2 | Ruberoid | Size: 1х15 m Weight 1 sq.m of material: not less than 1.5 kg Basis: Fiberglass Breaking tensile strength, not less than: 294 H Water absorption for 24 hours, by weight, not more: 1.50 % The temperature flexibility on a bar R = 25 mm: Not above: -5 degrees. Heat resistance, at least: 85 degrees The potential lifetime: 10 years+ Waterproof at a pressure of 0,001 MPa for 72 hours: absolutely Certificares: Hygienic, fire and radiation Certificate of conformity: UkrSEPRO | roll | 514 |
| 3 | Skate galvanized | Size: 200х200х2000 mm | lineal m | 736 |
| 4 | Roofing nails | 120 mm | kg | 493 |
| 5 | Nail | 3 x 70 mm | kg | 150 |
| 6 | Nail | 5 x 120 mm | kg | 49 |
| 7 | Nail | 100 mm | kg | 83 |
| 8 | Nail | 200 mm | kg | 15 |
| LOT #2. Timber |  |  |  |  |
| 1 | Timber-batten 25х100х4500 mm | 25х100х4500 mm Material: pine or fir tree, density P<0.5 g/cm3 Quality of drying/humidity %: CAT III / no more than 15% With antibacterial treatment Deviation size corresponding GOST 25346-89: IT (13-17) Compliant with radiological control Content of Cs-137 Bq/kg: no more 740 The activity of Cs-137Bq/kg: no more 5 Strength Class: C 24 acc. To MG B 52001 Certificate of origin, Phytosanitary certificate, Conclusion of the State sanitary-epidemiological expertise for applied antiseptic | pcs | 13309 |
| 2 | Timber beam 70х100х4500 mm | Size: 70х100х4500 mm Material: pine or fir tree, density P<0.5 g/cm3 Quality of drying/humidity %: CAT III / no more than 15% With antibacterial treatment Deviation size corresponding GOST 25346-89: IT (13-17) Compliant with radiological control Content of Cs-137 Bq/kg: no more 740 The activity of Cs-137Bq/kg: no more 5 Strength Class: C 24 acc. To MG B 52001 Certificate of origin, Phytosanitary certificate, Conclusion of the State sanitary-epidemiological expertise for applied antiseptic | pcs | 421 |
| 3 | Timber beam 70х150х4500 mm | Size: 70х150х4500 mm Material: pine or fir tree, density P<0.5 g/cm3 Quality of drying/humidity %: CAT III / no more than 15% With antibacterial treatment Deviation size corresponding GOST 25346-89: IT (13-17) Compliant with radiological control Content of Cs-137 Bq/kg: no more 740 The activity of Cs-137Bq/kg: no more 5 Strength Class: C 24 acc. To MG B 52001 Certificate of origin, Phytosanitary certificate, Conclusion of the State sanitary-epidemiological expertise for applied antiseptic | pcs | 456 |
| 4 | Timber beam 100x200x4500 mm | Size: 100х200х4500 mm Material: pine or fir tree, density P<0.5 g/cm3 Quality of drying/humidity %: CAT III / no more than 15% With antibacterial treatment Deviation size corresponding GOST 25346-89: IT (13-17) Compliant with radiological control Content of Cs-137 Bq/kg: no more 740 The activity of Cs-137Bq/kg: no more 5 Strength Class: C 24 acc. To MG B 52001 Certificate of origin, Phytosanitary certificate, Conclusion of the State sanitary-epidemiological expertise for applied antiseptic | pcs | 102 |

With purpose of project implementation on rehabilitation of damaged private houses in the non-Governmentally Controlled areas (NGCA) of Donetsk region, the successful bidder is required to offer unit cost DAP Donetsk and additionally indicate the delivery cost at the rate of a ton / km, by route: from the city of Donetsk to any other place of destination in Donetsk region, NGCA.

The final delivery point will be indicated in each Purchase Order issued by UNHCR.

| # | Product category | Specification | Unit of measure | Appr. annual q-ty |
| --- | --- | --- | --- | --- |
| LOT #3. Windows/Doors (manufacture +installation) |  |  |  |  |
| 1 | Glass unit/glazing | Single chamber 4-16 (24) -4 polishing М-1 | m2 | 9.6022 |
| 2 | Double-casement window | 1-blind, 1 opening + ventilation, three chamber profile, installation depth from 58 mm, mechanism - rotary-folding, glass unit - single chamber polishing М-1 | m2 | 375.4236 |
|  | Triple-casement window | 2-blind,1 opening + ventilation, three chamber profile, installation depth from 58 mm, mechanism - rotary-folding, glass unit - single chamber polishing М-1 |  |  |
|  | Single-casement window | Opening + ventilation, three chamber profile, installation depth from 58 mm., mechanism - rotary-folding, glass unit - single chamber polishing М-1 |  |  |
|  | Single-casement window blind | Three chamber profile, installation depth from 58 mm, glass unit - single chamber polishing М-1 |  |  |
|  | Balcony door single-casement | Three chamber profile, installation depth from 58 mm, mechanism - rotary-folding, glass unit - single chamber polishing М-1 |  |  |
| 3 | Window ledge | Plastic, white | m | 87.774 |
| 4 | Window sill | Galvanized, white | m | 67.102 |
| 5 | Metal sheet | Steal, thickness: 0,45 mm | m2 | 51.9 |

In addition to LOT №3: the second mandatory condition of ITB is the supply of plastic windows / doors with installation, therefore, bidders are required to offer unit cost DAP Donetsk (goods + installation), additionally indicate the delivery cost at the rate of a m3/km, by route: from the city of Donetsk to any other place of destination in Donetsk region, NGCA.

Please note that quantities of goods stated in Specification are to enable bidders to have an indication of the projected requirements. It does not represent a commitment that UNHCR will purchase the above quantity. Quantities may vary and will depend on the actual requirements and funds available regulated by issuance of individual Purchase Orders against the Frame Agreement. In this regard, please indicate prices that will be effective regardless of the ordered volume of goods.

B - PACKAGING, LABELLING FOR TRANSPORTATION

The bidder shall describe the proposed packaging for transportation and defining a packing unit (PU). The quantity of items in each PU must be standardized and this information must be clearly specified along with its weight, volume and the equipment required to handle it in the technical offer.

C- PRODUCTION AND DELIVERY CAPACITY

The bidder shall state its annual production capacity and the quantities available after one week of production lead-time. The production capacity will be compared and measured against the offer with the highest production capacity.

The bidder shall state existing stock quantity available for immediate dispatch and the delivery time. Also the time of availability of the full initial requirements quantity shall be clearly stated. Time, conditions and limitations of such delivery shall also be stated. Additional points of delivery can be subject to agreement during the contract implementation.

D – WARRANTY

The bidder must indicate the warranty period for construction materials and services. The minimum warranty period is 12 months.

The successful bidder may receive an order for supply of all volume of Goods or part to any place of destination in Donetsk region, NGCA.

Please note that list of places of destination is not limited. Additional delivery points may be agreed during the term of the Frame Agreement.