ANNEX B

In reference to the ITB/2022/COT/001, please see below our company's TECHNICAL PROPOSAL.

- TECHNICAL SPECIFICATIONS (Please make particular mention of any divergence to the requested specifications, indicating if these are considered equivalent or better than the requested specifications.)

| DESCRIPTION | MEETS REQUIREMENTS |  | YOUR PROPOSAL |
| --- | --- | --- | --- |
|  | YES | NO |  |
| Multi-functional solar lamp with phone charger - An “all-in-one” solar lantern and cell phone charger - Task light – when the lamp shade serves as a table stand - Ceiling light / lantern – when the lamp head is inserted into the lamp shade - Portable light / neck light – when the light unit and battery pack is detached from the lamp - High quality DC/DC USB adaptor that can be used with any mobile phone with a USB charging cable - 1 LiFePO battery, 1.400 mAh / 3.2V with 2.000+ cycles (5-7 years) (or equivalent) - 3 High quality LEDs with adjustable light from 5 to 50 to 150 lumen, 5.500-6.000 kelvin (or equivalent) - 1.8 W Solar panel with 3m/9ft cable rolled up in solar disc (or equivalent) - Color of outside case: white and blue - Light - High light output: 150 lm for 4.5+ hours - Medium light output: 50 lm for 14+ hours - Low light output: 5 lm for 130+ hours - LED lifespan: 50,000 hours - Light distribution: 120 degrees - Color: cool white (5.500 – 6.000 kelvin) - Automatic dimming to lower modes at low voltage - Charging and Battery - Solar panel with loops for fixture: 1.8 W with hardened glass cover (or equivalent) - Charging time PV: 3-8 hours depending on how much sun is available - Charging time on AC grid with adapter: 1.5 hours - Battery spec: 1 LifePO Battery, 1.400 mAh / 3.2V (or equivalent) - Battery protection: self-discharge, reverse polarity, trickle charge - Other Product Facts - Factory certification: ISO9001 / ISO4001 / ILO standards |  |  |  |

- COMPANY PROFILE

Included in the Technical Offer folder is our company profile.

Yes      No

- AVAILABILITY OF GOODS (please indicate quantity and immediate date of availability of goods)

| Item Description | Qty required |  | Qty offered | Availability |
| --- | --- | --- | --- | --- |
| Solar lamp with phone charger | 2,000 |  |  |  |

- SHELF LIFE

________________________________________________________________________

- ANNUAL PRODUCTION CAPACITY

________________________________________________________________________

- WARRANTY PERIOD

________________________________________________________________________

- VALIDITY OF PROPOSAL (preferably 60 days after the deadline of submission of proposals)

Less than 60 days, please indicate: _________________

60 days

Until 31 December 2022

- PAYMENT TERMS

UNHCR’s standard payment term is within 30 days upon receipt of final billing statement. Do you agree?    Yes      No

- DELIVERY

Place of delivery is at MCMIC Cargo Express (2Go Warehouse), Brgy Salimbao, Sultan Kudarat 7° 14’ 4.56” N 124° 15’ 47.88” E

Free of cost

With delivery charge (DO NOT indicate amount)

- VENDOR REGISTRATION FORM (properly filled-up, see Annex D)

- I have read and understood the UNHCR General Conditions of Contracts for the Provision of Goods & Services, July 2018 edition, Annex C:

Yes       No

Submitted  by:

(signature)       .

Name