#### RFP NO. 623783-AR CIRCULAR PLATE SAW

#### TECHNICAL PROPOSAL COMPLIANCE MATRIX

- A technical proposal compliance matrix shall be completed for the provision of Circular Plate Saw for the IAEA Mechanical Workshop, in Seibersdorf, Austria.

- Note: This is a summary document only. Bidders must provide detailed evidence outlining how they meet the Specification as per section “Content of Technical Proposal” of the document Special Instructions to Bidders.

| SUMMARY OF TECHNICAL COMPLIANCE |  |  |  |
| --- | --- | --- | --- |
| Specification Ref. | Specification Requirements | Compliant Yes/No | Bidder’s comments and reference to justification of compliance |
| 2 | Requirements |  |  |
| 2.1 | The Goods shall meet specific functional and performance requirements. The Goods shall: |  |  |
| a) | be able to cut plates to a thickness of 30mm; |  |  |
| b) | not have dimensions greater than the following: i. Length, 1950mm; ii. Width, 1900mm; and iii. Height, 1150mm. |  |  |
| c) | be able to cut plates with dimensions up to 2000mm x 1000mm x 30mm; |  |  |
| d) | be able to cut aluminium, PVC and PE; |  |  |
| e) | have a waste collector on wheels; |  |  |
| f) | have an emergency stop button on display; |  |  |
| g) | have an electrical connection 400V, 50Hz, 3,4kVA; |  |  |
| h) | be able to control the speed of the saw blade; |  |  |
| i) | have a spraying device to clean the saw blade; |  |  |
| j) | have a guard protecting the saw blade during operation; |  |  |
| k) | have a digital measuring system; |  |  |
| l) | have special grooving stones for plate placement; |  |  |
| m) | have 10 spare blades; and |  |  |
| n) | be CE certified; |  |  |
| 2.2 | The Contractor shall provide an operation manual and maintenance plan schedule. |  |  |
| 3 | Marking |  |  |
|  | The Goods shall have all safety markings in English or German language. |  |  |
| 4 | Packing |  |  |
|  | The Goods, for shipment to the IAEA, shall be packed in accordance with international standards that are applicable for the shipment of this kind of equipment. |  |  |
| 5 | Quality Requirements |  |  |
| 5.1 | The Goods shall be manufactured, shipped and installed in accordance with the Contractor’s ISO quality assurance system or an equivalent quality assurance system. |  |  |
| 5.2 | The Contractor shall document the compliance with this quality assurance system. |  |  |
| 6 | Testing and Acceptance |  |  |
| 6.1 | The Goods, prior to shipment, shall be tested for conformance of the Goods with manufacturer’s performance specifications and the minimum requirements specified herein. |  |  |
| 6.2 | The Goods, after installation, shall be tested by the Contractor together with the IAEA to demonstrate that the performance meets the manufacturer’s performance specifications and the minimum requirements specified herein as determined by the IAEA. |  |  |
| 6.3 | The results of the testing of the Goods shall be documented by the Contractor in an acceptance protocol that shall be signed by the IAEA. |  |  |
| 7 | Installation and Training |  |  |
| 7.1 | The Contractor shall install the Goods at the IAEA’s Laboratories in Seibersdorf. |  |  |
| 7.2 | The Contractor shall provide one (1) day training for up to six (6) staff of the IAEA in the operation and maintenance of the Goods at the IAEA’s location immediately after the installation of the Goods. |  |  |
| 8 | Deliverable Data Items |  |  |
|  | The Contractor shall provide two (2) complete sets of operation and servicing manuals and technical drawings in the English language. |  |  |

___________________________________________________________________________

___________________________________________________________________________