UNITED NATIONS OFFICE FOR PROJECT SERVICES

Yemen Emergency Human Capital Project (YEHCP), Yemen Integrated Urban Service Emergency Project II (YIUSEP II) and Co-financing.

Rehabilitation of Wastewater Pump Stations and Main Trunk Line at Several Places in the City (Crater, Malla'a, Tawahi, and Khormaksar)

Environmental and Social Management Plan

# SEPTEMBER 28 , 2022

Contents

Abbreviations	4

1.	Introduction	8

2. Sub-Project Description	11

2.1 Scope of Work	11

2-	Rehabilitation Plan	22

Rehabilitation of Force Mains	22

2.1	Rehabilitation of Pumping Stations	24

2.2	Rehabilitation Works for Pumping Station	26

2.3	Section Two: Population forecasting & Wastewater Generation	34

2.4	Population Growth	34

2.5	Planning Period	34

2.6	Projected Population	34

2.7	Generated wastewater	34

2.8	Peak Factors	36

World Bank Requirements Relevant to the project	38

Environment, Health and Safety Guidelines	38

COVID-19 Sensitivity	38

Gender and Social -related issues:	38

Annex 2: GRM Complaint and Suggestion Form	76

Annex 3:  Environmental and Social Requirements for Contractors	79

Contractor Environmental and Social Management Plan (C-ESMP)	79

ESHS Training	79

Orientation Training	80

Visitor Orientation	80

New Task Employee and Contractor Training	80

Construction Site Management	80

Vegetation	80

Protection of the Existing Installations	81

Waste from Construction Activities	81

Air Quality	82

Hazardous and Toxic Materials	82

Area Signage	83

Decommissioning of Camps, Worksites and Plant	83

Health and Safety	83

Severe Weather and Facility Shutdown	83

Lavatories and Showers	83

Potable Water Supply	83

Clean Eating Area	83

Personal Protective Equipment (PPE)	84

Noise	84

First Aid and Accidents	84

Communicable Diseases	85

COVID-19	86

Vector-Borne Diseases	86

Road safety and Traffic Safety	87

Emergencies	88

Stakeholder Engagement	88

Labor Conditions	89

Insurance	89

Grievance Mechanism for Workers	89

Protection from Sexual Exploitation and Abuse	90

Protection from Child Labor	90

Code of Conduct	90

Annex 5: Consultation Questionnaires	94

# Abbreviations

| AC | Alternative Current |
| --- | --- |
| CE | European Union’s (EU) mandatory conformity |
| DC | Direct Current |
| DCJB | DC Junction Box |
| DOD | Depth of Discharge |
| EHS | Environmental, Health and Safety |
| ESF | Environmental and Social Framework of the World Bank |
| ESHS | Environment, Social (including labor), Health, and Safety |
| ESMF | Environmental and Social Management Framework |
| ESMP | Environmental and Social Management Plan |
| ESS | Environmental and Social Standard |
| FMFA | Financial Management Framework Agreement |
| FCV | Fragility, Conflict and Violence |
| GBV | Gender Based Violence |
| GHS | General Health and Safety guidelines |
| GIIP | Good International Industry Practice |
| GM | Grievance Mechanism |
| GRM | Grievance Redress Mechanism |
| GSM | Global System for Mobile Communication |
| HSSE | Health, Safety, Social and Environment |
| IDA | International Development Association |
| IDP | Internally Displace Person |
| LCD | Liquid Crystal Display |
| LED | Light Emitting Diode |
| LMP | Labor Management Procedures |
| MCB | Miniature Circuit Breakers |
| MCCB | Molded Case Circuit Breaker |
| MPPT | Maximum Power Point Tracking |
| PAP | Project Affected People |
| PV | Photovoltaic |
| PVC | Permanent virtual circuit |
| RF | Resettlement Framework |
| ROY | Republic of Yemen |
| SEA | Sexual Exploitation and Abuse |
| SH | Sexual Harassment |
| SEP | Stakeholder Engagement Plan |
| SMP | Security Management Plan |
| SPD | Surge Protector Device |
| TPM | Third Party Monitoring |
| UL | Underwriters' Laboratories |
| UNOPS | United Nations Office for Project Services |
| UV | Ultraviolet |
| XLPE | Cross linked polyethylene cable. |
| YEHCP | Yemen Emergency Human Capital Project |

Executive Summary

This Environmental and Social Management Plan (ESMP) was prepared based on UNOPS Environmental and Social Management Framework (ESMF) of the Yemen Emergency Human Capital Project (YEHCP), Yemen Integrated Urban Service Emergency Project II (YIUSEP II), and other financing to address the potential environmental and social impacts, which may be triggered during the implementation of the sub-project of rehabilitation of wastewater pump stations and main trunk line at several places in the city (Crater, Malla'a, Tawahi, and Khormaksar) in Aden City that will be implemented by UNOPS.

The selection of activities was based on the priority needs and criteria identified by UNOPS in consultation with District Local Authorities DLAs, Local Communities and Aden Water and Sanitation Local Corporation AWSLC. The rehabilitation work contract will maximize the use of manual labor to support local employment.

This ESMP defines the potential impacts due to the rehabilitation activities and their proposed mitigation measures that would be the direct responsibility of contractor to implement under the supervision of UNOPS. Arrangement with Aden Water and Sanitation Local Corporation AWSLC for the required rehabilitation and maintenance of pump stations and network of Crater, Malla'a, Tawahi, and Khormaksar area was considered by UNOPS.

Proposed activities:

The proposed activities will include rehabilitation of the pump stations and the sewer trunk lines by the construction and of a trunk sewer lines in parallel to the old trunk, this rehabilitation work will help to avoid the direct disposal of raw sewage to the sea and avoid wastewater flooding into streets/neighborhoods and the proliferation of cholera, odor, and environmental pollution.

Aden Water and Sanitation Local Corporation is a governmental institution that has the Legal entity and all its assets are considered part of its capital.

The Aden Water & Sanitation Local Corporation was established by Republican Decree no. (267) in 2000, as the public body responsible for the provision water supply and sanitation services in Aden city.

Main activities to be implemented during the execution of the project are:

- Excavation /leveling/ filling/pipe sand bedding;

- Dewatering;

- Compaction and Backfilling;

- Cut, remove and re-asphalt in some cases, if needed;

- Cut, remove and re-pave with stones; if needed;

- Manhole work/rehabilitation/backdrop;

- Pump stations construction and rehabilitation;

- Connecting the new sewer lines with the pump stations;

- Connecting the new force main line with the treatment plant (Al-Areesh);

- Taking care and rehabilitation of other underground services connections if needed;

- Etc.,

Activities will include pumping work, sewers repairs and constructions, safety improvement works, and rehabilitation.

The work will take place partly by labor and by machines. The expected number of labors is less than 20 at one time.

Scope and magnitude of environmental and social impacts

The identified potential impacts are expected to have temporary effect during the rehabilitation work. No major potential environmental or social impacts are expected, but only minor impacts are associated with sites preparation, earth works and waste generation, all of which are localized with reversible impacts. The potential negative impacts are:

- Increased level of noise, emission and dust during preparation/leveling/excavation of sites.

- Generation and disposal of waste and debris.

- Safety of labor and community during work implementation on and around sites.

- Temporary traffic disturbance of pedestrian and neighbor households during work implementation. However, the shops will continue opening where work will be implemented in section by section in one line only, and after completing the last section work will start in the other section. For mobile petty traders they are not fixed in one place and usually market their goods in more than one street and will continue their businesses. In addition, implementation of rehabilitation work will be when both formal and informal businesses are not active.

The mitigation measures proposed to match the potential negative impacts are:

- Use of well-maintained equipment and machinery;

- Spraying of water during site preparation, excavation and backfilling works;

- Restriction of use of noisy equipment to the day time business hours only;

- Provide Personal Protection Equipment (PPEs) as well as occupational health and safety training to all workers;

- Properly collect, transport and dispose of solid waste at designated permitted site of landfill assigned by the local authorities and the Cleaning Fund in Aden;

- Alternative routes for traffic, install sheet piles pass way for pedestrians and neighbor households; shortening the period of execution; and implementing the sub-project in sections in a consecutive way (not opening/ excavating all sections at the same time).

Environmental and Social Management Plan (ESMP) table was prepared to summarize the findings of the study including the key potential impacts, proposed mitigation of potential negative impacts, implementation responsibility.  Clauses for contractor will be included in the tender documents and the contract of the subproject and will be monitored by UNOPS during work implementation.

UNOPS usually request specific progress report from the site engineer and supervisor for all work including the safeguard measures implementation, with documentation submitted by contractor and approved by the site engineer on the implementation of environmental and social safeguards and OHS requirements to be provided by the contractor as attachment with each installment. That will be in addition to the regular monitoring visits by UNOPS team with submission of frequent monitoring reports.

Environmental and Social Monitoring Plan was prepared to specify monitoring arrangements, frequency and responsibilities. The plan will allow effective monitoring of the contractor’s performance against the environmental and social mitigation and OHS measures. As well as the contractors monitoring.

Consultations was conducted by UNOPS and UW-PMU- Aden social facilitators in Aden. And the consulted people were provided with hard copies of the translated GRM Form were provided to interviewed people to ensure that any grievance can be addressed in an amicable manner. Resolving complaints at community level which is always encouraged to address the problem that a person may have during implementation and/ or operation phases.

Environmental and Social Screening Form for the Sub-Project was prepared to screen possible environmental and social impacts and corresponding mitigation measures were addressed.

| Sub-Projects Name | Rehabilitation of Wastewater Pump Stations and Main Trunk Line at Several Place in the City (Crater, Malla'a, Tawahi, and Khormaksar) |
| --- | --- |
| Sub-Project Location | Aden |
| Implementing Partner | UW-PMU-Local Contractor |
| Risk level | Moderate |
| Date of the field visit |  |
| Date of consultation |  |
| Observations/Comments: | Indicated below |
| Signature of ESSO: |  |
| Date : |  |

# Introduction

The Yemen Emergency Human Capital Project (YEHCP), Yemen Integrated Urban Service Emergency Project II (YIUSEP II), and other Funds aim to support the provision of Water Supply and Sanitation (WSS) services for the population of Yemen through rehabilitation of medium to large WSS infrastructure, response to COVID-19 and flash floods and strengthening the capacity building of the local water and sanitation institutions at decentralized level. YEHCP and co-financing support public health improvements and rehabilitation of the water and sanitation services, where the WASH interventions designed under this sub-component will include climate- resilience measures such as flood drainage interventions to help reduce the impact of the flash flood events. Solar energy will also be used to operate key water facilities, providing safe and clear water, with lower emissions.

The implement projects will help preserve and strengthen the WSS system through supporting, inter alia, procurement and contract management, low carbon and climate resilient infrastructure, technical design, asset management, O&M of WSS facilities for medium and long term, information management, safeguard and leadership capacities of local water and sanitation institutions, etc.

The Environmental and Social Management Plan (ESMP) for rehabilitation of wastewater pump stations and main trunk lines at several places in the city (Crater, Malla'a, Tawahi, and Khormaksar) in accordance with the Environmental and Social Management Framework (ESMF) for YEHCP and co-financing. The ESMF was prepared by UNOPS to meet the requirements of the World Bank’s Environmental and Social Framework (ESF), UNOPS requirements and the national environmental laws and regulations requirements.

The Yemen Emergency Human Capital Project (YEHCP) and co-financing ESMF will guide UNOPS and its implementing partners to ensure that all subprojects are in accordance with the ESF requirements, including the preparation and implementation of subproject specific Environmental and Social Management Plans (ESMP). For this purpose, the ESMF details how UNOPS will screen each subproject to assess its environmental and social risks and impacts, identify the mitigation measures, and monitor ESMP implementation, most particularly the environmental and social performance of project contractors.

The Yemen Emergency Human Capital Project YEHCP, Yemen Integrated Urban Service Emergency Project II (YIUSEP II), and other financing aim at sustaining basic service delivery while progressively supporting economic and human capital development and further strengthening public service delivery mechanisms aimed at improving health and nutrition service delivery while also stepping up focus on restoring water and sanitation services at selected urban, pre‐urban and rural areas.

The Environmental and Social Management Framework (ESMF) for YEHCP, YIUSEP II, and co-financing was prepared by UNOPS to meet the requirements of the World Bank’s Environmental and Social Framework (ESF), UNOPS requirements and the national environmental laws and regulations requirements.

The only relevant ESSs for this subproject are ESS1, ESS2, ESS3, ESS4, and ESS10.  As a result, this subproject will follow the requirements of the LMP for labor working conditions and OHS, the GBV action plan for any GBV issues, the SMP to manage any potential security risks, and SEP for consultation and information disclosure.

This ESMP has been prepared also in order to ensure that the proposed sub-project incorporates sound environmental and social management principles and practices and complies with World Bank environmental and social standards (ESSs), the general WBG EHS guidelines, and Sector-based EHS guidelines; Water and Sanitation, Electric Power Transmission and Distribution as well as with the applicable environmental policies and legal requirements of the Republic of Yemen (RoY).

The objective of the ESMP is to identify the environmental and social impacts of the proposed subprojects and to specify appropriate preventive actions and mitigation measures to prevent, eliminate or minimize any anticipated adverse impacts. The ESMP is the mechanism that ensures that the environmental prevention and mitigation measures identified are properly undertaken during the implementation of the proposed sub-projects.

SUMMURY YEHCP AND CO-FINANCING GOALS:

The Yemen Emergency Human Capital Project (YEHCP) and co-financing funds aims are:

- To Improve access to Water Supply and Sanitation (WSS) and strengthening local systems.

- To support the provision of WSS services for the population of Yemen through rehabilitation of medium to large WSS infrastructure, response to COVID-19 and flash floods and strengthening the capacity building of the local water and sanitation institutions at decentralized level.

- To support public health improvements and rehabilitation of the water and sanitation services, where the WASH interventions designed under this sub-component will include climate- resilience measures such as flood drainage interventions to help reduce the impact of the flash flood events. Solar energy will also be used to operate key water facilities, providing safe and clear water, with lower emissions.

- The projects will help preserve and strengthen the WSS system through supporting, inter alia, procurement and contract management, low carbon and climate resilient infrastructure, technical design, asset management, O&M of WSS facilities for medium and long term, information management, safeguard and leadership capacities of local water and sanitation institutions, etc.

Figure 1- Proposed Packages

2. Sub-Project Description

# 2.1 Scope of Work

During the last 10 years, the sewerage system suffered many problems due to the frequent collapses of the lines and the frequent malfunctions of the pumping stations, which led to the overflow of sewage. To deal with this, sewage was pumped directly to the sea causing great environmental damage to the coastline. For those reasons, the need for rehabilitation became more pressing.

At the previous stages of this project, two scenarios were proposed regarding the improvement/replacements of the existing sewage system at Aden. Scenario 2 was selected by the Ministry of Water and Environment, and LWASCA. At this scenario, it was proposed to connect “Crater Pumping Station” and “Ma’alla Pumping Station” directly to the force main with diameter of 1000 mm; the whole sewage from Crater & Ma’alla Pumping Stations will be pumped directly to Al Areesh Sewage Treatment Plant (STP) instead of pumping to Khormaksar Pumping Station as was done at the previous system. While the sewage from the old city of Khormaksar will be pumped via “Khormaksar Pumping Station” to Al Areesh Sewage Treatment Plant through 1000 mm force main.

The main objective of the sub-project is to restore sanitation service and prevent the environmental pollution due to the direct disposal to the sea and the overflow of raw sewage in the streets and neighborhood with the consequences of spread of cholera outbreak and other diseases caused by wastewater influent flooded in the streets. The proposed activities will be mainly rehabilitation and maintenance of selected critical sewerage network and rehabilitation of the sewage trunk lines by the implementation of a new force main lines in parallel to the old force main in the four districts in Aden (Crater, Malla'a, Tawahi, and Khormaksar). As follows:

- PK0 : Khormksar :

- Supply and laying new Force Main pipelines of HDPE pipes of 630mm OD and 710mm OD and
ductile pipe line of 1000mm including all related fittings, valves, and connections.

- Construction of chambers, for air valves, butterfly valves, as may be required.

- Construction all related works for interconnecting the new lines at connection-2.

- Construction all related works for connection-1 interconnecting the new 710mm line.

- Due to site situation, parts of the existing pipeline will be removed and replaced by the new
pipeline at the same rout.

- Rehabilitate the pump station. This includes the civil, mechanical and electrical works. This
includes the pumps room building and the surrounding service buildings and yards.

- Mechanical and electrical works. This includes replacing the old pumps by new pumps, including
the piping system and the related electrical works. Replacing the existing overhead crane by
new one.

2- PK-I-CRATER

- Supply and laying new Force Main pipeline of HDPE pipes of 560mm OD including all related
fittings, valves, and connections.

- Construction of chambers, for air valves, butterfly valves, as may be required.
Construction all related works for interconnecting the new 560 mm diameter pipe at
connection-1.

- Due to site situation, parts of the existing pipeline will be removed and replaced by the new
pipeline at the same rout.

- Rehabilitate the pump station. This includes the civil, mechanical and electrical works. This
includes the pumps room building and the surrounding service buildings and yards.

- Mechanical and electrical works. This includes replacing the old pumps by new pumps,
including the piping system and the related electrical works. Replacing the existing overhead
crane by new one.

3- PK-II-Khormaksar Force Main

- Supply and laying new Force Main pipeline of ductile iron pipes of 1000mm ID including all
related fittings, bends, tees, valves, and connections.

- Construction of chambers, for air valves, gate valves, as may be required.
Construction all related works for interconnecting the new 1000 mm diameter pipe at connection-
3.

- Construction all related works for interconnecting the new 1000 mm diameter pipe at WWTP bell
mouth inlet.

- Due to site situation, parts of the existing pipeline will be removed and replaced by the new
pipeline at the same rout.

4- PK-III-TAWAHI-MA'ALLA

- Supply and laying new Force Main pipelines of HDPE pipes of 450 mm OD and 630mm from
Tawahi Pumping Station to Maala Pumping station to connection-1. including all related fittings,
valves, and connections.

- Construction of chambers, for air valves, valves, as may be required.

- Construction all related works for interconnecting the new line 630mm diameter pipe at
connection-1.

- Due to site situation, parts of the existing pipeline will be removed and replaced by the new
pipeline at the same rout.

- Rehabilitate the two pump stations. This includes the civil, mechanical and electrical works. This
includes the pumps room building and the surrounding service buildings and yards.

- Mechanical and electrical works. This includes replacing the old pumps by new pumps, including
the piping system and the related electrical works. Replacing the existing overhead crane by
new one.

The sub-project is selected based on UNOPS sustainability criteria, in consultation with Local authorities and communities and in coordination with Aden Water and Sanitation Local Corporation. Contractor will implement the activities and maintain all requirements for optimal operation and safety. The Contractor may need support for some activities of this ESMP such as coordination with other services providers and local authorities. Such arrangements will be supported by Water and Sanitation Local Corporation in Aden with UWS/PMU and UNOPS Aden City Engineer. Contractor will conduct all works using manual labor that reside in the areas where the works are conducted. Thus, contractor is not expected to build or operate residential labor camps to host such workers.

The contractor is responsible for fulfilling all Environmental and Social safeguards as listed in the BOQs, environmental and social clauses of contractor and EHS clauses for contractor. UNOPS will monitor the contractor’s work by regular monitoring visits to the sites by Aden City Engineer and UNOPS site engineer / supervision.

UNOPS in coordination with UWS-PMU and Aden Water and Sanitation Local Corporation and with collaboration with other organizations such as Arab Fund for Economic & Social Development will ensure that the amount of sewage will be properly disposed of in the wastewater treatment plant.

Table 1: Existing force main and pumping stations at 2021.

| From | To | Length (m) | Diameter (mm) | Capacity pumping flow (L/sec) |
| --- | --- | --- | --- | --- |
| Crater PS | Khormaksar PS | 3780 Ductile Iron Pipe | 500 | Two pumps with total flow of 320 (each pump flow 160 l/sec) |
| Tawahi PS | Gravity Manhole M1A* | 2073 Ductile Iron Pipe | 400 | Two pumps with total flow of 204 (each pump flow 102 l/sec) |
| Ma’alla PS | Khormaksar PS | 3908 Ductile Iron Pipe | 600 | Two pumps with total flow of 580 (each pump flow 290 l/sec) |
| Khormaksar PS | Al–Areesh Sewage (STP) | 9670 Ductile Iron Pipe | 1000 | Two pumps with total flow of 1410 (each pump flow 705 l/sec) |

*Note: the pipe line segment between Tawahi and Ma'alla (from manhole M1A to Ma'alla PS) is working under gravity.

Table 2: Proposed analysis and improvement of the force main and pumping stations at 2046.

| From | To | Length (m) | Diameter (OD) & Material |
| --- | --- | --- | --- |
| Crater PS | Connection 1 | 3473 | 560 HDPE (ID: 493.6 mm) |
| Connection 1 | Connection 2 | 273 | 710 HDPE (ID: 625.8 mm) |
| Connection 2 | Al–Areesh Sewage (STP) | 9477 | 1000 DIP |
| Tawahi PS | Gravity Manhole M1A | 2100 | 450 HDPE (ID 396.6 mm) |
| Ma’alla PS | Connection 1 | 3675 | 630 HDPE (ID: 555.2 mm) |
| Connection 1 | Connection 2 | 273 | 710 HDPE (ID: 625.8 mm) |
| Connection 2 | Al–Areesh Sewage (STP) | 9477 | 1000 DIP |
| Khormaksar PS | Connection 2 | 255 | 630 HDPE (ID: 555.2 mm) |
| Connection 2 | Al–Areesh Sewage (STP) | 9477 | 1000 DIP |

| Prioritization | Components | Project |
| --- | --- | --- |
| High priority It should be constructed immediately; as this project will connect the other remaining projects on the connection points- connections 1, 2, and 3 | - Rehabilitation of Khormaksar Pumping Station (mechanical, electrical, and civil works). - Construction of Connection-1, Connection - 2, and Connection-3. - Construction of new Force main with diameter 710 mm HDPE from connection-1 to connection-2. - Construction of new Force main 630 mm HDPE from Khormaksar PS to Connection-2. - Construction of new force main 1000 mm DIP from connection-2 to connection-3. | P-0 |
| High priority It should be constructed in in parallel with P-0 and to be connected with connection-1 at P-0 | - Rehabilitation of Crater Pumping Station (mechanical, electrical, and civil works). - Construction of new Force main with diameter of 560 mm HDPE from Crater PS to Connection-1. | P-1 |
| High priority To be start in parallel with P-0 to connect with connection-3 as this project will convey all the sewage from Aden to Sewage Treatment Plant. The risk of this project is related to the availability of financial fund; which is related to high capital cost of this project. | - Construction of remaining parts of Force main 1000 mm DIP from connection-3 to Al Areesh Sewage Treatment Plant. - Connection to bell mouth at Al Areesh STP. | P-2 |
| Medium priority This project will connect P-3 to P-0 at connection 1. | - Rehabilitation of Tawahi & Ma’alla PSs (mechanical, electrical, and civil works). - Construction of new Force main 450 mm HDPE from Tawahi PS to gravity existing Manhole M1A. - Construction of new Force main 630 mm HDPE from Ma’alla PS to connection-1. | P-3 |
| Total |  |  |

Table 3: The proposed project package.

    - Projects Locations:

The sanitation network rehabilitation sub-project will follow the existing footprint of the current streets and the same path of the sewerage network in the four districts in Aden city (Crater, Malla'a, Tawahi, and Khormaksar) as shown in the Figure 1.

Figure2: Existing Sewerage System in Aden

Figure 3- Proposed Packages

Tables below summarize the existing sewer networks data available to the year 2006.

Table 4- Details of Gravity& Main Pipelines - District (1)

| Name of zone | Number of pumping stations |  | Type of sewer line | Length of sewers (m) |  |  |  |  |  |  |  |  |  |  |  |  |  | No. of Manholes |  | Length of Force main Lines (m) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main | Mi- nor |  | Diameter (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 150 | 200 | 225 | 250 | 300 | 350 | 375 | 400 | 450 | 500 | 600 | 700 | 800 | 900 |  |  |  |  |
| CRATER | 1 | 1 | AC | 2370 | - | - | - | - | - | 2335 | 550 | - | 829 | - | - | - | - | Old | New | Old | New |
|  |  |  | VC/old | 3875 | - | 8355 | - | 4282 | - | - | - | - | - | - | - | - | - | 623 | 64 | - | 3837 |
|  |  |  | UPVC | 1890 | 2184 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  |  |  | EX.VC | - | - | - | 1042 | 645 | - | - | 179 | - | 63 | 570 | - | - | - | - | - | - | - |
| KHORMAKSAR | 1 | 25 | AC | 6112 | _ | 17210 | - | 4782 | - | 3362 | - | - | - | 400 | - | - | - | 557 | 66 | 2100 | 17209 |
|  |  |  | VC/old | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  |  |  | UPVC | 925 | 2858 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  |  |  | EX.VC | - | - | - | 20 | 668 | 568 | _ | 376 | - | - | - | - | - | - | - | - | - | - |
| MA'ALLA | 1 | 5 | AC | 2410 | - | 4618 | - | 1785 | - | 1735 | - | 745 | - | - | - | - | - | 247 | 141 | - | 3923 |
|  |  |  | VC/old | 5278 | - | 3350 | - | 2639 | - | - | - | - | 660 | - | - | - | - | - | - | - | - |
|  |  |  | UPVC | 4215 | 2800 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  |  |  | EX.VC | - | - | - | 898 | 295 | 1407 | - | 308 | - | 75 | - | 522 | - | 1638 | - | - | - | - |
| TAWAHI | 1 | 5 | AC | 2640 | - | - | - | - | - | - | - | - | - | - | - | - | - | 182 | 54 | 1500 | 2073 |
|  |  |  | VC/old | 3309 | _ | 2206 | - | 1838 | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  |  |  | UPVC | 1970 | 550 | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - | - |
|  |  |  | EX.VC | - | - | - | 415 | 252 | - | - | 199 | - | 514 | - | - | - | - | - | - | - | - |

Notes:

A: Al-Mansura , Sheikh Othman , Dar-Saad , Al-Boreiqa B: 7 July Project , Madinat–Al Shaab district

C: Al-Boreiqa D: Salah Addin

Table 5 – Details of the Existing Pumping Stations in Aden

| District | Name of pumping station | No. of working pumps | Power | Capacity (l/s) |
| --- | --- | --- | --- | --- |
|  |  |  | (KW) |  |
|  | H | 1 | 5 | 24.4 |
|  | J | 1 | 12 | 18 |
| KHORMAKSAR | 10 | 1 | 4 | 8 |
|  | E | 1 | 7.5 | 11 |
|  | G | 1 | 12 | 18 |
|  | 6 | 1 | 5 | 8 |
|  | 7 | 1 | 11 | 30 |
|  | 8 | 1 | 2 | 3.8 |
|  | 15 | 1 | 4 | 24.4 |
|  | Hae Badder (16) | 1 | 2.5 | 14.7 |
|  | L | 1 | 4 | 8 |
|  | M | 1 | 13 | 30 |
|  | P | 1 | 12 | 18 |
|  | B | 1 | 4 | 24.4 |
|  | C | 1 | 11 | 30 |
|  | Main pump | 3 | 540 | 700 |
|  | A | 1 | 5 | 14.9 |
|  | Bader Ps. | 1 | 4.5 | 3.8 |
|  | Al - Rashid Ps. | - | - | - |
|  | Al - Gamhorea Ps. | 1 | 2.8 | 24 |
| CRAT ER | Al – Qatei | 3 | 7.5 | 45 |
|  | Main pump | 3 | 90 | 160 |
| TAWAHI | Al-Aroosah | 1 | 4 | 24.4 |
|  | Al – Abkare | 1 | 2.5 | 14.7 |
|  | Al – Fath | 1 | 12 | 18 |
|  | Al- Bahrea | - | - | - |
|  | Main Ps. | 3 | 65 | 102 |
| MA’ALLA | Hugef | 2 | 11 | 30 |
|  | Hafoon | 1 | 7 | - |
|  | Main Ps. | 3 | 205 | 290 |
|  | Al – Assmak | 1 | 12 | 18 |
|  | Al- Pipsi | 1 | 3 | 14.7 |
| MANSUR A | Main | 2 | 150 | 281.7 |
|  | Industrial Area | 2 | 55 | 150 |
|  | Al – Sakneia | - | - | - |
|  | Al – Drain | 2 | 7.5 | 25 |
| SHEIKH OTHMAN |  |  |  |  |
|  | College | 2 | 15 | 91.5 |
| KAHRA |  |  |  |  |
|  | Omer Al-Muktar | 4 | 22 | 101.3 |
| MEMDARA |  |  |  |  |
|  | Al-Memdara | 3 | 15 | 95 |
|  | 7-Jul | - | - | - |
| DAR SAAD | Dar Saad | 2 | 7.5 | 17.5 |
|  | Al-Shaab | 2 | 13 | 18 |
|  | Al-Saknea | 2 | 2.2 | 5 |
| AL SHAAB | 7-Jul | - | - | - |
| AL – BRAIQA (Little Aden) | A-Class | 3 | 13.3 | 260 |
|  | B-Class | 1 | 5 | 45 |
|  | C-Class | 3 | 7.35 | 60 |
|  | Police Station | 1 | 7.5 | 30 |
|  | Bander-Shiekh | 1 | - | 80 |
|  | Salah Al-Deen | 4 | - | - |

Sewage Treatment Plants (STPs)

Treatment methods of wastewater in Aden are restricted mainly to stabilization pond systems, due to availability of land area, climatic suitability and operational simplicity in comparing to other methods. There are two existing stabilization pond stations, which are:

Al-Mansoora treatment plant and Al–Areesh sewage treatment plant.

- AL-Mansoora Sewage Treatment Plant (STP)

The existing plant consists of 10 ponds, laid in two parallel lines, each line consisting of five ponds, one anaerobic and four facultative operated in series. Lines were constructed at different periods; the first train was constructed in 1979, and the second in 1989. The outlet lines from the two trains are joined downstream of the ponds and then the effluent is transmitted by a single pipeline to the disposal sea site.

- Al–Areesh Sewage Treatment Plant (STP)

  - Construction of this treatment plant started in July 2001 and the plant was commissioned on 22 May 2002. It consists of two parallel trains, each train consisting of one anaerobic, one facultative and two maturation ponds. The total design capacity of this treatment plant is 70,000 m3/day (capacity of each train is 35,000 m3/day).

  - Although the present total inflow received by this STP is only 20,000 m3/day, there appears to be some problems in the efficiency of this station in reducing BOD5. Therefore, efforts are ongoing to take frequent readings of samples from each pond, trying to get solution for this mater. Following are some details of the ponds:

    - First Train

Anaerobic Pond A1

- volume at water at depth 4 m is:	127,000m3

- Retention time is:	6 days Facultative Pond F2

- Water surface area is:	104,992 m2

- Depth of water is:	2 m. Maturation Ponds M3 & M4

| - Top water surface area of M3 is: | 65821 m2 |
| --- | --- |
| - Depth of water for pond M3 is: | 2 m |
| - Top surface water area of M4 is: | 75,600 m2 |
| - Depth of water in M4 is: | 2 m. |
| - Total volume of first train is: | 576,000 m3 |
| ii) Second Train |  |
| Anaerobic Pond A5 - Retention time is: | 8 days |
| Facultative Pond F6 - Surface Water area is: | 106,000 m2 |

# Rehabilitation Plan

This section presents recommendation for the rehabilitation/construction plan for the force mains and pumping station including the construction cost estimates, phasing and implementation schedule considerations. This Rehabilitation Plan follows results of the site investigations along the existing force mains and pumping stations as well as the hydraulic calculations for force mains from the period (year 2021 to year 2046).

# Rehabilitation of Force Mains

Table 30 summarizes the recommended force main improvements; replacement of the existing force mains is recommended because of the status of inside the pipe “corrosion”, and to provide capacity for future growth since the main improvements are sized up to 2046 design year.

The rehabilitation works for force mains shall include air valves chambers and all the required fit- tings and bends. It is recommended to use polyethylene pipes for pipes with diameters less than or equal to 700 mm and to use ductile iron for the force mains more than 700 mm in diameter due to its high strength and high flow capacity.

In order to prevent the corrosion at the interior surface of pipe; it is recommended the following:

    - It is recommended to use ductile iron pipe with internal lining, a passive lining like Polyurethane as per EN15655 in order to prevent the corrosion at the interior surface of pipe.

    - As that the generated sewage at Aden consists of sea water with high salinity along with the domestic wastewater. In such soils, it is recommended to use externally Polyurethane.

    - It is recommended to have an air valve at each high elevation point along the route of the force mains.

    - It is recommended to carry out a frequent cleaning and maintenance and /or Periodic flush- ing of force mains to remove solids and grease build up and minimize corrosion due to the high concentration of sulfide.

Regarding the material of pipes, it proposed two alternatives at the previous stage of this project as follows:

- Alternative A: using High Density Polyethylene HDPE for force mains with diameter of 700 mm and less and using Ductile Iron for diameter larger than 700 mm.

- Alternative B: using Ductile Iron for all force mains.

Polyethylene Pipe (PE) is selected as an alternative for the existing material of force mains for pipes with diameter of 700 mm and less due to the following reasons:

- Handling of pipes: Dealing with ductile pipes needs heavy machinery for loading and handling, while PE pipes are relatively lighter in weight than DIP and can be handled by laborer’s or even by light machinery.

- Smooth internal surface of polyethylene pipe which gives those excellent hydraulic characteristics.

- Ductile pipes need more protection internally and externally while PE pipe do not need that protection from internal and external.

Alternative A is selected at Aden sewerage Project of using High Density Polyethylene HDPE for force mains with diameter of 700 mm and less.

Due to the existence of force mains along the cost of Aden city, the excavated trenches will be sub- ject to be flooded during the construction phase. It is assumed to use the dewatering technique to dispose the water out of trenches.

Table 5 – Proposed Replacement Plan for Force main

| Force Main | Existing Diameter & Material | Replaced Diameter (OD) & Material | Year for Construction Based on current status of pipe |
| --- | --- | --- | --- |
| From Tawahi PS to Existing Manhole M1A | 400 mm-DIP | 450 mm- HDPE | 2022 |
| From Ma’alla PS to Connection-1 | 600 mm-DIP | 630 mm- HDPE | 2022 |
| From Crater PS to Connection-1 | 500 mm-DIP | 560 mm- HDPE | 2022 |
| From Connection -1 to Connection 2 | 500 mm-DIP | 710 mm- HDPE | 2022 |
| From Khormaksar PS to Connection-2 | 1000 mm-DIP | 630 mm- HDPE | 2022 |
| From connection-2 to STP | 1000 mm-DIP | 1000 mm-DIP | 2022 |

# Rehabilitation of Pumping Stations

Table 31 summarizes recommended rehabilitation for the pumping stations including the pro- posed year needed for the construction.

Since the existing pumping stations are old and outdated, rehabilitations work will involve re- placement of the existing pumps with new pumps along with all civil, mechanical, and electrical works.

Pump stations shall provide the required capacity with a standby pump.

Table 6 – Proposed Rehabilitation Plan for Pumping Stations

| Pump Station | Proposed rehabilitation Plan | No. of Pumps | Flow (L/sec) Head (m) | Power (KW)/ each pump | Year for Construction based on flow capacity |
| --- | --- | --- | --- | --- | --- |
| Tawahi PS to Manhole M1A | Replace existing pumps with new pumps; total capacity of 124 l/s. This will meet sewage generation up to Yr.2046 | 2 duty/ 1 standby | 62 l/sec 14.6 m | 28 | 2 pumps @ Yr. 2022 |
| Ma’alla PS to STP | Replace existing pumps with new pumps; total capacity of 365 l/s. this will meet sewage generation up to Yr.2046 | 2 duty/ 1 standby | 182 l/s 34.2 m | 195 | 2 pumps @ Yr. 2022 |
| Crater PS to STP | Replace existing pumps with new pumps; total capacity of 248 l/s. this will meet sewage generation up to Yr.2046 | 2 duty/ 1 standby | 124 l/s 29.0 m | 125 | 2 pumps @ Yr. 2022 |
| Khor- maksar PS to STP | Replace existing pumps with new pumps; total capacity of 258 l/s. this will meet sewage generation up to Yr.2046 | 2 duty/ 1 standby | 129 l/s 20.9 m | 98 | 2 pumps @ Yr. 2022 |
| Total |  |  |  |  |  |

Regarding the re-sizing of existing wet well; the following criterion were taken into consideration to avoid failure of the pumping station:

    - The maximum detention time in the wet well shall be on average no more than 30 minute.

    - The selected number of pumping cycles in the calculations of the wet well volume shall be six cycles per hour for Ma’alla , Crater and Khormaksar and Ten cycles per hour for Tawahi; that is the pumping cycle/hours shall be not more than 10 cycles/hour.

    - Detention time in the force main shall be less than 6 hours.

With this figure, the required volume for the wet well is illustrated at table 7 below.

| Pump Station | New Wet well volume (m3) | Existing wet well volume (m3) |
| --- | --- | --- |
| Tawahi PS to Manhole M1A | 9.05 | 67 |
| Ma’alla PS to STP | 40.95 | 140 |
| Crater PS to STP | 27.9 | 75 |
| Khormaksar PS to STP | 29 | 160 |

The emergency diesel generator is accommodated in a separate superstructure next to the pump station wet well. The emergency diesel generator will be sized to be sufficient for the ultimate phase.

# Rehabilitation Works for Pumping Station

The rehabilitation works at each pumping station are summarized in the following section:

Crater Pumping Station

- Silent generator to be installed according to the capacity of the pumps with special specifications regarding the gases spread strongly in the station.

- The control panel is very old and needs to be replaced with new specifications; this panel is the same since station's construction in 2002.

- All valves inside pumping station should be replaced

- An Automatic wastewater screen shall be installed due to the large number of impurities in the tank.

- Automatic crane to be installed with capacity of 5 tons.

- All mechanical work inside pumps room, electrical room, penstocks, manual screen and mechanical equipment's inside chambers shall be replaced.

- The pumping station contains a number of rooms/ buildings which are central control panel room, generator room, transformer room, facilities (WC and kitchen) and opera- tor’s room. All rooms are in good condition except the generator room, it need to be reconstruct but the whole buildings need to be restored only.

- Replace all metal elements (handrails, steel cover and doors) with new ones due to its corrosion.

- Penstock valve shall be installed with high quality steel specifications to withstand this environment.

- All mechanical, plumping equipment and sanitary inside services buildings shall be re- placed.

Regarding the civil works of Crater pumping station, the following is needed:

- Reconstruct new structure (generator room) including all Civil and Architectural Works; concrete works (foundation, columns, ceiling…. etc.), block work, plastering, painting, floor- ing, windows (louvers) and doors.

- Supply standby generator with 500 kva run by diesel.

- Plastering and Re-painting all the buildings (walls and ceiling) of this Station.

- Repairing the electricity for all the buildings inside the Station.

- Renovating and repainting the fence around the Station.

- Replace doors and windows of all the buildings inside the Station.

- Repair of the bathrooms by replacing the water & drainage network, doors and windows, electricity, ceramics of the walls & floors and the sanitary ware (Toilet and Sink) with new one.

- Replace all barriers and ladders.

- Providing Generator for the Station used when the currency off.

- Apply torch applied damp proof membrane at the inlet channel and wet well for insulation purpose.

- Replace the main gates with new steel gates.

- Repairing the damage part and cleaning the yard and cleaning the joints and Appling seal- ant.

- Replace all metal elements (handrails and steel cover) with new ones.

- Reuse the guard room in the west of the station as a storage room and remove the existing door on the street and Add a new door on the opposite side.

- Close the Maintenance area by construct new block wall and added new door and windows and Repair the current corrugated metal panel on the roof in case of any damage.

Control Panel	                                                                                Pumps room

Screen channel	Wet well

Khormaksar Pumping Station

  - Silent generator to be installed according to the capacity of the pumps with special specifications regarding the gases spread strongly in the station.

  - The pumping station contains a number of rooms/ buildings which are central control panel room, generator room, transformer room, medium voltage generator room, WC and operator’s room. All rooms are in good condition, but need to be restored.

  - Replace all metal elements (handrails, steel cover and doors) with new ones due to its corrosion as a result of pollution near the station.

  - Penstock valve shall be installed with high quality steel specifications to withstand this environment.

  - Re-install the Odour system in the sewers.

  - Fix the mechanical screening system, and this system is very important, especially in Aden, as it protects the pump from damage.

  - All valves inside pumping station shall be replaced. 8- Install ventilation system for the pump valves room.

- All mechanical work inside pumps room, electrical room, penstocks, manual screen and mechanical equipment's inside chambers shall be replaced.

- Automatic screen shall be added.

- All mechanical, plumping equipment and sanitary inside services buildings should be re- placed.

- Reducing the volume of the wet well by sealing part of it with hardcore to a height of 5 m only and constructing a new structural wall separating the remaining space from the silt. (rock fill shall be placed behind new wall to fill the empty space and top concrete slab shall be casted over fill).

- Automatic crane shall be added.

Regarding the civil works of Khormaksar pumping station, the following is needed:

- Cleaning the collection basin and wet well, water proofing (torch applied damp proof membrane), concrete repair, and repair the steel cover or replacing with precast concrete cover.

- Demolishing the current screen channel and Re-construct it to suit the new flow. Refer to the images that show the damaged current channel.

- Plastering and Re-painting all the buildings (walls and ceiling) of this Station

- Repairing the electricity for all the buildings inside the Station.

- Renovating and repainting the fence around the Station.

- Replace doors and windows of all the buildings inside the Station.

- Repair of the bathrooms by replacing the water & drainage network, doors. and windows, electricity, ceramics of the walls & floors and the sanitary ware (Toilet and Sink) with new one.

- Replace all barriers and ladders.

- Providing Generator for the Station used when the currency off.

- Repairing the sea outfall (in case it damaged).

- Apply torch applied damp proof membrane at the inlet channel and wet well for insula- tion purpose.

- Replace the main gates with new steel gates.

- Repairing the yard damage part and cleaning the yard, Joints and applying sealant.

- Replace the concrete cover over the wet well with steel covers as in the rest of the station.

Screen channel

Control Panel

Wet well covered by precast concrete

Transformer door

Ma’alla Pumping Station

- Replace the old crane with a new electrical crane.

- This station suffers from permanent accumulation of dirt. Dirt form of the solid deposits that cause a permanent blockage, sewage overflowing in the streets as shown in the picture below. This station is the highest station among the four main stations in terms of sediments. A solution to this problem, such as a sedimentation tank or a mechanical screen before sewage enters the septic tank, is needed.

- Automatic screen shall be added.

- Replace all metal elements (handrails, steel cover and doors) with new ones due to its corrosion as a result of pollution near the station.

- The pumping station contains a number of rooms/ buildings which are central control panel room, generator room, transformer room, storage room, maintenance area, facilities (WC and kitchen) and Guard room. All rooms are in good condition and need to be restored.

- The generator room shall be demolished and reconstruct based on the new generator layout.

- All mechanical work inside pumps room, electrical room, penstocks, manual screen and mechanical equipment's inside chambers shall be replaced.

- All mechanical, plumping equipment and sanitary inside services buildings shall be re- placed.

Regarding the civil works of Ma’alla pumping station, the following is needed:

- Plastering and Re-painting all the buildings (walls and ceiling) of this Station.

- Repairing the electricity for all the buildings inside the Station.

- Plastering and Re-painting the fence around the Station.

- Replace the doors and windows with new ones.

- Repair of the bathrooms by replacing the water & drainage network, doors and win- dows, electricity, ceramics of the walls & floors and the sanitary ware (Toilet and Sink) with new one.

- Reconstruct new structure (generator room) including all Civil and Architectural Works; concrete works (foundation, columns, ceiling…. etc.), block work, plastering, painting, flooring, windows (louvers) and doors.

- Replace all barriers and ladders.

- Apply torch applied damp proof membrane at the inlet channel and wet well for insulation purpose.

- Replace the main gates with new steel gates.

- Repairing the damage part and cleaning the yard and cleaning the joints and Appling sealant.

Wet well

Screen channel

Tawahi Pumping Station

- All valves inside pumping station shall be replaced.

- Control Panel is damaged and need to be replaced.

- Silent generator to be installed according to the capacity of the pumps with special specifications that bear the gases spread strongly in the station.

- Penstock valve shall be installed with high quality steel specifications to withstand this environment.

- This station suffers from permanent accumulation of dirt. Dirt form of the solid deposits that cause a permanent blockage, sewage overflowing in the streets as shown in the picture be- low. This station is the highest station among the four main stations in terms of sediments. A solution to this problem, such as a sedimentation tank or a mechanical screen before sew- age enters the septic tank, is needed.

- Replace all metal elements (handrails, steel cover and doors) with new ones due to its corrosion as a result of pollution near the station.

- The pumping station contains a number of rooms/ buildings which are central control panel room, generator room, transformer room, storage room, maintenance area, facilities (WC and kitchen) and Guard room. All rooms are in good condition, need to be restored.

- All mechanical work inside pumps room, electrical room, penstocks, manual screen and mechanical equipment's inside chambers shall be replaced.

- All mechanical, plumping equipment and sanitary inside services buildings shall be replaced. 10- Automatic crane shall be added.

11- Automatic screen shall be added.

Regarding the civil works of Tawahi pumping station, the following is needed:

- Plastering and Re-painting all the buildings (walls and ceiling) of this Station.

- Repairing the electricity for all the buildings inside the Station.

- Renovating and repainting the fence around the Station.

- Replace doors and windows of all the buildings inside the Station.

- Repair of the bathrooms by replacing the water & drainage network, doors and windows, electricity, ceramics of the walls & floors and the sanitary ware (Toilet and Sink) with new one.

- Repair the existing hole beside the main door.

- Replace all barriers and ladders.

- Re-wiring all electricity system

- Replace the penstocks.

- Replace the steel covers.

- Apply torch applied damp proof membrane at the inlet channel and wet well for insulation purpose.

- Plastering and Re-painting the fence and repairing any damage.

- Replace the main gates with new steel gates.

- Repairing the damage part and cleaning the yard and cleaning the joints and Appling sealant.

Damage in ground beside the control	Wet well

# Section Two: Population forecasting & Wastewater Generation

Population estimates are essential in order to present, develop and project wastewater generation in the future. The amounts of flow are necessary for the design of wastewater collection, force main and pumping stations.

# Population Growth

The first results of 2004 Census revealed that the resident population of Aden for that year was approximately 600,000, about 3% of the national population. The total population of the Governorate of Aden, according to the latest estimates at year 2020, reached 925,000 covering the eight districts.

# Planning Period

The planning period for the project was established to be 25 years (up to year 2046). Therefore, key planning such as population and wastewater generation had been defined in five-year increments based on the first year 2021.

# Projected Population

For determining corresponding population projections for wastewater systems requirements for Aden districts, the population datum year of 2005 was used according to the information given by LWSCA through the Statistics Department and by using census data of year 2004 and available data for wastewater generation as mentioned at the report that was prepared by MWH dated 2009.

Forecasting is used on the assumption that the recent trends will continue in a liner manner to the 2046 horizon design year. Thus, projections are based on the assumption that the percentage growth will remain relatively constant throughout the design period.

It was assumed that based on the Development Programme report prepared that the population increase has stopped in Ma’alla, Tawahi and Crater districts; and the After 2025, the household bracket will be raised by 1 liter/year; that was reflect into the population forecasting.

Table 8 shows the projected population at year 2046 for the four districts.

Table 8– Population forecast at year 2046

| Year | Tawahi | Ma'alla | Crater | Khormaksar City | Total |
| --- | --- | --- | --- | --- | --- |
| 2046 | 32,000 | 70,000 | 75,000 | 57,500 | 234,500 |

# Generated wastewater

The design of sewage network and the pumping stations requires evaluation of the current and future population. The estimated per capita wastewater generation rates for the design period in Aden was determined based to figure which were given in the Development Programme Report prepared by MWH; which are taken as follows at year 2020:

128 l/c/day for Khormaksar City

105 l/c/day for Crater district

107 l/c/day for Ma’alla district

111 l/c/day for Tawahi district

Based on LWASCA letter dated 6th of Jan.2022; the estimated per capita to be increase 1 litre /year until year 2046; which are taken as follows at year 2046:

154 l/c/day for Khormaksar City

131 l/c/day for Crater district

133 l/c/day for Ma’alla district

137 l/c/day for Tawahi district

Table 5 shows the estimated average domestic and non-domestic sewage generation from the three districts and Khormaksar City up to year 2046.

Table 9 – Estimated Avg. Generated Domestic Sewage in m3/day up to year 2046

| District | Year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2021 | 2026 | 2031 | 2036 | 2041 | 2046 |
| Khormaksar City | 7,418 | 7,705 | 7,993 | 8,280 | 8,568 | 8,855 |
| Crater | 7,950 | 8,325 | 8,700 | 9,075 | 9,450 | 9,825 |
| Ma'alla | 7,560 | 7,910 | 8,230 | 8,610 | 8,960 | 9,310 |
| Tawahi | 3,584 | 3,744 | 3,904 | 4,064 | 4,224 | 4,384 |
| Total | 26,512 | 27,684 | 28,857 | 30,029 | 31,202 | 32,374 |

Based on the information given at Development Programme Report; that there is a groundwater infiltration into the existing collection system. The infiltration quantities per day at the year 2025, which are supposed to be constant, are shown at table below. Table 10

| District | Area, ha | Infiltration (m3/day) |
| --- | --- | --- |
| Khormaksar | 491 | 2,455 |
| Crater | 8 | 40 |
| Ma'alla | 63 | 315 |
| Tawahi | 25 | 140 |

# Peak Factors

Sizing of the main force lines is proposed based on the peak sewage flow. Due to lack of information regarding the annual wastewater flow for the previous 3 years, the peak factor was estimated based on the comparison of results from the different equations. Table 11

| District | Babbitt’s formula | Harmon’s | International Standard | Avg. Peak factor |
| --- | --- | --- | --- | --- |
| Khormaksar | 2.55 | 2.21 | 2.0 | 2.25 |
| Crater | 2.43 | 2.11 | 2.0 | 2.18 |
| Ma'alla | 2.46 | 2.13 | 2.0 | 2.20 |
| Tawahi | 2.81 | 2.45 | 2.0 | 2.42 |

Accordingly, the design hourly peak flow at year 2046 from the four districts is calculated to be as shown at Table 12.

Table 12 – Estimated Peak Sewage in m3/hr up to year 2046

| District | Year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2021 | 2026 | 2031 | 2036 | 2041 | 2046 |
| Khormaksar | 690 | 717 | 744 | 777 | 798 | 824 |
| Crater | 722 | 756 | 790 | 824 | 585 | 893 |
| Ma'alla | 692 | 725 | 757 | 789 | 821 | 853 |
| Tawahi | 361 | 377 | 393 | 410 | 426 | 442 |
| Total | 2,466 | 2,575 | 2,684 | 2,793 | 2,902 | 3,011 |

The Design flow at year 2046 from the four districts including the infiltration quantities are as shown at Table 13.

Table 13 – Estimated Sewage in l/sec up to year 2046

| District | Year |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2021 | 2026 | 2031 | 2036 | 2041 | 2046 |
| Khormaksar | 192 | 228 | 235 | 243 | 250 | 245 |
| Crater | 201 | 211 | 220 | 229 | 239 | 248 |
| Ma'alla | 192 | 205 | 214 | 223 | 232 | 241 |
| Tawahi | 100 | 106 | 111 | 115 | 120 | 124 |
| Total | 685 | 750 | 780 | 810 | 840 | 871 |

- Climate and Weather

3.1 Aden city

#### Temperature

The maximum temperature in Aden all the year ranges from 27 Degrees Celsius in winter and rises in the summer up to 40 Degrees Celsius. Generally, the rainfall rate is extremely low, with an often-rainy winter or spring and dry summer. The median rain total is about 50 mm/year, with a humidity rate of 62% - 73%.

#### Air quality and noise

Air pollution in Yemen is caused by a variety of factors, including emissions from vehicles. The main source of air pollution, however, particularly in cities such as Aden, is emissions from vehicles. The Pollutant concentrations are estimated to be several times higher than set standards for air quality as well as the noise level.

#### Existing sewer network in Aden

The first sewerage network in Aden was commissioned in the year 1954 and was operated by the Municipality of Aden. The service area was limited at that time to the most urban districts such as Crater and newly built parts of Al-Ma’alla, Al-Tawahi and Al-Boreiqa districts. The sewerage networks in Khormaksar, Sheikh Othman and Al-Mansura districts were constructed in late 1950s and early 1960s. The old sewerage networks in several parts, depended mainly on open channel passages system to collect the wastewater from houses. The old sewer network in Sheikh Othman and Al–Kahera districts was used to collect the wastewater from bath and kitchen only, while the night soil was collected separately and taken away by vehicles to a burning site. During the period between 1997 and 2002, there was a steep development of the sewerage network in Aden. This development mainly concentrated first in providing the necessary maintenance equipment, spare parts for pump units in pumping stations, working crews for the treatment ponds and equipment for the wastewater laboratory.

After the implementation of basic developments, mentioned above, LWSCA are now planning to make extensions and improvements to the old and new existing sewerage networks to satisfy the increasing needs for sanitation services. Twenty years ago, the sewage networks at districts Crater, Tawahi, Ma'alla and Khormaksar were connected to a new force main. The new force main conveyed wastewater to the wastewater treatment plant at the north of Aden via a pumping station.

3.2   Legal and Institutional Framework of the Project

This ESMP was prepared according to YEHCP and co-financing ESMF to meet the requirements of the World Bank’s Environment and Social Standards (ESSs), including the World Bank Group Environment, Health and Safety (EHS) Guidelines, and other guidelines and guidance and to comply with national environmental and social laws and regulations.

### World Bank Requirements Relevant to the project

The only relevant ESSs for this sub-project are ESS1, ESS2, ESS3, ESS4, ESS6 and ESS10. The ESS5 is not relevant because the subproject does not include to any land acquisition, restrictions on land use, or involuntary resettlement as the interventions will be on an existing facility.

### Environment, Health and Safety Guidelines

The ESF also requires all projects to apply the relevant requirements of the World Bank Group Environmental, Health and Safety Guidelines (EHSGs). These are technical reference documents, with general and industry specific examples of Good International Industry Practice (GIIP).  They define acceptable pollution prevention and abatement measures and emission levels in World Bank financed projects.

The EHS Guidelines contain the performance levels and measures that are generally considered to be achievable in new facilities by existing technology at reasonable costs.  Application of the EHS Guidelines to existing facilities may involve the establishment of site-specific targets, with an appropriate timetable for achieving them.

The application of the Guidelines to existing facilities may involve the establishment of site-specific targets with an appropriate timetable for achieving them.  The environmental assessment process may recommend alternative (higher or lower) levels or measures, which, if acceptable to the World Bank, become project - or site-specific requirements.

If less stringent levels or measures than those provided in the EHS Guidelines are appropriate, in view of specific project circumstances, a full and detailed justification for any proposed alternatives is needed as part of the site-specific environmental assessment.  This justification should demonstrate that the choice for any alternate performance levels is protective of human health and the environment.  When host country regulations differ from the levels and measures presented in the EHS Guidelines, projects are expected to achieve whichever is more stringent.

COVID-19 Sensitivity

The subproject like other sectors will be under the risks of Covid-19 pandemic. The precautionary measures for Corona virus against were applied during the preparation period and it will be strictly applied in the subproject worksites and worker’s accommodation place during implementation. Awareness-raising will be conducted for all workers and engineers before and during implementation by UWS-PMU/ADEN REGION and UNOPS. The awareness-raising plan of UWS-PMU/ADEN REGION includes 2-day training prior implementation for all key personnel (contractor engineers and the supervision consultants) who in turn will convey the training for temporary workers. It is also planned to conduct training sessions during implementation by UWS-PMU/ADEN REGION safeguards specialists on Covid-19, OHS, and other ESS issues. Furthermore, the contractor will implement training and awareness-raising for   workers before implementation and during implementation and the C-ESMP will highlight specific details of the training and awareness-raising plan.

  - Gender and Social -related issues:

The beneficiaries, males and females, have been considered along with the other stakeholders in designing of the subprojects. Their views and needs are taken into consideration in all phases. The consultations with men and women, which took place in Aden city in the forms of face-to-face interviews and group discussions, addressed the social issues including community needs, concerns, rights, and risks. The participants in consultations confirmed the selection priority needs. They showed their complete support for implementation.

    - Child Labor:

Child labor is totally prohibited in the subproject. No child labor will be hired in implementation and the contractors are forbidden to hire a person under the age of 18 years. The minimum age of workers has specified in the tender documents for contractors. A labor log including names ages and other details will be kept, and all workers will be registered. The contractor, UWS-PMU/ADEN REGION and UNOPS will continuously monitor this issue.

    - Gender

UWS-PMU/ADEN REGION has ensured gender equality as possible in the subproject's cycle as a core principle for success. Gender mainstreaming is considered in all aspects of the subproject cycle. 60 women and men were engaged in public consultations, in which the gender issues among other were discussed. They were informed about gender mainstreaming, rights, roles, and other important issues, such road safety   and GRM, GBV, etc. It was also listened to their concerns and feedback as they highlighted their support and needs to involved local people in implementation in suitable works. Their concerns and feedback were taken into consideration.

    - Gender-Based Violence GBV and Sexual Exploitation and Abuse SEA

UWS-PMU/ADEN REGION raised the awareness of community members for men and women on GBV & SEA during the public consultation process as well as on GRM processes and how they can be used to report gender-based violence, gender discrimination and SH/SEA cases with the highest level of confidentiality and anonymity of complaints. UWS-PMU/ADEN REGION plans to conduct training for contractors and engineers about GBV, SH/ SEA and GRM. They were informed about national laws that make sexual harassment, abuse, and gender-based violence a serious and punishable offense. The contracts with contractors will include Code of Conduct which will highlight the GBV, PSEA and SH. UWS-PMU/ADEN REGION is currently preparing a policy for GBV, PSEA/SH.

    - Conflict sensitivity and Do No Harm

During public consultations at all levels, UWS-PMU/ADEN REGION investigated whether the subproject is a priority, free of conflict, or negative effects on the implementation. Accordingly, it was ensured that the subproject is an urgent priority for community and beneficiaries and it is completely supported by all consulted persons. Moreover, it was proven that the subproject is free of conflict sensitivity and any negative effects that may hinder implementation according to screening and consultations.

###### 4. Environmental and Social Impacts Assessment:

Applicability:

The ESMF applies because the subproject is likely to have direct or indirect environmental or social impacts.

Eligibility (Exclusion List)

The subproject is eligible for support because it does not have any of the attributes in the following exclusion list. Table 14

Exclusion list

| # | Statement | Yes | No |
| --- | --- | --- | --- |
| 1 | Production or activities involving harmful or exploitative forms of forced labor/ harmful child labor; |  | X |
| 2 | Production or trade in any product or activity deemed illegal under host country laws or regulations or international conventions and agreements; |  | X |
| 3 | Production or trade in weapons and munitions; |  | X |
| 4 | Gambling, casinos and equivalent enterprises; |  | X |
| 5 | Trade in wildlife or wildlife products regulated under CITES; |  | X |
| 6 | Production or trade in radioactive materials; |  | X |
| 7 | Production or trade in or use of un-bonded asbestos fibers; |  | X |
| 8 | Production or trade in wood or other forestry products from unmanaged forests; |  | X |
| 9 | Production or trade in products containing PCBs; |  | X |
| 10 | Production, trade, storage, or transport of significant volumes of hazardous chemicals, or commercial scale usage of hazardous chemicals; |  | X |
| 11 | Production or trade in pharmaceuticals subject to international phase outs or bans; |  | X |
| 12 | Production or trade in pesticides / herbicides subject to international phase outs or bans; |  | X |
| 13 | Production or trade in ozone depleting substances subject to international phase out; |  | X |
| 14 | Production or activities that impinge on the lands owned, or claimed under adjudication, by indigenous peoples, without full documented consent of such people; |  | X |
| 15 | Power plants; |  | X |
| 16 | Large-scale transport infrastructure such as highways, expressways, urban metro- systems, railways, and ports; |  | X |
| 17 | Investments in extractive industries; commercial logging; |  | X |
| 18 | Dams, or projects involving allocation or conveyance of water, including inter-basin water transfers or activities resulting in significant changes to water quality or availability; |  | X |
| 19 | Activities that would significantly convert natural habitats or significantly alter potentially important biodiversity and/or cultural resource areas; |  | X |
| 20 | Activities that would require the relocation of residential households and/or significant involuntary land acquisition; or |  | X |
| 21 | Activities in disputed areas. |  | X |

4.1  Environmental and Social Screening Form:

UWS-PMU/Aden used the following form to screen for the potential environmental and social risks and impacts for the selected subprojects under YEHCP and co-financing ESMF. The risk level is moderate which required preparation of ESMP.

UNOPS include environmental and social clauses in all work contracts and ensure that contractor personnel are familiar with these clauses, and by also requiring contractors to comply with the World Bank Environmental Health and Safety (EHS) Guidelines and National Labor Law (Decree 5/1995) and applicable International Labor Organization conventions on workplace conditions.

| Subprojects Name | Rehabilitation of Wastewater Pump Stations and Main Trunk Line at Several Place in the City (Crater, Malla'a, Tawahi, and Khormaksar). |
| --- | --- |
| Subproject Location | Aden city |
| Implementing Partner | UW-PMU-Local Contractor |
| Estimated Investment |  |
| Was the site visited beforehand | Yes |
| Estimated Start/Completion Date |  |
| Observations/Comments | Indicated below |
| Signature of ESSO |  |
| Signature of Program Manager |  |

4.2 Environmental and Social Screening Form

Table 15

| Question | Answer |  | ESS relevance | Due diligence/ Actions |
| --- | --- | --- | --- | --- |
|  | Yes | No |  |  |
| Does the subproject involve civil works including new construction, expansion, upgrading or rehabilitation of existing infrastructure? | X |  | ESS1 | ESMP, SEP |
| Does the subproject involve land acquisition and/or restrictions on land use? |  | X | ESS5 | SEP |
| Is the subproject associated with any external waste management facilities such as a sanitary landfill, incinerator, or wastewater treatment plant? |  | X | ESS3 | ESMP, SEP |
| Does the sub-project use additional technically feasible water conservation measures? | X |  | ESS1, ESS3 | ESMP |
| Does the sub-project consider additional strategies to adopt measures that avoid or minimize negative effects of emissions? | X |  |  |  |
| Does the subproject have an adequate system in place (capacity, processes and management) to address waste? | X |  |  |  |
| Does the subproject involve the recruitment of workers including direct, contracted, primary supply, and/or community workers? | X |  | ESS2 | LMP, SEP |
| Does the subproject have appropriated OHS procedures in place, and an adequate supply of PPE (where necessary)? | X |  | ESS2, ESS4 | LMP |
| Does the sub-project have a GM in place, to which all workers have access, designed to respond quickly and effectively? | X |  | ESS10 | SEP |
| Does the subproject involve use of security or military personnel during construction and/or operation of healthcare facilities and related activities? |  | X | ESS4 | ESMP, SEP |
| Does the sub-project establish and implement appropriate quality management systems to anticipate and minimize risks and impacts that services may have on community health and safety? | X |  |  |  |
| Does the sub-project apply the concept of universal access where technically and financially feasible? |  | X |  |  |
| Is the subproject located within or in the vicinity of any ecologically sensitive areas? |  | X | ESS6 | ESMP, SEP |
| Is the subproject located within or in the vicinity of any known cultural heritage sites? |  | X | ESS8 | ESMP, SEP |
| Does the project area present potential Gender-Based Violence (GBV) and Sexual Exploitation and Abuse (SEA) or Sexual Harassment (SH) risk? | X |  | ESS1/ESS4 | ESMP, SEP/GBV Action Plan |

###### 5. Environmental and Social Impacts and Mitigation Measures

###### 3.1 Environmental and Social Impacts

UNOPS and UWS-PMU/Aden will mitigate the above-mentioned impacts by including environmental and social clauses in the contract and ensure that contractor personnel are familiar with such clauses, and also by requiring contractors to comply with the World Bank Environmental Health and Safety (EHS) Guidelines and National Labor Law (Decree 5/1995) and applicable International Labor Organization conventions on workplace conditions.

The sub-project assigned risk is moderate, because the subproject intervention does not involve activities that have a high potential environmental and social impacts. Thus, the sub-project require preparation of Environmental and Social Management Plan ESMP as detailed in the Environmental and Social Management Framework ESMF for Yemen Emergency Human Capital (YEHCP), Yemen Integrated Urban Service Emergency Project II (YIUSEP II), and other financing. However, OHS impact may be triggered. Therefore, UNOPS will include environmental and social requirements for contractor including all OHS requirements, as well as Health and Safety prevention measures from COVID-19 in the contract and tender documents.

UNOPS will ensure that:

- The environmental and social requirements for contractor including all OHS requirements (disclosed as part of the ESMF) will be applied to contractor, to fully avoid or mitigate environmental, social, occupational health and safety impacts that might arise from this activity.

- The equipment for the rehabilitation and construction of the pump stations and new trunk lines will be compliant to environmental, health and safety standards and specifications including electricity safety, weather resistance, and UL standards.

- The pump station operators will receive proper training on the safe operation and maintenance of the pumping systems.

Some negative impacts may be triggered during the implementation of the sub-project, and they vary from minor, moderate and substantial; however, they are localized and temporary. The potential negative impacts are mostly related to workers OHS which will be tackled with proper mitigation plan. Other potential negative impacts are those related to generation and disposal of debris and waste as well as dust and noise. These highlighted potential impacts will be addressed and mitigated by the necessary matching mitigation measures and full implementation of this Environment and Social Management Plan. The potential impact of this sub-project is electrical safety and matching mitigation measures for insulated work tools and isolated work instruments were included in the ESMP and tender documents.

UNOPS and UWS-PMU/Aden  will mitigate the identified impact of increased noise during procurement, by including in the specifications and the technical standards for all supplied electrical equipment’s  specification of low noise equipment according to international technical practices, safety measures and quality control, and for all electrical parts used in wet environment to be double insulated, and for all material used in manufacturing wet parts not to contain any toxicological properties or substances harmful to health.

UNOPS will also require that contractor:

- Inspect existing facility and apply all safety measures to reduce the risk of any injury to the workers during construction or the users during pump operation, subject to written approval by the UNOPS engineer provided before implementation of work.

- Conduct risk assessment during the execution, evaluate the risk, and put the appropriate safety measures in place and submit it for review and approval.

- Fully implement permit to work system, method of statement and job safety analysis to ensure all tasks are well prepared and follow all necessary safety mitigation and prevention measures.

- Provide safety training to all workers including working at height, lifting operations, electrical safety and permit to work before commencing any work.

- Provide the required safety and health PPE and hygienic materials to workers to protect workers and ensure their safety and prevent them from Covid-19 infection.

- Provide fully insulated PPE, isolated installation tools, instruments and equipment.

- Provide appropriate training on the use, serviceability and integrity of the necessary PPE.

- Ensure proper use of ladders and scaffolds by trained employees, apply regular inspection and testing, use of fall prevention devices, including safety belt and lanyard travel limiting devices to prevent access to fall hazard area, or fall protection devices such as full body harnesses used in conjunction with shock absorbing lanyards or self-retracting inertial fall arrest devices attached to fixed anchor point or horizontal life-lines.

- Prepare emergency response plan including contact numbers, evacuation plan and provide necessary first aid equipment in site and transportation and contracted nearest hospital in case of any emergency.

Follow the fall prevention and protection measures by:

- Installation of guardrails with mid-rails and toe boards at the edge of any fall hazard area.

- Inclusion of rescue and/or recovery plans, and equipment to respond to workers after an arrested fall and a fall protection plan should be in place which includes the following aspects:

- Training and use of temporary fall prevention devices, such as rails or other barriers able to support a weight of 200 pounds, when working at heights equal or greater than two meters or at any height if the risk includes falling through an opening in a work surface.

- Training and use of personal fall arrest systems, such as full body harnesses and energy absorbing lanyards able to support 5000 pounds.

A health and safety training should be provided to workers to avoid electrocutions and potential electric hazards and wearing proper PPEs.

Follow the slip prevention measures in the same elevation by:

- Use of control zones and safety monitoring systems to warn workers of their proximity to fall hazard zones, as well as securing, marking, and labeling covers for openings in floors, roofs, or walking surfaces.

UNOPS has already taken the following steps in GBV/SEA/SH:

- In the stakeholder consultation meetings UNOPS has presented the project GBV SEA/SH action plan and during the meetings we focused on female’s participants and ensure to explain about the GM mechanism and highlighted how it is transparent, secure and confidential to use any of the GM access point.

- UNOPS has developed visibility materials to promote awareness for PSEA/SH in local language (Arabic) the materials and messages used adapted to be suitable for Yemen context and sensitivity of the subject.

- GM focal point received specialized training about SEA/SH cases and the way to deal with it using Victim centered approach.

- UNOPS is developing SOP and protocol for GM in how to deal with SEA/SH cases.

- UNOPS has conducted refresh sessions for Project Personnel in GBV/SEA/SH and trained retainer’s sites engineers as well.

- UNOPS has prepared risk assessment tools for GBV and will require contractors to fill a checklist on GBV/SEA/SH and to prepare code of conduct for their workers/staff.

- UNOPS will roll out  SEA/SH prevention and response plans for contractors , where the contractors need to prepare the action plan as part of the tender documents , UNOPS is supporting to enhance the contractors capacity in this area looking to the fact that almost they have zero knowledge and capacity , for that UNOPS developed contractors action plan template where it covers the most priority areas and UNOPS conducted induction session for contractors about this requirement and presented to  contractors  on how to prepare their own GBV SEA/SH prevention and response plans (GBV Action Plans) using the developed template , other in depth training sessions will follow and will continue during project life span.

- UNOPS will train contractors’ PSEA/SH focal points.

- UNOPS will require contractors to employ at least 5% female staff to encourage gender mainstreaming.

Labor Management:

The estimated/planned number of labors for sewer network rehabilitation project is 7 skilled and 19 unskilled workers in which the contractor is responsible for.

Child Labor and Forced Labor:

Ensure all workers are more than 18 Years old, and no child, forced, involuntary or unpaid labor will be used in any work.

The contractor shall protect the workers from any risk that may be encountered during the implementation including the exposure to the virus (COVID-19).

The contractor shall maintain occupational health and safety system in the site to protect workers from hazards and risks and provide adequate health and safety training, required PPE, first aid box, and toilets and potable drinking water.

Community health and safety

Exposure to COVID risk during the rehabilitation of sewer network.

These impacts on the community health and safety can be mitigated through:

Install barriers, danger warning signs and restriction signs to only authorized persons and signs showing the potential danger to the public. And establish barriers around the working site, equipment area and excavation area.

The following table includes the mitigation measures for the main generic related impacts:

5.1   Potential Impacts and Corresponding Mitigation Measures

Table (    16   ): Potential Impacts and Corresponding Mitigation Measures

| Potential Impact | Mitigation Measure | Implementation Responsibility | Estimated cost (USD) |
| --- | --- | --- | --- |
| Occupational health and safety |  |  |  |
| Lifting Operations During the lifting of the vacuum circuit breaker activity the following potential impact should be avoided: Failure of lifting equipment; Falling loads; workers being crushed by a moving Load or lifting equipment which all might result in fatalities or injuries. | Prevent access at areas with lifting operations Ensure safe distance is maintained from lifted equipment Install warning signs for lifting activities Provide required information and training on the lifting operations to the site workers. Ensure applying safe lifting operations. Ensure existence of a flag man and use of proper communication means. Ensure testing and checking the lifting equipment and license of trained operator. Ensure use of proper PPE safety materials and tools. Use well-maintained equipment for lifting that are appropriate for the weight; well checked and tested by a third party. Secure loads when lifting and use strong and reliable fixation materials to make sure that the load is well tighten and no solid parts falls from the load during lifting. Lifting device capacity shall be 1.65 times the maximum calculated static load at that point. An ultimate load shall be ≥ 4 times the maximum static load. | Contractor UW, S-PMU/Aden and WSLC Aden. | $ 8000 |
| Infection by Covid-19 Transmission of corona virus between site workers Site workers lives could be at risk( illness / Death) | UNOPS will ensure that contractor will provide health, safety and hygiene materials to staff, workers and visitors and provide proper training on health and hygiene issues. Contractor to maintain routine cleaning and disinfecting of surfaces, equipment, and other elements of the work environment. When choosing cleaning chemicals, employers should follow the manufacturer’s instructions for use of all cleaning and disinfection products (e.g., concentration, application method and contact time, PPE). Workers should wear masks, gloves and goggles and maintain social distancing at all time in the sites. | Contractor UW, S-PMU/Aden and WSLC Aden. | $ 4000 |
| Working conditions Working at height | Ensure that the roof is well protected by proper parapet without openings and enough and clean space. Ensure proper use of ladders and hiab use by trained workers and inspected, tested regularly by competent inspectors, use of fall prevention devices, including safety belt and lanyard to prevent access to fall hazard area, or fall protection devices such as full body harnesses and head helmet used in conjunction with shock absorbing lanyards. Do not move ladders or hiabs when workers are actually standing on them. Installation of guardrails with mid-rails and toe boards at the edge of any fall hazard area. Inclusion of rescue and/or recovery plans, and equipment to respond to workers after an arrested fall and a fall protection plan should be in place which includes the following aspects: Training and use of temporary fall prevention devices, such as rails or other barriers able to support a weight of 200 pounds, when working at heights equal or greater than two meters or at any height if the risk includes falling through an opening in a work surface. Training and use of personal fall arrest systems, such as full body harnesses and energy absorbing lanyards able to support 5000 pounds. | Contractor UW, S-PMU/Aden and WSLC Aden. | $ 7500 |
| Manual Handling Manual Handling Injuries that includes Fractures Damage to muscles, ligaments and tendons Spinal disc injuries Trapped nerves Abrasions and cuts Burns Hernias | Provide required information and training on manual handling to the site workers. Ensure applying safe handling techniques. Remove space constraints, ensure good housekeeping and providing improved layouts Keep manual handling to one level, improve floor conditions and improve the environmental conditions. The floor must be clean from any obstacles and should be open, clean and well protected. Ensure use of appropriate PPE and safety materials. Addressing potential use of handling aids with matching safety measures. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Excavation Generation of Dust that can be inhaled by laborers, Trench collapse, falls into trenches and/ or tripping over equipment, debris and spoil | Excavation will be less than 50cm depth for the concrete base and cables. Excavation area to be appropriately secured using barricades, fences and precaution tapes Reflective Safety signs to be placed Continuously remove the waste and transfer it to the approved disposal site by the local district authorities | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Life and fire safety prevention measures | Conduct several fire prevention measures during the design preparation, design review, technical specification preparation, work implementation and operation. Fire Prevention measures during design stage: -Selecting proper size of cabling compatible with international standards to avoid overloading/overheating of the cables. -Ensure high quality cables standard outdoor and indoor is applied. -Ensure high quality circuit breakers is provided. Fire Prevention measures during implementation and operations stage: -Detection and fire alarm system -CO2 fire-extinguishers -Powder fire-extinguishers -Emergency Response plan -Provide Fire Safety training and drill for the facility operation staff and technicians. The following fire extinguishers should be provided: Powder extinguisher, according to BS EN 3 Parts 7 to 9 and SS EN3 &UL listed. Wheeled Foam Extinguisher, Approved to EN1866. Working Pressure to be not less than 12 Bar, Test Pressure not less than 22 Bar, Temperature Range (-5/+60), 2 Year Warranty and testing check list of the extinguishers to be provided. Training on fire safety and how to use fire extinguishers shall be provided to staff. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Medium Voltage General Risk | All conductors on which high voltage may be present should be confined within grounded or properly insulated enclosures. Instrumentation cabinets containing high voltage conductors should have safety interlocks on access doors. If confinement of high voltage is not possible, then bare conductors at high voltage must be enclosed within grounded safety enclosures with working interlocks. Except by deliberate breach of the enclosure, contact with bare conductors at high voltage should be impossible without tripping the interlock. All interlocks should be tested during safety review. All permanent setups with bare conductors at medium voltages must be surrounded by a safety fence or other barrier for one-person or unattended operation, with working interlocks in place for unattended operation. The distance between the fence or rope and the exposed conductor must be 1 m or greater. Climbing over ropes is strictly forbidden, and the voltage must be controlled by a switch outside of the test area. Special confinement (fences or other barriers) is not required for two-person operation, but is recommended. Permanent setups with covered or insulated conductors, and unexposed terminals, require one-time approval from the Group Leader, and this approval is sufficient for unattended operation. DANGER HIGH VOLTAGE signs must be on display on all entrances to all test areas where bare conductors are present at both moderate and high voltages. These signs should be in the vicinity of the test area and on the outside of the door leading to the working area. Before touching a high-voltage circuit or before leaving it unattended and exposed, it must be de-energized and grounded with a grounding stick. The grounding stick must be left on the high-voltage terminal until the circuit is about to be re-energized. Grounding sticks must be available near entrances to high-voltage areas. Automatic grounding arrangements or systems that employ audible warning tones to remind personnel to ground the high-voltage equipment are strongly encouraged for two-person operation, and are mandatory for one-person or unattended operation. Two-person: Two-person operation is the normal mode of operation where high or moderated voltages are present with appropriate review and authorization, for systems having no bare or exposed conductors, and where required warning signs, lights, and barriers are present. Make certain that one terminal of each transformer winding used to provide a separately derived system (this excludes the winding connected to the power supply) as well as the transformer or Variac case are properly grounded. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Electricity Shock Thermal burns Muscle, nerve and tissues damage due to electrical shock Fall from height due to sudden electric shock Fatalities or injuries | Inspect equipment and area and apply all safety measures to reduce the risk of any injury to the workers by electricity shock during installation or the users during operation, and apply Hot Work Permit and Electricity Isolation Certificate subject to written approval by the UNOPS engineer provided before implementation of work. Carefully design using appropriate technologies to minimize hazards. Build security fences around electricity areas. Contractor electricians should be well trained and provided with insulated PPE and isolated work tools. Install danger signage in the electrical hazard areas and apply all safety measures to prevent exposures. Avoid working during rainy times Ensure skilled workers are hired for each work. Conduct regular awareness sessions and daily Toolbox Talks on OHS requirements before commencing any work. Periodic inspection to ensure that mitigation measures are implemented and stop any unsafe act or unsafe situation. Emergency response plan to be in place with details and contact of the nearest hospital or medical center, responsibilities are understood for all works, first aid boxes are available and a list of trained first aiders is posted and known by all workers with available transportation. Immediately report all accidental occurrences with serious accident potential such as major equipment failures, contact with high-voltage lines, and exposure to hazardous materials, slides, or cave-ins to UNOPS Contractors shall monitor, keep records and report on the following environmental and social issues: Safety: hours worked, lost time injury (LTI), lost workdays, recordable incidents and corresponding Root Cause Analysis (lost time incidents, medical treatment cases), first aid cases, high potential near misses, and remedial and preventive activities required (for example, revised job safety analysis, new or different equipment, skills training, and so forth). Environmental incidents and near misses: environmental incidents and high potential near misses and how they have been addressed, what is outstanding, and lessons learned. Major works: those undertaken and completed, progress against project schedule, and key work fronts (work areas). ESHS requirements: noncompliance incidents with permits and national law (legal noncompliance), project commitments, or other ESHS requirements. ESHS inspections and audits: by Project Company, Independent Engineer, UNOPS and its implementing partners, or others—to include date, inspector or auditor name, sites visited and records reviewed, major findings, and actions taken. Maintaining a record of injuries and accidents specifying cause and location Provide a list of trained workers, whom will be checked for their training skills. Measures will be implemented onsite and followed by regular monitoring visits. Ensuring the contractor is taking care of the safety of workers while working in the site and give all necessary vaccines to workers to prevent any infection with epidemic and pandemic diseases. | Contractor UW, S-PMU/Aden and WSLC Aden. | 7000 |
| Electrical Burns | when a person gets too close to an electrical wire that is not properly insulated, the air can break down and form a conducting path between them to earth. Coils and capacitors store electrical energy and release it after power has been turned off and should, therefore, be discharged before work commences. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Fire and Explosion | There is risk of fire and explosion when working with Medium Voltage equipment, due to the large fault currents that can flow in the system. Oil Circuit Breakers (OCBs) and Oil Mini Sub Stations (MSS) and Ring Main Panels (RMPs) pose a particular threat.. Heat can build up in wires. A light weight extension lead gets hot when used for heavy duty service. Avoid using extension leads at all, if possible. If they must be used, ensure they can carry the current without overheating. Do not string them overhead, across aisles and under mats, where heat can build up and fully extend them. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Hazardous Substances and Wastes | Ensure proper lifting, handling and existence of containment to prevent any leak of lubricant from the transformer. Ensure all chemicals are stored at safe insulated areas according to their Material Safety Data Sheet (MSDSs) and handled by trained employees | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Environmental pressures on workers (heat strokes, dust storms) | Not working during high temperatures and start the work early in the morning and working in calm weather conditions. raise awareness on the importance of drinking enough water Provide proper PPEs against heat and dust Do not allow working during bad weather, rain, dust storms Provide adequate and suitable breaks and supply workers with drinking/potable water | Contractor and UNOPS | NA |
| Environmental Impacts |  |  |  |
| Air pollution due to emissions from lifting , excavation equipment/transportation equipment which may cause emissions and health issues to workers | Visual observation and applying equipment checklist for inspection to ensure low emission and well-maintained equipment will be only used. Provide workers with proper PPEs Use minimal water and preferably grey water for dust suppression. Use dust sweeping methods | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Noise from construction equipment | Noise should be minimum during lifting operation and installation, and proper mitigation measure and PPE should apply. Ensure all work will take place during daylight Provide ear mufflers to workers | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Soil, surface and ground water | Ensure that work wastes are properly stored and regularly collected and transported to authorized disposal site and arrange for safe path of last destination of E-waste. Ensure waste areas are properly fenced and insulated. Ensure all hazardous materials and wastes are stored at designated areas and insulated from the soil and away from any runoff zones Ensure the presence of spill prevention kits | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Solid waste produced by work accumulated and may pollute the environment. | Ensure that work wastes are properly stored and regularly collected and transported to authorized disposal site and arrange for safe path of last destination of E-waste. Expected waste is the Soil resulted from excavation of 50 cm depth for cabling and earthling. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Biodiversity (urban wildlife) | Urban wildlife may be directly impacted from improper waste management and hazardous material, contaminated water and air and noise disturbances. Mitigation measures include implementing proper air, noise, soil and water, solid and hazardous wastes mitigation measures | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Social Impacts |  |  |  |
| Lack of workers awareness and knowledge on social safeguard issues on gender, SEA and GBV. | Contractor and workers to sign the code of conduct, and ensure workers respected and adhere to the code of conduct. Conduct regular awareness session on site in GBV prevention. GM system is in place to handle any issue on Gender SEA and GBV. GM system for all workers including providing complaints box and complaint means. | Contractor UW, S-PMU/Aden and WSLC Aden. | $ |
| Child Labor | All workers should be more than 18 years old. Verifying age of workers by checking IDs and official documents. Ensure a worker log is available, and all workers are registered. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Visual impacts | Ensure proper waste management and good housekeeping is kept Ensure all construction materials are stored properly and away from the public | Contractor and UNOPS | NA |
| Public Health: Access of public working site. Impacts: Public Exposure to high risk activities (Lifting, Excavation,) Water cuts | Install barriers, danger warning signs and restriction signs to only authorized persons and signs showing the potential danger to the public. And establish barriers around the working site rooftop, equipment area and excavation area. Do not allow public to access working sites in all cases Avoid construction work during academic seasons Ensure proper storage of construction material and fencing the storage area to prevent accessibility. Inform the public on work periods. | Contractor and UNOPS | $ 8000 |
| Poor coordination, planning and sequencing of work could lead to breakage of underground pipes (electric power cables, telephone lines, water distribution. | Inspection and photographic documentation. The contractor and UNOPS engineers should ensure that the site supervisor shall submit daily report on the movement of workers, approved and trained workers only in place and conduct monitoring to ensure Permit To Work PTW applied and workers to be well informed. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |
| Operation and maintenance phase |  |  |  |
| Operation and Maintenance (Staff Health and Safety) | Same mitigation measures for installation will apply for inspection and maintenance as well. | Contractor UW, S-PMU/Aden and WSLC Aden. | NA |

- Environmental and Social Monitoring Plan:

The implementation of the mitigation measures will be monitored accordingly through daily, weekly, monthly and event basis by the contractor, supervisor consultant, UWS-PMU/ADEN and UNOPS engineers as well as by TPM. The following aspects will be monitored (though the list will keep updated to accommodate any emergency or updated aspects that may be recommended by the monitoring reports):

Table(  17  ) : Environmental and Social Monitoring Plan

| Monitoring | Measurements (incl. methods & equipment) | Frequency | Implementation responsibility |
| --- | --- | --- | --- |
|  | Community Health and Safety |  |  |
| Public safety during the work. | Visual observation and photographic documentation of safety measures. Visual observation for installing of warning signs, barricading of working area with safety tapes and fencing/barricades to prevent unauthorized access of public to the working site including worker’s entrance. Visual observations and records of any construction or storage areas without fencing Visual observations of any non-allowed public access Indicators: - Number of recorded complaints - Number of reported incidents | Continuous/Daily and weekly during site inspection visits, On any complaint. | Contractor and UNOPS |
|  | General Environmental Impacts |  |  |
| Dust generation during work. | Visual observation and photographic documentation of equipment induced dust clouds during work activities. Indicators: - Number of recorded complaints - Number of reported incidents | Continuous/ Weekly | Contractor and UNOPS |
| Increased level of noise and vibration. | Site supervision/inspection and documentation to ensure compliance with the noise mitigation measures. Indicators: - Noise level analysis data | Continuous/ Weekly during site inspection. Upon any complaint. | Contractor and UNOPS |
| Air pollution due to emissions from equipment/transportation trucks. | Visual observation and photographic documentation of equipment induced emissions during implementation of activities. Indicators: Presence of dust cloud Number of GRM relating to air emissions. Quantity of consumed fuel | Continuous/ Weekly | Contractor and UNOPS |
| Production, proper disposal and disposal of work’s debris and waste materials. | Inspection and photographic documentation. - Number of recorded complaints - Number of reported incidents | Continuous /Daily | Contractor and UNOPS |
| Poor coordination, planning and sequencing of work could lead to breakage of underground pipes (electric power cables, telephone lines, water distribution. | Inspection and photographic documentation. The contractor and UNOPS engineers should ensure that the site supervisor shall submit daily report on the movement of workers, approved and trained workers in place and conduct monitoring to ensure Permit to Work PTW and TBT applied and workers to be well informed about risks, mitigation measures and OHS requirements before commencing any work. Indicators: Number of recorded complaints | Continuous/Daily | Contractor and UNOPS |
| Operation and Maintenance (cleaning using water) | Ensure that water saving practice is implemented efficiently. Indicators: Number of recorded complaints | Continuous | Facility Administration |
| Occupational Health and Safety |  |  |  |
| Working at Height Activities | Visual inspection to ensure that all working at height activities are monitored and all safety associated instructions are implemented according to OSH requirements. Indicators: - Number of received complaints | Continuous/Daily | Contractor and UNOPS |
| Lifting Operations | Visual inspection to ensure that all lifting activities in the work site are executed safely and as per the standard lifting safety rules. Indicators: - Number of received complaints | Continuous/Daily | Contractor and UNOPS |
| Electricity Operation | Visual inspection to ensure e that all electricity safety rules are implemented, followed and communicated. Ensure that only skilled workers are authorized to perform any electrical operations through qualification inspections Indicators: - Number of injured worker and activity leading to injury. - Number of received complaints | Continuous/Daily | Contractor and UNOPS |
| Manual Handling | Visual inspection to ensure that all manual handling activities are performed according to the OSH manual handling safety rules and instructions. Record any noncompliance. Ensure that the implementation of the safety techniques to control the manual handling risk is monitored continuously. Indicators: - Number of recorded complaints | Continuous/Daily | Contractor and UNOPS |
| Excavation | Visual inspection to ensure that all excavation activities are executed safely and all safety rules are implemented. Record any noncompliance. Indicators: - Number of recorded complaints - Number of reported incidents | Continuous/Daily | Contractor and UNOPS |
| Hazardous Substances and Wastes | Visual inspection to ensure batteries are well placed in a safe and proper ventilated room with appropriate fire extinguisher and conduct regular monitoring. Visual inspection to ensure proper battery recycling and disposal procedures are implemented. Record any noncompliance and take photographic proof. Indicators: - Number of recorded complaints - Number of reported incidents | Continuous | Contractor and UNOPS |
| Infection by Covid-19 | Visual inspection to ensure that health, safety and hygiene awareness are followed and communicated. Visual inspection to ensure that all health, safety and hygiene materials are provided. Record any noncompliance in wearing PPEs. Indicators: - Number of recorded complaints - Number of reported incidents | Continuous | Contractor and UNOPS |
| Work related accidents and injuries. | Inspection and photographic documentation Indicators: - Number of recorded complaints - Number of reported incidents | Continuous daily | Contractor, UNOPS and TPM |
| Poor onsite housekeeping, toilet and water supply, leading to illness and disease. | Visual site inspection and photographic proof Indicators: - Number of recorded complaints | Weekly during site inspection and regularly by TPM | UNOPS and TPM |
| The risk of employing children for work activities. | Site inspection, checking and documentation of contractor employee records and checking/verifying age documents. Indicators: - Number of hired children | Weekly during site inspection and regularly by TPM | UNOPS and TPM |
| Low aesthetic value of landscape such as accumulation of waste and debris in the site. | Site inspection and documentation of general landscape Indicators: - Number of recorded complaints Number of reported incidents | Continuous/Monthly | Contractor and UNOPS |
| Complaints. | Indicators: Number of GRM Reports and number of solved issues | Continuous/weekly | Contractor and UNOPS |
| Safety | Hours worked, recordable incidents and corresponding Root Cause Analysis (lost time incidents, medical treatment cases, first aid cases, high potential near misses, remedial and preventive measures required (for example, revised job safety analysis, new or different equipment, manual handling and skills training etc. Indicators: - Number of recorded complaints Number of reported incidents | Continuous/ Daily | Contractor and UNOPS |
| Environmental incidents and near misses | Records of environmental incidents and high potential near misses and how they have been addressed, how they have been reported, incidents review, and lessons learned. Monitoring working in good weather conditions. Indicators: - Number of recorded complaints Number of reported incidents | Continuous/ Daily | Contractor and UNOPS |
| Major works: | Work undertaken and completed, progress against project schedule, and key work fronts (work areas). Indicators: - Number of recorded complaints | Continuous/ Daily | Contractor and UNOPS |
| E&S and OHS requirements: | Non-compliance with OHS requirements, national law (legal noncompliance), project commitments and E&S requirements. Indicators: - Number of recorded complaints | Continuous/ Daily | Contractor and UNOPS |
| E&S/OHS inspections and audits: | By contractor, engineer, or others, including authorities to include date, inspector or auditor name, sites visited and records reviewed, major findings, and actions taken. Indicators: - Number of recorded complaints | Continuous/ Daily | Contractor and UNOPS |
| Workers: | Number of workers, indication of origin (expatriate, local, nonlocal nationals), gender, age with evidence that no child labor is involved, and skill level (unskilled, skilled, supervisory, professional, management). Indicators: - Number of recorded complaints | Continuous/ Daily | Contractor and UNOPS |
| Training on E&S issues | Including dates, number of trainees, and topics. Indicators: - Number of Training on E&S | Continuous / Weekly | Contractor and UNOPS |
| Footprint management: | Details of any work outside boundaries or major off-site impacts caused by ongoing work—to include date, location, impact, and actions taken. Indicators: - Number of recorded complaints Number of reported incidents | Continuous/ Monthly | Contractor and UNOPS |
| External stakeholder engagement: | Highlights, including formal and informal meetings, and information disclosure and dissemination—to include a breakdown of women and men consulted and themes coming from various stakeholder groups, including vulnerable groups (e.g., disabled elderly, children, etc.). Indicators: - Number of women and men consulted. - Stakeholder groups engagement | Continuous/ Monthly | Contractor and UNOPS |
| Details of any security risks | Details of risks the contractor may be exposed to while performing its work—the threats may come from third parties external to the project. Indicators: - Number of recorded complaints | When occurred | Contractor and UNOPS |
| Worker grievances: | Number of grievances and details including occurrence date, grievance, and date submitted; actions taken and dates; resolution (if any) and date; and follow-up yet to be taken grievances listed should include those received since the preceding report and those that were unresolved at the time of that report. Indicators: - Number of recorded grievances - Number of grievances solved | Continuous | Contractor, UNOPS and TPM |
| External stakeholder grievances | Number of grievances and date submitted, action(s) taken and date(s), resolution (if any) and date, and follow-up yet to be taken grievances listed should include those received since the preceding report and those that were unresolved at the time of that report. Grievance data should be gender disaggregated. Indicators: - Number of recorded grievances Number of grievances solved | Continuous/ Monthly | Contractor, UNOPS and TPM |
| Major changes to contractor’s environmental and social practices. | Monitor and record contractor’s environmental and social practices noncompliance through visual inspections. Indicators: - Number of recorded complaints Number of reported incidents | Continuous/ Monthly | Contractor, UNOPS and TPM |
| Deficiency and performance management | Actions taken in response to previous notices of deficiency or observations regarding E&S performance and/or plans for actions to be taken—these should continue to be reported until UNOPS determines the issue is resolved satisfactorily. Indicators: - Number of recorded complaints - Number of Action taken. | Continuous/ Monthly | Contractor, UNOPS and TPM |
| Complaints Handling | Complaints register will be kept on site and this will feed into the GRM. Details of complaints received will be incorporated into the audits as part of the monitoring process. Indicators: - Number of recorded complaints | Continuous/ Monthly | Contractor and UNOPS |
| GBV and SEA issues | Number of reported and registered cases of SEA/SH through project GM Number of reported cases of contractors noncompliance to PSEA/SH obligations in work sites Indicators: - Number of reported and registered cases of the SEA/SH through project GM. Number of reported cases of contractors noncompliance to PSEA/SH obligation on work sites. | Continuous/ Monthly | Contractor, UNOPS and TPM |
| Workers are not wearing full PPEs | Visual inspection and photographic documentation | Continuous daily | Contractor, UNOPS and TPM |
| Operation and Maintenance (Staff Health and Safety and community health and safety) | Ensure same monitoring measures are implemented during operation and maintenance | Continuous daily | UNOPS, Contractor and Facility Administration. |
| All costs will be under the responsibility of each party including the contractor and will be included in the contract BoQ. |  |  |  |

###### PUBLIC CONSULTATION

Urban water and sanitation project engineer has conducted consultation with 60 persons (22 females and 38male) in sub-projects areas in  August and September, 2022, where individual residents interviewed were informed with the sub-project activities that will be conducted in the target sites of the four districts (Crater, Malla'a, Tawahi, and Khormaksar). As well as a clear explanation of the benefits and services of the project and the potential problems and risks that the residents of the target sites may encounter during and after the execution of the project.

Topics of the consultations are to:

- Inform beneficiaries about the activities to be undertaken and the sub-projects timetable;

- Document and address the local beneficiaries' concerns, expectations and feedback;

- Ensure participation of sub-project beneficiaries both females and males with awareness on their rights to participate and give feedback including GRM contacts, anonymous complaints and escalation of grievances if not satisfied with the resolution and action taken;

- Discuss the positive impacts that the sub-project will have and the sub-project potential negative impacts and proposed mitigation measures to avoid possible impacts.

- Raise the awareness on the protective measures from Covid-19 Pandemic.

- Provide awareness to the consulted persons both males and females on their rights to participate in all sub-project implementation phases, give their feedback and raise their concerns.

- The Sub-project activities were discussed with the consulted people.

- The sub-project environmental and social impacts and the planned mitigation measures discussed with the consulted people in details.

Stakeholder Engagement Plan

In the context of YEHCP, YIUSEP, and co-financing- UNOPS, in consultation with the World Bank, must develop and implement a Stakeholder Engagement Plan (SEP) proportional to the nature and scale of the project including the rehabilitation of the sewer network sub-projects and its potential risks and impacts.  The SEP must:

●	Describe the timing and methods of engagement with stakeholders throughout the life cycle of the project, distinguishing between project-affected parties and other interested parties.

●	Describe the range and timing of information to be communicated to project-affected parties and other interested parties, as well as the type of information to be sought from them.

●	Take into account the main characteristics and interests of the stakeholders, and the different levels of engagement and consultation that will be appropriate for different stakeholders.

●	Set out how communication with stakeholders will be handled throughout project preparation and implementation.

Describe the measures that will be used to remove obstacles to participation, and how the views of differently affected groups will be captured.  Where applicable, the SEP will include differentiated measures to allow the effective participation of those identified as disadvantaged or vulnerable. Dedicated approaches and an increased level of resources may be needed for communication with such differently affected groups so that they can obtain the information they need regarding the issues that will potentially affect them.

When the stakeholder engagement with local individuals and communities depends substantially on community representatives, UNOPS will make reasonable efforts to verify that such persons do, in fact, represent the views of such individuals and communities, and that they are facilitating the communication process in an appropriate manner.

The SEP for YEHCP, YIUSEP II, and co-financing is a standalone document that is disclosed separately.

6.1 Consultation Findings and Feedback

Interviews were conducted by the Female Social Facilitators with the stakeholders and feedback was updated and collected by questionnaire through sites visit . The interviewer started by a brief explanation of the nature and objective of the sub-projects and potential impact with proposed mitigation measures. The local residents emphasized on the urgent need of rehabilitation of the existing sewerage system network to provide protection for them from pollution and disease due to the overflow of the sewage in the streets and the direct disposal of raw wastewater into the sea and their consequent impacts on environment and health, where rehabilitation of sewer systems will provide proper sanitation service free from blockage, overflows, and disease spread.

The consulted people main concern was the temporary impact on access to their residence during the work activity. All safety mitigation measures were discussed in details with the consulted persons.

6.2 Consultation during implementation

Consultations will continue during implementation with the previously consulted representative local communities to assess beneficiaries’ satisfaction on implementation of mitigation measures and accommodation of all their concerns and will conduct broader consultation with more beneficiaries and wider representatives of local communities’ citizens.

To ensure citizen engagement during subproject implementation, the following mechanism will be adapted:

- Continue consultation with local communities by interviews and using questionnaires to assess beneficiaries’ satisfaction on the implementation of activities and safeguards.

- Hold interviews during implementation of subproject in the targeted areas with both male and female citizens/ beneficiaries conducted by male and female moderators/facilitators in separate sessions;

- Utilize GIS-based portal mapping for all activities including sub projects supported to promote transparency to reach more citizens.

- Utilize citizen engagement findings conducted for Yemen emergency Human Capital project YEHCP, Yemen Integrated Urban Service Emergency Project II (YIUSEP II), and other financing which include a wide range of stakeholders.

- Receive feedback from citizens through the UWS-PMU/ADEN and UNOPS established GRM during implementation of subprojects.

- Explore using UNOPS’ remote monitoring tools for engaging citizens in monitoring and quality assurance of subproject activities.

###### 7 . Grievance Redress Mechanism GRM

Bank procedures require that Grievance Redress Mechanisms (GRMs) be established and operational prior to commencement of the sub-projects, and that they continue to operate for one year following completion of the works for third party settlement of disputes arising from resettlement. This GRM should take into account the availability of judicial recourse as well as traditional and community dispute resolution mechanisms.

UNOPS has established Grievance Redress Mechanism (GRM) for Yemen emergency Human Capital project YEHCP, Yemen Integrated Urban Service Emergency Project II (YIUSEP II), and other financing to enable beneficiaries to communicate their concerns regarding the project activities. More specifically, the GRM details the procedures that communities and individuals, who believe they are adversely affected by the project or a specific sub-project, can use to submit their complaints, as well as the procedures used by UNOPS and UWS-PMU/ADEN to systematically register, track, investigate and promptly resolve complaints.

Accordingly, hard copies of the translated application of the GRM (attached in Annex 2) were provided to interviewed people and they have been informed that the GRM contact information will be posted at the sub-project site to ensure any grievance can be addressed in an amicable manner. During monitoring, UWS-PMU/ADEN will conduct inspections for complaints. Resolving complaints at community level is always encouraged to address the problem that a person may have during implementation and/or operation phase.

UWS-PMU/ADEN maintains records for grievances and complaints including minutes of discussions, recommendations and resolutions made.

7.1 Registering Complaints

UWS-PMU/ADEN has developed GRM system for Yemen emergency Human Capital project YEHCP, Yemen Integrated Urban Service Emergency Project II (YIUSEP II), and other financing along with UNOPS GRM. It is to improve access to GRM. this is to provide multiple access points to the GRM for beneficiaries to voice their concerns. These access points will be advertised at sub-project level and put on the sign boards on each sub-project site, and include GRM contact information including hotline, landline, mobile SMS, email and website:

UNOPS GRM

Address:	Haddah Street, former European Union Office Building, Sana’a Tel:

Landline +967 1 504914 and +967 1 504915, Tool Free 8000190

Focal Point:	Marwa Obaid

SMS and WhatApp: +967 739888388

Email:  grm-yemen@unops.org Website: www.unops.org.

The GRM contact information will be posted in Arabic and be communicated through multiple channels to ensure all groups can easily access contact information and relevant mechanisms to provide feedback.

Grievances can be brought up by affected people including workers in case of: (i) non-fulfillment of contracts or agreements; (ii) disputes related to destruction of assets or livelihoods; (iii) disturbances caused by rehabilitation activities; (iv) concerns around safety and protection related to project’s activities.

Anonymous complaints will be admissible to their attention verbally or in writing by sub-projects affected communities or individuals, and will relay these concerns in writing to UNOPS on a next day basis. UNOPS will determine if these concerns rise to the level of a complaint.

UNOPS will register the complaint in a dedicated log by gender, age, and location, and include a copy of the complaint and supporting documents. A draft template for registering grievances is annexed.

UNOPS will record and document complaints received in the sub-project file and the sub-project progress reports, including the number and type of complaints and the results of their resolution.

7.2 Tracking, Investigating and Resolving Complaints

The GRM log maintained by UNOPS will track the date the complaint was received, date responded to, the type of response, and if the complaint was resolved to the satisfaction of the plaintiff.

The ESSO will coordinate with local partners, local field staff and local government officials to ensure prompt follow up action in response to each complaint. More specifically, the GRM focal point wills forename complaints:

- Inform the plaintiff if the complaint is accepted or rejected within 3 days from receiving the complaint; any technical input from project engineers; if necessary, the response will require input from project engineers.

If the complaint is accepted, send the plaintiff an officially stamped review card indicating:

- plaintiff name or legal representative

- plaintiff address

- complaint title

- review date

- list of annexes submitted with the complaint.

- The plaintiff has a right to submit an anonymous complaint without name or address.

- Work with engineers, local partners, and contractors to resolve the complaint within 28 days of its submission.

Grievance Categories

The grievance could be among but not limited to the following categories:

- Access to project benefits (e.g., no or insufficient jobs created for local communities);

- Non-equal distribution of project services among target beneficiaries;

- Disputes (e.g. matters raised by/related to beneficiaries).

- Disturbance (e.g. noise, traffic road access and public safety etc.).

- GBV/SH grievance

- Internal grievance (worker’s grievance)

Steps to handle GRM

Publicizing: stakeholder’s consultation, printed materials;

Receiving and registering complaints: staff at local and central level who will be responsible for receiving registering and tracking complaints;

Acknowledging:

The GRM staff (team) acknowledges receipt of the complaint within 2-3 working days. Inform the complainant on the eligibility of his/her complaint;

Anonymous complaints:

To be studied as well;

Reviewing and investigating, collect, review and analyze related documents;

Conducting interviews of the involved persons, officers and staff;

Analyzing the related national legislations & regulations, World Bank Policies & Guidelines and UNOPS standards;

Summarizing the facts and findings;

Developing resolution options: based on the collected evidence, the GRM staff (team) will draw conclusions, make recommendations for solutions, and present it to the complainant;

If the solution is not accepted, complaint will be presented to the Program Manager as a second level to appeal who can make the resolution and/or can delegate an arbitrary to investigate on the complaint and propose recommendations for resolution;

Implementing resolution: If the solution is accepted, then will be implemented;

Monitoring and closing: the complaint should be monitored for a reasonable period of time to make sure that the complainant does not express additional concerns, and then the complaint could be closed.

Reporting (recording): prepare concise summary reports of the complaints received, with the resolutions taken and status of resolutions implementation, and filled in the database with detailed records. Table 10 summarizes the complaints management review process.

8. Reporting of ESMP

The UWS-PMU ESSO will report to UNOPS on monthly basis the implementation of the ESMP and UNOPS will report the ESMP implementation to the WB. There will be also irregular reports base on the situation and updates. The UWS-PMU Supervision Consultant will monitor and report monthly and irregularly on the level of mitigation measures implementation and environmental issues. The contractors shall monitor, keep records and report on the following environmental and social issues: safety, Environmental incidents and near misses, major works, ESHS requirements, ESHS inspections and audits: workers, training on ESHS issues, footprint management, external stakeholder engagement, details of any security risks, worker grievances, external stakeholder grievances, major changes to Contractors environmental and social practices, deficiency and performance management.

The following table provides indicative reporting plan. Table (18)

Reporting Plan :

| What | How | Who | When |
| --- | --- | --- | --- |
| Compliance level to the ESMP including environmental and social issues, OHS, GM, etc. | Based on monitoring and inspections, log, the consultant reports, GM log | Environmental Specialist | Monthly from UWS-PMU/ADEN to UNOPS and quarterly from UNOPS to WB. |
| Compliance level to the ESMP and environmental and social issues: safety, environmental incidents and near misses, major works, ESHS requirements, ESHS inspections and audits: workers, training on ESHS issues, footprint management, external stakeholder engagement, details of any security risks, worker grievances, external stakeholder grievances, major changes to Contractors environmental and social practices, deficiency and performance management. | Consultant based on monitoring, inspection, records, logs, contractor reports. | UWS-PMU/ADEN Supervision consultant | Monthly and based on cases |
| Environmental and social issues: safety, environmental incidents and near misses, major works, ESHS requirements, ESHS inspections and audits: workers, training on ESHS issues, footprint management, external stakeholder engagement, details of any security risks, worker grievances, external stakeholder grievances, major changes to Contractors environmental and social practices, deficiency and performance management. | Contractor ESSO based on monitoring, inspection, records, and logs. | Contractor | Monthly and based on cases |

# Annex 2: GRM Complaint and Suggestion Form

| Yemen Integrated Urban Service Emergency Project YEHCP Sample of GRM Complaint and Suggestion Form | ستمارة توثيق ومتابعة شكاوى المستفيدين من المشروع الطارئ للخدمات الحضرية المتكاملة – المرحلة الثانية | المشروع الطارئ للخدمات الحضرية المتكاملة المرحلة الثانية نموذج لألية التظلمات والشكاوى |
| --- | --- | --- |

"Documenting and Monitoring Complaints Form of

Beneficiaries of Yemen Emergency Human Capital Project YEHCP

"

| الاسم الثلاثي للمستفيد: Beneficiary Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| رقم البطاقة الشخصية: ID No. |  |  | رقم الهاتف للمتابعة Tel Number for follow up |  |  |
| العنوان الدائم: Permanent Address |  |  |  |  |  |
| اسم النشاط المنفذ (مركز/وحدة) Name of Activity under implementation |  |  |  |  |  |
| مكان تنفيذ النشاط: Place of activity under implementation | القرية: Village | المديرية: District |  | المحافظة: Governorate |  |

| نوع الشكوى Complaint Type | إدارية Administrative | فنية Technical | مالية Financial | أخرى Other |
| --- | --- | --- | --- | --- |

موضوع الشكوى:

Complaint Subject

| الوضع الحالي: Current Situation |  |  |  |
| --- | --- | --- | --- |
| أسباب المشكلة: Reason of the problem |  |  |  |
| التاريخ: Date |  | توقيع صاحب الشكوى: Complainant Signature |  |

- الجهة التي يجب أن يقدم لها الشكوى:..UNOPS – Tool Free No 8000190 Tel: 01 504914/915 - SMS: 739888388 Email: GRM.yemen@unops.org........................................................................................

The entity, which the complaint should be forwarded to:

-الرأي في جدية الشكوى:.......................................................................................................

Opinion on the seriousness of the complaint

-الجهة المحول لها الشكوى :.........................................................................................................

The complaint transferred to

- المدة الزمنية اللازمة للبت في الشكوى:...........................................................................................

Time required for response

-مدى رضى المستفيد عن الاستجابة  لحل شكواه:..................................................................................

Satisfaction of beneficiary in responding to his/her complaint

| الإجراءات المتخذة : Action taken |  |  |  |
| --- | --- | --- | --- |
| ما ترتب عليها من نتائج: The results of the action taken |  | التاريخ: Date |  |

اسم مستلم الشكوى ووظيفته: ................................................

Name of person received the complaint and his/her position

التاريخ/ Date :   ..................................                                                       توقيع الموظف المختص/ Signature :......................................

# Annex 3:  Environmental and Social Requirements for Contractors

These requirements for contractors are generic and clauses applied as relevant to each sub-project.

Contractors shall meet the following Environmental, Health, Safety and Social (including labor) requirements – thereafter called ESHS requirements.

The ESHS requirements include 10 sections

- Contractor Environmental and Social Management Plan (C-ESMP)

- ESHS Training

- Construction Site Management

- Occupational Health and Safety (OHS)

- Road safety and Traffic Safety

- Chance Find Procedures

- Emergency Preparedness and Response

- Stakeholder Engagement

- Code of Conduct

- Contractor Environmental and Social Reporting

## Contractor Environmental and Social Management Plan (C-ESMP)

- Prepare and submit to UNOPS for approval a Contractor Environmental and Social l and Social Management Plan (C-ESMP).

- Include in the C-ESMP a detailed explanation of how the contractor’s performance will meet the ESHS requirements.

- Ensure that sufficient funds are budgeted to meet the ESHS requirements, and that sufficient capacity is in place to oversee, monitor and report on C-ESMP performance.

- Put in place controls and procedures to manage their ESHS performance.

- Get prior written approval from UNOPS Engineers before starting construction or rehabilitation activities.

## ESHS Training

- Determine ESHS training needs in collaboration with UNOPS.

- Maintain records of all ESHS training, orientation, and induction.

- Ensure, through appropriate contract specifications and monitoring that service providers, as well as contracted and subcontracted labor, are trained adequately before assignments begin.

- Demonstrate that its employees are competent to carry out their activities and duties safely.  For this purpose, the Contractor shall issue a Competence Certificate for every person working on site (relative to aspect of work assignment) that specifies which tasks can be undertaken by which key personnel.

## Orientation Training

- Provide ESHS orientation training to all employees, including management, supervisors, and workers, as well as to subcontractors, so that they are apprised of the basic site rules of work at/on the site and of personal protection and preventing injury to fellow employees.

- Training should consist of basic hazard awareness, site-specific hazards, safe work practices, and emergency procedures for fire, evacuation, and natural disaster, as appropriate.  Any site-specific hazard or color coding in use should be thoroughly reviewed as part of orientation training.

## Visitor Orientation

- Establish an orientation program for visitors, including vendors that could access areas where hazardous conditions or substances may be present.

- Visitors shall not enter hazard areas unescorted.

- Ensure that visitors shall always be accompanied by an authorized member of the contractor, or a representative of UNOPS or of its implementing partners, who has successfully fulfilled the ESHS orientation training, and who is familiar with the project site construction hazards, layout, and restricted working areas.

## New Task Employee and Contractor Training

- Ensure that all workers and subcontractors, prior to commencement of new assignments, have received adequate training and information enabling them to understand work hazards and to protect their health from hazardous ambient factors that may be present.  The training should adequately cover the step by step process that is needed for Project activities to be undertaken safely, with minimum harm to the environment, including:

Knowledge of materials, equipment, and tools.

Known hazards in the operations and how they are controlled.

Potential risks to health.

Precautions to prevent exposure.

Hygiene requirements.

Wearing and use of protective equipment and clothing.

Appropriate response to operation extremes, incidents and accidents.

## Construction Site Management

### Vegetation

- Prevent any unnecessary destruction, scarring, or defacing of the natural surroundings in the vicinity of the construction site.

- Protect all trees and vegetation from damage by construction operations and equipment, except where clearing is required for permanent works, approved construction roads, or excavation operations.

- Re-vegetate damaged areas on completion of the Works, and for areas that cannot be re vegetated, scarifying the work area to a condition that will facilitate natural re vegetation, provide for proper drainage, and prevent erosion.

- Use, as much as possible, local species for replanting and species that are not listed as a noxious weed or invasive species.

- Repair, replant, reseed or otherwise correct, as directed by UNOPS or its representative, and at the Contractor’s own expense, all unnecessary destruction, scarring, damage, or defacing of the landscape resulting from the Contractors operations.

- Transport labor and equipment in a manner to avoid as much as possible damage to grazing land, crops, and property.

## Protection of the Existing Installations

- Safeguard all existing buildings, structures, works, pipes, cables, sewers, or other services or installations from harm, disturbance or deterioration during construction activities.

- Coordinate with local authorities to identify existing infrastructure that might not be visible.

- Repair any damage caused by the Contractor’s activities, in coordination with concerned authorities.

- Take all reasonable precautions to prevent or reduce any disturbance or inconvenience to the owners, tenants or occupiers of properties to the construction activities, and more generally to the public.

- Maintain safe access to public and private properties that might be affected by construction activities.  If necessary, provide acceptable alternative means of passage or access to the satisfaction of the persons affected.

- Avoid working during night hours.

## Waste from Construction Activities

- Collect and properly store and manage all solid wastes and hazardous wastes resulting from the construction activities, including construction debris and spoils, to prevent the contamination of soil and groundwater. Hazardous E-waste should be managed stored and disposed according to widely accepted guidelines. In case chemicals are present they should be stored and disposed according to their Material Safety Data Sheets (MSDSs)

- Remove unneeded excavation material from construction sites as soon as possible.

- Agree with relevant municipalities about solid waste disposal during construction.

- Carefully select waste disposal sites, to be approved by UNOPS or its implementing partner.

- Minimize littering of roads by ensuring that vehicles are licensed and loaded in such a manner as to prevent falling off or spilling of construction materials, and by sheeting the sides and tops of all vehicles carrying mud, sand, other materials or debris.

- Transfer construction waste to assigned places in the selected waste disposal sites with documented confirmation.

- Properly dispose of solid waste and hazardous wastes and debris at designated permitted sites waste disposal sites allocated by the local authorities, and obtain a receipt of waste from the authorized landfill authority.

## Air Quality

The Contractor shall:

- Use dust control methods, such as covers, water suppression, or increased moisture content for open materials storage piles, or controls, including air extraction and treatment through a bag house or cyclone for material handling sources, such as conveyors and bins.

- Use water suppression for control of loose materials on paved or unpaved road surfaces. Oil and oil by-products is not a recommended method to control road dust.

- Use wheel washes at quarries, ready-mix plants, construction sites, and other facilities to prevent track-out of mud, dust and dirt on to public road.

- Regularly clean road surfaces within the construction sites to remove accumulated fine material, and regularly clean transportation vehicles.

- Cover open bodied trucks handling sand, gravel or earth.

- Minimize smoke from diesel engines by regular and proper maintenance, in particular by ensuring that the engine, injection system and air cleaners are in good condition.

## Hazardous and Toxic Materials

The Contractor shall take precautions relative to the conditions specified herein.

- Train workers regarding the handling of hazardous materials.

- Store hazardous materials as per the statutory provisions of the Manufactures, Storage and Import of Hazardous Chemicals Rules (1989), under the Environment (Protection) Act, 1986.

- Provide adequate secondary containment for fuel storage tanks and for the temporary storage of other fluids such as lubricating oils and hydraulic fluids.

- Use impervious surfaces for refueling areas and other fluid transfer areas.

- Train workers on the correct transfer and handling of fuels and chemicals and the response to spills.

- Provide portable spill containment and cleanup equipment on site and training in the equipment deployment.

- Deposit or discharge toxic liquids, chemicals, fuels, lubricants and bitumen into containers for salvage or subsequent removal to off-site locations.

- Treat hazardous waste separately from other waste.

- Avoid the storage or handling of toxic liquid adjacent to or draining into drainage facilities.

- Keep absorbent materials or compounds on Site in sufficient quantities corresponding to the extent of possible spills.

- Locate landfill pits for the disposal of solid waste at least 100 m from water courses, and fencing them off from local populations.

- Ensure adequate primary treatment of sanitation effluents and installing septic tanks away from village watering points.

## Area Signage

- Appropriately mark hazardous areas.

- Install warning signs

- Ensure that signage is in accordance with international standards and is well known to, and easily understood by workers, visitors and the general public as appropriate.

- Demarcate work sites with safety tape, fencing or barricades, as appropriate, to prevent unauthorized access to the construction sites

- Safeguard public safety by covering holes and by installing guardrails along temporary pathways.

## Decommissioning of Camps, Worksites and Plant

- Clear construction sites of any equipment or waste, and ensuring that the sites are free from contamination.

- Dispose of or recycle any equipment or waste in an appropriate and environmentally sound manner.

- Hand construction sites over to the original owners, taking into account his/her wishes and national legislation.

## Health and Safety

### Severe Weather and Facility Shutdown

- Design and build work place structures to withstand the expected elements for the region and designate an area designated for safe refuge, if appropriate.

- Develop Standard Operating Procedures (SOPs) for project or process shut-down, including an evacuation plan.

### Lavatories and Showers

- Provide adequate lavatory facilities (toilets and washing areas) for the number of people expected to work at the construction sites, and make allowances for segregated facilities, or for indicating whether the toilet facility is “In Use” or “Vacant”.

- Provide toilet facilities with adequate supplies of hot and cold running water, soap, and hand drying devices.

- Where workers may be exposed to substances poisonous by ingestion and skin contamination may occur, provide facilities for showering and changing into and out of street and work clothes.

### Potable Water Supply

- Provide adequate supplies of potable drinking water from a fountain with an upward jet or with a sanitary means of collecting the water for the purposes of drinking

- Ensure that water supplied to areas of food preparation or for the purpose of personal hygiene (washing or bathing) meets drinking water quality standards

### Clean Eating Area

- Where there is potential for exposure to substances poisonous by ingestion, make suitable arrangements to provide clean eating areas where workers are not exposed to the hazardous or noxious substances.

### Personal Protective Equipment (PPE)

- Identify and provide at no cost appropriate PPE to workers, the workers of subcontractors, as well as to visitors, which gives adequate protection without incurring unnecessary inconvenience to the individual.

- Ensure that the use of PPE is compulsory.

- Provide sufficient training in the use, storage and maintenance of PPE to its workers and workers of its subcontractors.

- Properly maintain PPE, including cleaning when dirty and replacement when damaged or worn out;

- Determine requirements for standard and/or task-specific PPE based on of Job specific Safety Analysis (JSA).

- Consider the use of PPE as a last resort when it comes to hazard control and prevention, and always refer to the hierarchy of hazard controls when planning a safety process.

### Noise

Institute appropriate measures to reduce the exposure of workers to construction noise, including but not limited to:

- Avoid exposure to a noise level greater than 85 dB(A) for a duration of more than 8 hours per day without hearing protection.  In addition, no unprotected ear should be exposed to a peak sound pressure level (instantaneous) of more than 140 dB(C).

- Enforce the use of hearing protection should be enforced actively when the equivalent sound level over 8 hours reaches 85 dB(A), the peak sound levels reach 140 dB(C), or the average maximum sound level reaches 110 dB(A).

- Provide hearing protective devices capable of reducing sound levels at the ear to at most 85 dB(A).

- Reduce the “allowed” exposure period or duration by 50 percent for every 3 dB(A) increase in in excess of 85 dB(A).

- Perform periodic medical hearing checks on workers exposed to high noise levels.

- Rotate staff to limit individual exposure to high levels.

- Install practical acoustical attenuation on construction equipment, such as mufflers.

Use silenced air compressors and power generators

Keep all machinery in good conditions.

Install exhaust silencing equipment on bulldozers, compactors, crane, dump trucks, excavators, graders, loaders, scrapers and shovels.

- Post signs in all area where the sound pressure level exceeds 85 dB(A).

- Shut down equipment when not directly in use.

- Provide advance notice to occupants if an activity involving high level impact noise is in close proximity to buildings.

### First Aid and Accidents

- Ensure that qualified first-aid by qualified personnel is always available.  Appropriately equipped first-aid stations should be easily accessible throughout the place of work.

- Provide workers with rescue and first-aid duties with dedicated training so as not to inadvertently aggravate exposures and health hazards to themselves or their co- workers.  Training would include the risks of becoming infected with blood–borne pathogens through contact with bodily fluids and tissue.

- Provide eye-wash stations and/or emergency showers close to all workstations where immediate flushing with water is the recommended first-aid response.

- Provide dedicated and appropriately equipped first-aid room(s) where the scale of work or the type of activity being carried out so requires.

- Equip first aid stations and rooms with gloves, gowns, and masks for protection against direct contact with blood and other body fluids.

- Make widely available written emergency procedures for dealing with cases of trauma or serious illness, including procedures for transferring patient care to an appropriate medical facility.

- Immediately report all accidental occurrences with serious accident potential such as major equipment failures, contact with high-voltage lines, and exposure to hazardous materials, slides, or cave-ins to UNOPS.

- Immediately investigate any serious or fatal injury or disease caused by the progress of work by the Contractor, and submit a comprehensive report to UNOPS.

### Communicable Diseases

The Contractor shall implement a combination of behavioral and environmental modifications to mitigate communicable diseases:

- Conduct Information, Education and Consultation Communication (IEC) campaigns, at least every other month, addressed to all construction site staff (including all the Contractor’s employees, all subcontractors of any tier, consultants' employees working on the site, and truck drivers and crew making deliveries to the site for Works and Services executed under the Contract, concerning the risks, dangers and impact, and appropriate avoidance behavior of communicable diseases.

- Provide treatment through standard case management in on-site or community health care facilities.

- Ensure ready access to medical treatment, confidentiality and appropriate care, particularly with respect to migrant workers.

- Promote collaboration with local authorities to enhance access of workers families and the community to public health services and ensure the immunization of workers against common and locally prevalent diseases.

- Provide basic education on the conditions that allow the spread of other diseases such as COVID-19, Lassa fever, Cholera and Ebola. The training should cover sanitary hygiene education.

- Prevent illness in immediate local communities by:

Implementing an information strategy to reinforce person-to-person counselling addressing systemic factors that can influence individual behavior as well as promoting individual protection, and protecting others from infection.

Training by health workers in disease treatment.

Conducting immunization programs for workers in local communities to improve health and guard against infection.

Providing health services.

### COVID-19

In the context of the COVID-19 pandemic, Contractors shall develop and implement measures to prevent or minimize an outbreak of COVID-19, and develop procedures indicating what should be done if a worker gets sick.  The measures shall include:

- Assessing the characteristics of the workforce, including those with underlying health issues or who may be otherwise at risk.

- Confirming that workers are fit for work, including temperature testing and refusing entry to sick workers.

- Considering ways to minimize entry/exit to site or the workplace, and limiting contact between workers and the community/general public

- Training workers on hygiene and other preventative measures, and implementing a communication strategy for regular updates on COVID-19 related issues and the status of affected workers.

- Treating workers who are or should be self-isolating and/or are displaying symptoms

- Assessing risks to continuity of supplies of medicine, water, fuel, food and PPE, taking into account international, national and local supply chains

- Reducing, storing and disposing of medical waste

- Adjusting work practices, to reduce the number of workers and increase social distancing

- Expanding health facilities on-site compared to usual levels, developing relationships with local health care facilities and organize for the treatment of sick workers

- Building worker accommodations further apart, or having one worker accommodation in a more isolated area, which may be easily converted to quarantine and treatment facilities, if needed

- Establishing a procedure to follow if a worker becomes sick (following WHO guidelines)

- Implementing a communication strategy with the community, community leaders and local government in relation to COVID-19 issues on the site.

### Vector-Borne Diseases

Reducing the impact of vector-borne disease on the long-term health of workers is best accomplished by implementing diverse interventions aimed at eliminating the factors that lead to disease.  The Contractor, in close collaboration with community health authorities, shall implement an integrated control strategy for mosquito and other arthropod-borne diseases that includes the following measures:

- Prevent of larval and adult propagation through sanitary improvements and elimination of breeding habitats close to human settlements

- Eliminate unusable impounded water

- Increase water velocity in natural and artificial channels

- Consider the application of residual insecticide to dormitory walls

- Implement integrated vector control programs

- Promote the use of repellents, clothing, netting, and other barriers to prevent insect bites

- Use chemoprophylaxis drugs by non-immune workers and collaborating with public health officials to help eradicate disease reservoirs

- Monitor and treat circulating and migrating populations to prevent disease reservoir spread

- Collaborate and exchange in-kind services with other control programs in the project area to maximize beneficial effects

- Educate project personnel and area residents on risks, prevention, and available treatment

- Monitor communities during high-risk seasons to detect and treat cases

- Distribute appropriate education materials

- Follow safety guidelines for the storage, transport, and distribution of pesticides to minimize the potential for misuse, spills, and accidental human exposure

## Road safety and Traffic Safety

The Contractor shall ensure traffic safety by all project personnel during displacement to and from the workplace, and during the operation of project equipment on private or public roads.  The Contractor shall adopt best transport safety practices across all aspects of project operations with the goal of preventing traffic accidents and minimizing injuries suffered by project personnel and the public, including:

- Emphasize safety aspects among drivers.

- Improve driving skills and requiring licensing of drivers.

- Institute defensive driving training for all drivers prior to starting their job.

- Adopt limits for trip duration and arranging driver rosters to avoid overtiredness.

- Avoid dangerous routes and times of day to reduce the risk of accidents.

- Use speed control devices (governors) on trucks, and remote monitoring of driver actions.

- Require that drivers and co-passengers wear seatbelts, and duly sanction defaulters.

- Regularly maintain vehicles and use manufacturer approved parts to minimize potentially serious accidents caused by equipment malfunction or premature failure.

Where the project may contribute to significant changes in traffic along existing roads the contractor shall:

- Commence activities that affect public motorways and highways, only after all traffic safety measures necessitated by the activities are fully operational.

- Arrange diversions for providing alternative routes for transport and/or pedestrians.

- Minimize pedestrian interaction with construction vehicles, particularly at crossing points to schools, markets, and any animal crossing points of significance, through appropriate signage, engineered footpaths or traffic slowing devices.

- Organize meaningful road accident awareness events at all roadside schools and communities within 150 meters of the road centerline, covering safe road crossing, road accident hazards from weather conditions and vehicle roadworthiness, overloading and driver alertness, dangers posed by parked and broken-down vehicles, etc.

- Collaborate with local communities and responsible authorities to improve signage, visibility and overall safety of roads, particularly along stretches located near schools or other locations where children may be present.

- Collaborate with local communities on education about traffic and pedestrian safety (e.g. school education campaigns).

- Coordinate with emergency responders to ensure that appropriate first aid is provided to all affected persons in the event of accidents.

- Use locally sourced materials, whenever possible, to minimize transport distances, and locate associated facilities such as worker camps close to project sites.

- Employ safe traffic control measures, including road signs, traffic cones, removable barriers, and flag persons to warn of dangerous conditions.

## Emergencies

- Establish and maintain an emergency preparedness and response system, in collaboration with appropriate and relevant third parties including to cover: (i) the contingencies that could affect personnel and facilities of the project to be financed; (ii) the need to protect the health and safety of project workers; (iii) the need to protect the health and safety of the Affected Communities.  The emergency preparedness and response system shall include:

Identification of the emergency scenarios.

Specific emergency response procedures.

Training of emergency response teams.

Emergency contacts and communication systems/protocols (including communication with Affected Communities when necessary).

Procedures for interaction with government authorities (emergency, health, environmental authorities).

Permanently stationed emergency equipment and facilities (e.g., first aid stations, firefighting equipment, spill response equipment, personal protection equipment for the emergency response teams).

Protocols for the use of the emergency equipment and facilities.

Clear identification of evacuation routes and muster points.

Emergency drills and their periodicity based on assigned emergency levels or tiers.

Decontamination procedures and means to proceed with urgent remedial measures to contain, limit and reduce pollution within the physical boundaries of the project property and assets to the extent possible.

## Stakeholder Engagement

The Project Company will be required to undertake a process of stakeholder engagement with representative persons and communities directly affected by the activities it undertakes, including if necessary, the public disclosure of its C-ESMP.  The Project Company shall also maintain throughout the Project good relations with local communities and will give these communities prior notice of plans and schedules as they might affect local people.

The stakeholder engagement process will also be applicable in the event of land acquisition associated with changes in the footprint of activities.

### Labor Conditions

- Implement the measures and commitments defined in the Labor Management Procedures.  A copy of the LMP can be found in the Project ESMF

- Provide all workers with terms and conditions that comply with Yemeni Labor Legislation, most particularly Decree 5/1995) and applicable International Labor Organization conventions on workplace conditions.

### Insurance

- Provide insurance for call employees involved in onsite activities, as indicated by Yemen’s Labor Law

- Compensate any employee for death or injury, except to the extent that liability arises from the

## Grievance Mechanism for Workers

The Contractor shall put in place a Grievance Mechanism for workers and the workers of its subcontractors that is proportionate to its workforce.  The GM shall be distinct from the Project level Grievance Mechanism for affected individuals and communities, and shall adhere to the following principles:

- Provision of information.  All workers should be informed about the grievance mechanism at the time they are hired, and details about how it operates should be easily available, for example, included in worker documentation or on notice boards.

- Transparency of the process.  Workers must know to whom they can turn in the event of a grievance and the support and sources of advice that are available to them. All line and senior managers must be familiar with their organization's grievance procedure.

- Keeping it up to date. The process should be regularly reviewed and kept up to date, for example, by referencing any new statutory guidelines, changes in contracts or representation.

- Confidentiality.  The process should ensure that a complaint is dealt with confidentially. While procedures may specify that complaints should first be made to the workers’ line manager, there should also be the option of raising a grievance first with an alternative manager, for example, a human resource (personnel) manager.

- Non-retribution.  Procedures should guarantee that any worker raising a complaint will not be subject to any reprisal.

- Reasonable timescales.  Procedures should allow for time to investigate grievances fully but should aim for swift resolutions.  The longer a grievance is allowed to continue, the harder it can be for both sides to get back to normal afterwards. Time limits should be set for each stage of the process, for example, a maximum time between a grievance being raised and the setting up of a meeting to investigate it.

- Right of appeal.  A worker should have the right to appeal to the World Bank or national courts if he or she is not happy with the initial finding.

- Right to be accompanied.  In any meetings or hearings, the worker should have the right to be accompanied by a colleague, friend or union representative.

- Keeping records.  Written records should be kept at all stages.  The initial complaint should be in writing if possible, along with the response, notes of any meetings and the findings and the reasons for the findings.  Any records on SEA shall be registered separately and under the strictest confidentiality.

- Relationship with collective agreements.  Grievance procedures should be consistent with any collective agreements.

- Relationship with regulation.  Grievance processes should be compliant with the national employment code.

## Protection from Sexual Exploitation and Abuse

- Provide repeated training and awareness raising to the workforce about refraining from unacceptable conduct toward local community members, specifically women.

- Inform workers about national laws that make sexual harassment and gender-based violence a punishable offence which is prosecuted.

- Prohibit its employees from exchanging any money, goods, services, or other things of value, for sexual favors or activities, or from engaging any sexual activities that are exploitive or degrading to any person.

- Develop a system to capture gender-based violence, sexual exploitation and workplace sexual harassment related complaints/issues.

- Adopt a policy to cooperate with law enforcement agencies in investigating complaints about gender-based violence.

## Protection from Child Labor

- Verify that workers are older than 18 when hiring.

- Exclude all persons under the age of 18.

- Review and retain copies of verifiable documentation concerning the age of workers.

## Code of Conduct

Contractors shall ensure that all employees, including those of subcontractors, are informed about and sign the following Code of Conduct:

CODE OF CONDUCT FOR CONTRACTOR’S PERSONNEL

We the Contractor [enter name of Contractor] have signed a contract with UNOPS for [enter description of the activities].  These activities will be carried out at [enter the Site and other locations where the activities will be carried out].  Our contract requires us to implement measures to address environmental and social risks related to the activities, including the risks of sexual exploitation and assault and gender-based violence.

This Code of Conduct is part of our measures to deal with environmental and social risks related to the activities.  It applies to all our staff, including laborers and other employees at the at all the places where the activities are being carried out.  It also applies to the personnel of every subcontractor and any other personnel assisting us in the execution of the activities.  All such persons are referred to as “Contractor’s Personnel” and are subject to this Code of Conduct.

This Code of Conduct identifies the behavior that we require from all Contractor’s Personnel.

Our workplace is an environment where unsafe, offensive, abusive or violent behavior will not be tolerated and where all persons should feel comfortable raising issues or concerns without fear of retaliation.

Required Conduct

Contractor’s Personnel shall:

carry out his/her duties competently and diligently.

comply with this Code of Conduct and all applicable laws, regulations and other requirements, including requirements to protect the health, safety and well-being of other Contractor’s Personnel and any other person.

maintain a safe working environment including by:

ensuring that workplaces, machinery, equipment and processes under each person’s control are safe and without risk to health.

wearing required personal protective equipment.

using appropriate measures relating to chemical, physical and biological substances and agents; and

following applicable emergency operating procedures.

report work situations that he/she believes are not safe or healthy and remove himself/herself from a work situation which he/she reasonably believes presents an imminent and serious danger to his/her life or health.

treat other people with respect, and not discriminate against specific groups such as women, people with disabilities, migrant workers or children.

not engage in any form of sexual harassment including unwelcome sexual advances, requests for sexual favors, and other unwanted verbal or physical conduct of a sexual nature with other Contractor’s or Employer’s Personnel.

not engage in Sexual Exploitation, which means any actual or attempted abuse of position of vulnerability, differential power or trust, for sexual purposes, including, but not limited to, profiting monetarily, socially or politically from the sexual exploitation of another.  In Bank financed projects, sexual exploitation occurs when access to or benefit from Bank financed Goods, Works, Consulting or Non-consulting services is used to extract sexual gain.

not engage in Sexual Assault, which means sexual activity with another person who does not consent. It is a violation of bodily integrity and sexual autonomy and is broader than narrower conceptions of “rape”, especially because (a) it may be committed by other means than force or violence, and (b) it does not necessarily entail penetration.

not engage in any form of sexual activity with individuals under the age of 18, except in case of pre-existing marriage.

complete relevant training courses that will be provided related to the environmental and social aspects of the Contract, including on health and safety matters, and Sexual Exploitation and Assault (SEA).

report violations of this Code of Conduct; and

Not retaliate against any person who reports violations of this Code of Conduct, whether to us or the Employer, or who makes use of the [Project Grievance [Redress] Mechanism].

Raising Concerns

If any person observes behavior that he/she believes may represent a violation of this Code of Conduct, or that otherwise concerns him/her, he/she should raise the issue promptly.  This can be done in either of the following ways:

- Contacting the Individual designated by the Contractor [enter name of Contact)

- In writing at this address [   ]

- By telephone at [   ]

- In person at [   ]

- Calling [   ] to reach the Contractor’s hotline and leave a message (if available)

The person’s identity will be kept confidential, unless reporting of allegations is mandated by the country law.  Anonymous complaints or allegations may also be submitted and will be given all due and appropriate consideration.  We take seriously all reports of possible misconduct and will investigate and take appropriate action.  We will provide warm referrals to service providers that may help support the person who experienced the alleged incident, as appropriate.

There will be no retaliation against any person who raises a concern in good faith about any behavior prohibited by this Code of Conduct.  Such retaliation would be a violation of this Code of Conduct.

Consequences of Violating the Code of Conduct

Any violation of this Code of Conduct by Contractor’s Personnel may result in serious consequences, up to and including termination and possible referral to legal authorities.

For Contractor’s Personnel

I have received a copy of this Code of Conduct written in a language that I comprehend.  I understand that if I have any questions about this Code of Conduct, I can contact [enter name of Contractor’s contact person with relevant experience in handling gender-based violence] requesting an explanation.

Name of Contractor’s Personnel: [insert name]

Signature: ______________________________________________

Date: (day month year): ______________________________________________

Countersignature of authorized representative of the Contractor:

Signature: ______________________________________________

Date: (day month year): ______________________________________________

A copy of the code shall be displayed in a location easily accessible to the community and project affected people.  It shall be provided in languages comprehensible to the local community, Contractor’s personnel (including sub-contractors and day workers), Project Company’s and Project Manager’s Personnel, and affected persons.].

Annex 4 : Contractor Environmental and Social Reporting

Contractors shall monitor, keep records and report on the following environmental and social issues:

- Safety: hours worked, lost time injury (LTI), lost workdays, recordable incidents and corresponding Root Cause Analysis (lost time incidents, medical treatment cases), first aid cases, high potential near misses, and remedial and preventive activities required (for example, revised job safety analysis, new or different equipment, skills training, and so forth).

- Environmental incidents and near misses: environmental incidents and high potential near misses and how they have been addressed, what is outstanding, and lessons learned.

- Major works: those undertaken and completed, progress against project schedule, and key work fronts (work areas).

- ESHS requirements: noncompliance incidents with permits and national law (legal noncompliance), project commitments, or other ESHS requirements.

- ESHS inspections and audits: by Project Company, Independent Engineer, UNOPS and its implementing partners, or others—to include date, inspector or auditor name, sites visited and records reviewed, major findings, and actions taken.

- Workers: list of workers at each site, confirmation of ESHS training, indication of origin (expatriate, local, nonlocal nationals), gender, age with evidence that no child labor is involved, and skill level (unskilled, skilled, supervisory, professional, management).

- Training on ESHS issues: including dates, number of trainees, and topics.

- Footprint management: details of any work outside boundaries or major off-site impacts caused by ongoing construction—to include date, location, impacts, and actions taken.

- External stakeholder engagement: highlights, including formal and informal meetings, and information disclosure and dissemination—to include a breakdown of women and men consulted and themes coming from various stakeholder groups, including vulnerable groups (e.g., disabled, elderly, children, etc.).

- Details of any security risks: details of risks the Project Company may be exposed to while performing its work—the threats may come from third parties external to the project.

- Worker grievances: details including occurrence date, grievance, and date submitted; actions taken and dates; resolution (if any) and date; and follow-up yet to be taken grievances listed should include those received since the preceding report and those that were unresolved at the time of that report.

- External stakeholder grievances: grievance and date submitted, action(s) taken and date(s), resolution (if any) and date, and follow-up yet to be taken grievances listed should include those received since the preceding report and those that were unresolved at the time of that report. Grievance data should be gender disaggregated.

- Major changes to Contractors environmental and social practices.

- Deficiency and performance management: actions taken in response to previous notices of deficiency or observations regarding ESHS performance and/or plans for actions to be taken should continue to be reported to UNOPS until it determines the issue is resolved satisfactorily.

# Annex 5: Consultation Questionnaires

figure 4      : Project Maps

Figure  5   – main pipes lines from Tawahi to Manhole

Figure 6 – Configuration of force main from Ma’alla PS to Khormaksar

Figure 7 – Configuration of force main from Crater PS to

Khormaksar District

Figure 8 – Configuration of force main from Khormaksar PS to Al Areesh STP

The ESHS requirements build on the General EHS Guidelines of the World Bank Group, but also take into account other World Bank guidelines, and good practice notes

Based on the World Bank COVID-19 LMP Template, April 16, 2020