Hazard and Risk Assessment

| Location/Project/Office | Aden Wastewater Network Rehabilitation Project (PK-0) |  |  |
| --- | --- | --- | --- |
| Details of what is being assessed (activity, functional area) | - Excavation /levelling/ filling/pipe sand bedding; - Compaction and Backfilling; - Cut, remove and re-asphalt in some cases, if needed; - Cut, remove and re-pave with stones; if needed; - Manhole work/rehabilitation/backdrop; - Painting of manhole covers with epoxy; - Taking care and rehabilitation of other underground services connections if needed; - Connecting the new trunk sewer line to the existing sewer pump station |  |  |
| Document prepared by (name & signature) | Fahmi Alaswar | Reviewed and approved by (name & signature) | Abdulwahab Salah Fayyaz Ahmad Faiz-Rasul |
| Date | 27th June 2022 | Date |  |

| Revision | Date | Author | Description of main changes |
| --- | --- | --- | --- |
| 0 | 27/06/2022 | Fahmi Alaswar | Document Creation |

| Consequence | Likelihood |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Unlikely | Slightly likely | Likely | Very likely |
|  | Negligible | 1 | 2 | 3 | 4 |
|  | Minor | 2 | 4 | 6 | 8 |
|  | Moderate | 3 | 6 | 9 | 12 |
|  | Major | 4 | 8 | 12 | 16 |

Follow the guidance given in the Health and Safety handbook under the title “Hazards and their control” in order get a full understanding of the hazard identification and risk assessment process.

Step-by-Step Guidance:

Stage One: Using the matrix above to carry out an initial assessment to determine the risk rating of each hazard(s) of the activity. To calculate the risk rating of a hazard, multiply the value of its consequence with the value for likelihood (note: here the hazard should be assessed without any control measures). Make sure relevant stakeholders (e.g. local UNDSS focal points) are consulted and involved in the risk assessment.

Stage Two: Use the results from the assessment in Stage One to prioritize control activities. As a general rule, if the risk is High, then the hazard must be eliminated and/or work activity prohibited or mitigation measures put in place to reduce the risk. If the risk is Medium, then additional safety controls are required to eliminate/isolate/minimise the risk. If the risk is Low, then the work can proceed with the current standard site controls in place.

The hierarchy of controls (elimination, substitution, engineering controls, administrative controls, personal protective equipment) should be applied when choosing controls, with more effective ones to be applied first. See the H&S handbook for more guidance.

Stage Three: Using the matrix to review again the hazards, this time with the safety controls proposed. If the risk rating remains Medium or High, then provide alternative or additional safety controls until the risk is assessed as Low.

Stage Four: Review the risk assessment periodically in order to take account of changes in the environment. In each revision, make sure new and modified content is easily identifiable (by text highlighting, use of a different font, use of a different font colour, and similar techniques).

When identifying hazards, consider among other things the following:

Routine and non-routine activities

Any planned changes

All people involved

Human factors

Design of work processes or work areas that may generate a hazard

Legal obligations

The frequency of the task and the number of people that could be potentially affected should be considered.

For the identification of hazards to be effective, a variety of sources should be investigated, as for example:

Observations

Benchmarking

Safety tours, inspections

Incident reviews

Monitoring

Workflow and process analysis

Note that more detailed assessments might be necessary for the risk assessment to evaluate the harm from exposure to chemical, biological and physical agents.

Identification and ranking of natural hazards may be done by using the country specific information found on http://www.thinkhazard.org/.

Note on Safety and Security: Threats directly resulting from or related to terrorism, civil unrest, armed conflicts and crime falls under the responsibility of the UN Security Management System. An assessment of these should not be included in this document.

Fire, aviation safety and road transport safety are also the responsibility of the UN Security Management System; those should not be included in the risk assessment for UNOPS office facilities.

| Item | Task/Activity | Potential Hazards/Risks for each task | Risk Rating (1-16) | Hazard Control Method | Control Risk Rating (1-16) | Person to implement and monitor implementation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Access /Egress | - Unauthorized entry to site - Injury to public - Vehicle collision | 12 | - Banksman in place for any awkward reversing - Mandatory PPE worn - HSSE induction complete - Driving with due care & attention - Adherence to site safety signage - Never block access and egress routes with materials and equipment - Site tidiness maintained by cleaning up during work, at end of day and(major) weekly to avoid slips and trips – Clean as you go. - No protruding nails allowed: all such nails must be clawed out or hammered flat so as to foot injuries - Routes delineated with barriers; barriers secured with sandbags where necessary - Excavations backfilled as soon as possible - Barriers erected around unattended excavations - Open excavations attended. - Access to the site must be controlled to ensure the safety of site employees and visitors. - Sign in/ sign out to take place - Ensure site is secured against unauthorized operation | 2 | Contractor and UNOPS supervision |
| 2 | Loading & Unloading | - Falls from height - Crushing | 16 | - Safe lift plan in place for lifts by mobile crane. - Operator trained to the approved CPCS (Construction Plant Competence Scheme) or UNOPS standards - Vehicles should never be overloaded. Loads should be evenly distributed, secured and not protruding beyond the sides or back of the vehicle. - Reverse beeper and flashing beacon fitted. - All guards and covers to be in position and secured. - Windows, lights and mirrors to be kept clean at all times. - The cab floor to be kept clean and free from any obstacles which may affect the safe operation of the Machine. - Ground workers to wear hi-visibility vests at all times and hard-Hats. - Warning signs will be erected to alert pedestrians of unloading/loading operations - Ground workers to keep clear of plant at all times and not to approach until signalled to do so by the trained Operator while loading/unloading operations are underway. - All lifting equipment must be certified for its use and have proof of such inspections available to all supervisory staff. No work shall be - carried out on site unless certification has been checked by Site Supervisor/Engineer. - Loads to be tipped only on suitably level ground to prevent overturning. - All sites and equipment will be equipped with a Fire extinguisher. - The use of mobile phones will be prohibited during lifting operations. - When mounting or dismounting the machine the handholds and steps must be used and must be maintained in safe condition. - Access roads will be damped down during fine weather periods to prevent dust from rising. - Where the operators direct field of vision is obstructed a banksman should be available to assist the operator, this person should be adequately trained. | 2 | Contractor and UNOPS supervision |
| 3 | Excavation of Trenches | - Underground services - Telephone, - electricity or water - Falling materials or plant - Falls of persons - Ingress of water - Collapse of sides - Weakening of adjacent structures - Noise | 12 | - Ground conditions established by a survey to identify the type of - ground in which the excavation is to be carried out; means of trench support confirmed upon completion of trial holes - Remember that even work in shallow trenches can be dangerous. - You may need to provide support if the work involves bending or kneeling in the trench. - No one should be in the trench unless it is safe to be there. - Where possible ensure that all water sources are turned off before entering an excavation - Do not go into unsupported excavations that have not being battered to a suitable slope. Never work ahead of the support. - Do not store spoil or other materials close to the sides of excavations. The spoil may fall into the excavation and the extra loading will make the sides more prone to collapse - Contaminants removed following a suitable risk assessment and method statement - Employees and Management must ensure no-one enters an unsupported unsafe trench - Support materials on site before excavation starts. - Plant and materials kept away from the side of excavations to prevent undue pressure or ingress of exhaust fumes. - if the depth is a particular risk of anybody falling, suitable guard-rails placed and suitable access arrangements, such as ladders or ramps, provided. - If there is a risk of water ingress, suitable methods and/or equipment provided to either prevent the entry of water or to remove water, e.g. water pumps - Baulks or earth bunds should be used if a plant/Vehicles/equipment could fall into the excavation. - Where excavations are dug next to structures, suitable preventative measures established in case of collapse, subsidence or damage - Inspections of excavations carried out prior to each shift, after any event likely to affect strength or stability be provided. - Record of inspections recorded on approved UNOPS form - Suitable personal protective equipment [PPE] - The location of electric cables, drains, Telephone and water mains etc. will be determined and marked prior to the commencement work. Local authorities staff to be consulted on underground services and, where necessary, scanning is to be used to determine services locations. - It will be the liability of the contractor to make good any damage done to such services during excavation work, to render them safe and secure without undue delay. - Excavations or openings will be properly sloped or shored at all times. - A competent person must approve trench designs prior to entry. Warning lamps must be used during hours of low visibility. - Excavations or openings in floors must be properly fenced at all times. - During excavation work the surrounding areas must be maintained in an orderly and tidy condition. - Where vehicles have to tip materials into excavations, stop blocks can be used to prevent them from over-running. Remember that the sides of the excavation may need extra support. - Always work outside the reach of an excavator bucket. Where this is not possible, use safe systems of work to prevent people being struck. - Do not stand directly under the bucket. - Safe means of access will be provided to excavations. Ladders will be in good condition, extend 1 meter above the excavation edge and be secured. - Heavy vehicles are to be kept away from the edge of an excavation. - At no time should materials be stored along the edge of an excavation. - Flooded trenches should be "de-watered" as quickly as possible. Once emptied the trench is to be fully inspected prior to entry. - All personnel on sites, where work at heights is being carried out, will wear safety helmets. - Appropriate safety equipment will be used when necessary | 2 | Contractor and UNOPS supervision |
| 4 | Laying of Pipes, | - Entrapment due to fall/ roll - of pipe. - Operators direct field of vision obscured by load leading to contact with other persons or plant. - Failure of lifting equipment. - Inadequate supervision. - Untrained Operatives - Use of mobile phones by operators. - Displacement of load - Person being struck by suspended pipe causing serious injury of harm. - Defective and broken skids causing crushing of limbs of - entrapment of body parts. - Manual Handling. | 12 | - Operator trained to the Approved CPCS or UNOPS standard. - Vehicles should never be overloaded. Loads should be evenly distributed, secured and not protruding beyond the sides or back of the vehicle. - Reverse beeper, flashing beacon, and or mirrors fitted - Defective skids removed immediately from the line. - Heavy duty gloves to be worn by operatives handling skids. - All engines guards and covers to be in position and secured. - Windows, lights and mirrors to be kept clean at all times - The cab floor to be kept clean and free from any obstacles which may affect the safe operation of the Machine. - Ground workers to wear Hi-Visibility Vests at all times and Hard-Hats. - Warning signs will be erected to alert pedestrians of loading/unloading operations. - Traffic management must be in place where loading/unloading is being carried out on public roads. - Loads to be tipped only on suitably level ground to prevent overturning of skip. | 2 | Contractor and UNOPS supervision |
| 5 | Work at Height | - Fall of a person a distance liable of causing personal - Injury - Falls of persons off the edge of structures | 16 | - Take into accounts the principles of prevention when working at heights. - Work at height must be adequately planned and supervised - Collective fall protection measures must be given priority. - Trench boxes must have guard and mid rails. - Inspections on equipment carried out (i.e., crowd control barriers, harnesses, fall arrest devices etc.) - Safe means of access to the work location provided using ladders | 2 | Contractor and UNOPS supervision |
| 6 | Manual Handling | - Ligament strains. Muscle strains. - Fatigue. - Abdominal hernias. - Prolapsed/herniated discs. - Paralysis. | 6 | - Full training and instruction provided in both induction and on- going safety training. - Design out the need for manual handling where possible - Mechanical aids used wherever possible - Workers adhere to guideline weights & to only lift what they are able to lift - Assess the load - Weights marked on all loads. - Lift smaller loads. - Cutting pipes to length - Mandatory PPE worn | 1 | Contractor and UNOPS supervision |
| 7 | Unauthorized Access | - Personal injury | 12 | - Unauthorized entry to the working area to be reported to site management. - Barriers to be installed to secure access | 2 | Contractor and UNOPS supervision |
| 8 | Vehicle Fuels & chemicals & Dust | - Fire - Explosion - Unintended release / spills - Environmental Pollution | 16 | - COSSH (Control of substance hazardous to health) assessments complete and communicated to workers - Appropriate PPE worn - Ensure employees have a spill kit available and know how to use it. - Fuels stored in safe locations, away from sunlight and in a secure location - Water suppression used - Dust masks worn - Face fit testing complete | 2 | Contractor and UNOPS supervision |
| 9 | Slips, Trips & Falls | - Cuts and bruises - Laceration - Serious bodily injury | 12 | - Workplace tidiness maintained by cleaning up during work, at end of day to avoid slips and trips – Clean as you go. - Vigilance from employees as to the dangers from slips trips and falls. | 2 | Contractor and UNOPS supervision |
| 10 | Working on or near roads | - Collision with plant or traffic - Traffic Accidents. - Inadequate Traffic Management Arrangements - Misleading signs - Close proximity of the public. - Collision with plant or traffic - Traffic Accidents. - Inadequate Traffic Management Arrangements - Misleading signs - Close proximity of the public | 16 | - Traffic management plan (TMP) in accordance with UNOPS Standard - Correct signing/coning/barriers in place for all roadwork’s and path works; including site entrance signage - Competent contractors implementing the TMP - Employees are trained to maintain safety at roadwork’s - Longitudinal & size safety zones in place - Tapering in accordance with UNOPS Standard - Signs in place in accordance with UNOPS Standard & made stable by sandbags - Consultation to occur with the local authority; appropriate permits obtained & appropriate controls implemented - Safe access and egress for vehicles, plant and persons - Safe crossing of traffic lanes. - Traffic management equipment maintained clean - Traffic control supervisor appointed who inspects traffic management arrangements twice daily - All delivery drivers to adhere to the rules of the road. - Hard hats; Hi-visibility clothing & steel toe capped boots must be worn at all times - Safe access for vehicles, plant and persons - Safe crossing of traffic lanes by site workers - Employees set out signs before moving onto work site. - Trained employees, measure out the distance for the roadwork’s ahead signs/flashing warning signs as per traffic management plan, place signs on left hand side. - Trained employees, work back towards the site, placing signs as necessary. - All employees always stay on the verge if possible - Where there is two-way traffic, repeat the procedure for traffic going in the opposite direction. - If portable traffic lights are to be used, Trained employees start using them before going in the opposite direction with signage & cones - Employees cone off the works area, always face the traffic when setting out cones for the lead in taper and always start from the verge. - Trained employees set up End of Roadwork’s sign to show that the road is clear in both directions. - When removing the signs, trained employees always reverse the procedure - Site or works entrance signs installed 100 m before entrance. - Continual monitoring of work in progress and traffic control | 2 | Contractor and UNOPS supervision |
| 11 | Backfilling Machinery and equipment | - Use of machinery may lead to accidents | 16 | - Ensure machinery and equipment are in good condition before use - Clear the work area and ensure only assigned workers are in place - Ensure a banksman is on location to give direction and only one person is to give direction - Ensure the use of warning light when working | 2 | Contractor and UNOPS supervision |
| 12 | Cutting, removing , and re-asphalt | - Use of Jack hammer and Compressors that lead to increase of vibration | 12 | - Only use good condition equipment - Well trained workers - Ensure proper PPE is used to prevent accidents - Ensure proper rest is given to workers between work on vibrating equipment - Ensure removed asphalt is collected and disposed of properly | 2 | Contractor and UNOPS supervision |
| 13 | Lone Working | - May lead to accidents and fatalities | 16 | - Ensure no worker is working alone a minimum no of two labours is a must. - Proper supervision on location to ensure no worker is left alone. - Enforcement of permit to work system to ensure all safety precaution are in place to ensure workers safety - List of workers to be available with supervisor - Raise awareness on the dangers of lone working. | 2 | Contractor and UNOPS supervision |
| 14 | Waste generation | - Environmental damage. - Health Illness | 12 | - Continuously remove the waste and transfer it to the approved disposal site by the local district authorities - work section by section and not opening new section until finishing and Cleaning the way from the first one - moving solid or liquid construction materials and waste shall take strict measures to minimize littering of roads - Vehicles are loaded in such a manner as to prevent falling off or spilling of construction materials - Sheeting the sides and tops of all vehicles carrying mud, sand, other materials and debris - Debris should be transferred to assigned places in the landfill with documented confirmation. | 2 | Contractor and UNOPS supervision |

Space for comments

Appendix: The following is a list of possible hazards.

| Physical hazards | Chemical hazards | Biological hazards | Psychosocial hazards | General environment hazards |
| --- | --- | --- | --- | --- |
| - slippery or uneven ground, - working at height, - objects falling from height, - inadequate space to work, - poor ergonomics (e.g. workplace design that does not take account of human factors), - manual handling, - repetitive work, - trappings, entanglement, burns and other hazards arising from equipment, - transport hazards, either on the road or on premises/sites, while travelling or as a pedestrian (linked to the speed and external features of vehicles and the road environment), - fire and explosion (linked to the amount and nature of flammable material), - harmful energy sources such as electricity, radiation, noise or vibration (linked to the amount of energy involved), - stored energy, which can be released quickly and cause physical harm to the body (linked to the amount of energy), - frequently repeated tasks, which can lead to upper limb disorders (linked to the duration of the tasks), - unsuitable thermal environment, which can lead to hypothermia or heat stress, - violence to staff, leading to physical harm (linked to the nature of the perpetrators), - ionizing radiation (from x- or gamma-ray machines or radioactive substances), - non-ionizing radiation (e.g. light, magnetic, radio-waves) | Substances hazardous to health or safety due to: - inhalation of vapours, gases, or particles, - contact with, or being absorbed through, the body, - ingestion, - the storage, incompatibility, or degradation of materials. | Biological agents, allergens, or pathogens (such as bacteria or viruses), that might be: - inhaled, - transmitted via contact, including by bodily fluids (e.g. needle‑stick injuries), insect bites, etc. - ingested (e.g. via contaminated food products) | Situations that can lead to negative psychosocial (including psychological) conditions, such as stress (including post-traumatic stress), anxiety, fatigue, depression, from e.g.: - excessive workload, - lack of communication or management control, - workplace physical environment, - physical violence, - bullying or intimidation. | - Environmental risks - Endemic diseases - Disease Outbreaks - Pandemics - Quality and availability of health care - Natural Disasters - Nuclear Disaster - Security conditions, Crime, Social Unrest, Political violence fall under the responsibility of the UN Security Management System |

Source: OHSAS 18002:2008

In order to manage and control change, this template should be reviewed each time a change happens in the organization (e.g. in the structure, personnel, management system). The results of the risk assessment should be communicated with the staff as appropriate.