SECTION: 9

IRONMONGERY

1	Description

The Contractor shall provide and fix the ironmongery required by the Particular Specifications or shown on the Drawings complete, including all necessary screws, bolts, plugs and other fixings. The use of nails for fixing ironmongery shall not be permitted. The Contractor shall hand over all in a finished state and to the satisfaction of the Engineer.

All ironmongery shall be of first quality from an approved brand such as Union, Tesa or equivalent. Butt hinges are to be aluminium alloy with silver anodised finish with double stainless steel washers, or as approved by the Engineer.

The Contractor shall be required to submit for approval samples of all items of ironmongery he proposes to use.

All doors shall be provided with an approved door stop plugged and screwed to the floor and all opening areas of aluminium work shall be provided with appropriate friction stays. The size, materials, finishes, type and quality of ironmongery shall be as described in the Particular Specification or as shown on the Drawings.

2	Finish

The finish of the various items of ironmongery shall be as described in the Particular Specification or as shown on the Drawings or as required and directed by the Engineer.

3	Fitting and Testing

All screws used for fixing ironmongery shall be of a suitable type, material, finish, size and shape to the satisfaction of the Engineer.

The hinges on which doors, windows, flyscreen doors etc., are hung shall be carefully housed or let into the door, window, flyscreen door etc., and to the frames.

All fittings shall be removed before commencing any painting operations and shall be refixed in place after all painting works are completed and approved by the Engineer.

All ironmongery shall be carefully wrapped and protected until completion of the work and any items or parts which are damaged or defaced or found to be defective shall be replaced at the Contractor's expense before handing over.

On completion of all locks , catches and similar items of ironmongery they shall be clearly labelled , with metal tags approximately 50x20mm and securely fixed to the keys and handed to the Engineer .

Door closers shall be fitted a maximum of two weeks before handover.

All floor and door springs are to be fully charged with oil and their operation checked to the satisfaction of the Engineer.

4	Standard Ironmongery for Internal Doors

Ironmongery is to be hard satin anodised aluminium alloy with matching screws.

All locks are to be provided with 2 keys on a key ring neatly labeled to indicate clearly the corresponding lock.

All knob sets shall include the appropriate mortice latch or lock with a 70 mm backset and with standard faceplates and roses unless otherwise noted.