SECTION: 2

EXCAVATION AND EARTHWORK

1	Datum and Nature of Excavation

The Contractor shall be responsible for setting up and maintaining a site datum level. 'Zero' datum shall be given on the site by the Engineer, unless otherwise noted on the Drawings.

Information pertaining to the nature of the ground may be given to the contractor, when available, but without any grantee of correctness or accuracy.

2	Setting Out

Each Benchmark (B.M.) or centerline for the building and/or other constructions to be under this contract, shall be properly set out by the Contractor as shown on the Drawings and inspected and approved by the Engineer prior to commencing excavations.

3	Surface Leveling

All earth works on Site shall be completed before any fill is deposited. Excavations over areas of Site or filling with approved material shall be carried out where required to the levels shown on the Drawings or to such other levels as may be directed by the Engineer.

4	Size and Depth of Excavation

Excavation shall be cut to the size of the foundation shown on the Drawings and taken down to the foundation levels shown on the Drawings. If, without the Engineer's written instructions, the Contractor goes down below the foundation level specified, he shall fill up the part so excavated with concrete of the same type and grade as that required for the piles as defined in Concrete Works and the Bill of Quantities at his own expense.

5	Shoring Excavations and Dewatering

The sides of excavations shall be supported as necessary to maintain a vertical face and/or to prevent caving-in of any nature, especially during subsequent operations. The Contractor shall be responsible for the design, supply, fixing, safety and removal of all planking, strutting and shoring required to the side of the excavation. The Contractor is responsible on the type and nature of soil to be excavated; also he is to use any equipment, including dewatering equipment, to carry out the work required by this Contract.

6	Types of Excavations

Excavations shall be classified according to the nature of the materials to be removed either as excavation in rock or as excavation in earth. Excavation in rock shall be classified as including all excavations in hard and consolidated materials which cannot be removed by normal excavation tools and equipment, and which require drilling or other special means for their removal. All excavation which is not in rock shall be defined as excavation in earth.

7	Required Bearing Capacity

The Contractor shall notify the Engineer and obtain instructions if the required bearing capacity:

(1)  is obtained at a lesser depth than that shown on the drawings.

(2)  is not obtained at the depth shown on the drawings.

Excavation bottoms are to be approved before new work is laid on them. The Contractor is to inform the Engineer when excavations are ready for inspection. If, after approval, surfaces become unsuitable due to flooding or other causes, the Contractor shall excavate further, backfill with approved filling material and compact to approval at no extra cost to the owner.

Should the soil condition, after the Engineer has been informed the excavations are ready for inspection, be such that the Engineer deems it necessary to carry out tests, the Contractor shall perform such tests to the satisfaction of the Engineer. Any expenses incurred in the performance of such tests is deemed to be included by the Contractor in his tender.

8	Preparation And Inspection Of Excavation

All excavations shall be kept free of water arising from whatever source, and shall be properly cleaned out of all loose and foreign matter, leveled and rammed. The Contractor is to report to the Engineer when excavations are ready for inspection and all excavations are to be approved before any further work is done.

9	Fill Material

All fill materials shall be approved by the Engineer before being placed in position. Approved earth, sand, or any other suitable material free from rubbish shall be used to make up levels as shown on the Drawings. The material shall be placed in successive layers each having a finished thickness not exceeding 250mm before compaction, watered and compacted to at least 98% compaction ratio of the maximum dry density of the Modified Proctor test (ASTM D-1557) prior to the placement of the succeeding layer.

Any exposed side or edge of fill shall be properly and evenly graded to a slope as directed or specified. When the required quantities of approved fill are not found on Site the Contractor shall, at his own expense, obtain them from locations approved by the Engineer.

Filling to make up levels under building shall be executed with approved suitable material from existing ground levels up to underside of ground floor slab, and shall be placed in successive layers each having a finished thickness not exceeding 250mm, watered and compacted to at least 98% compaction ratio of the maximum dry density of the Modified Proctor test (ASTM D-1557) prior to the placement of the succeeding layer.

The ground surrounding the building and pavements shall be filled with approved material, fine sand and coarse materials, free of foreign material, debris, clay lumps, organic and vegetation.

a)	Course materials:

Such as sandy gravel, gravelly sand, …etc., which is the material retained on sieve no. 4, and consists of crushed rock. It shall be clean, hard, tough and free from deleterious substance.

b)   Fine Sand Materials

Shall consist of that portion of the total aggregate/fines passes no.4 sieve, such as sand, silty sand,…etc.

The fill materials shall consists of the combination of coarse and fine sand and conform to the following grading:

Sieve Size 	1/ 2 "	3/8"	# 4	# 10	# 30	# 60	# 200

%	90-100	80-90	58-72	42-50	28-38	8-18	2-5

70-100	60-85	50-75	30-60	20-40	10-25	0-10

Pass	100	90-100	65-85	35-45	20-30	15-30	3-8

The fine sand shall conform to the following grading:-

Sieve Size 	# 4	# 30	# 60	# 200

%	100	80-100	30-50	4-8

90-100	80-90	20-30	0-10

Pass	100	90-100	40-90	5-15

In addition, shall conform to the following physical requirements:

- Minimum CBR		35 %   Coarse materials, Kurkar

25 %   Fine sand, Sand

10	Disposal of Surplus Material

All surplus excavated material not used in backfilling or leveling shall be loaded and transported elsewhere on the Site as required by the Engineer or, if not required on the Site, shall be loaded and carted away from the Site to a dump to be selected by the Contractor and approved by the Municipality at the Contractor's own expense.

All rubbish arising from the Works shall be cleared away and removed from the Site as it accumulates according to the Engineer's instructions and also on completion of the Works.