# Doors and door hardware

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to conventional door frames of metal and timber with door leaves of timber and various timber and plastic products. Because of the range of products available, performance requirements and design choices, this worksection only covers some key generic requirements.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Windows for metal framed glazed doors forming part of window assemblies.

- Overhead doors for sectional and tilting overhead doors, roller shutters (including fire shutters), grilles and garage doors.

NATSPEC worksection(s) on which this worksection relies

The following worksections must be included, if this worksection is used, to provide a complete specification:

- General requirements.

- Windows for door frames which form part of a window/door assembly.

- Door hardware.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

Worksection cross-referencing

Other worksections that cross reference this worksection are:

- Door hardware.

Documenting this and related work

You may document this and related work as follows:

- Doors and hatches need comprehensive detailing and scheduling beyond the scope of this worksection.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

### Interpretation

#### Definitions

For the purposes of this worksection the definitions given below apply.

- Door frame: Includes door trims.

- Doorset: An assembly comprising a door or doors and supporting frame, guides and tracks including the hardware and accessories necessary for operation.

- Fire-doorset: A doorset which retains its strength and limits the spread of fire.

- Smoke-doorset: A doorset which restricts the movement of smoke.

- Flush door: A door leaf having two flat faces which entirely cover and conceal its structure. It includes doors with cellular and particleboard cores.

- Joinery door: A door leaf having stiles and rails, framed together. A joinery door may also incorporate glazed panels.

- Louvred door: A joinery door in which the panel spaces are filled in with louvre blades.

The Definitions subclause can be edited to suit the project or deleted if not required.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following:

- Door frames standing in place before building in to brickwork.

- Door frames installed before fixing trim.

### Submissions

#### Samples

Submit samples of all hardware items for approval by the Employers representative before use in the works.

#### Subcontractors

Automatic sliding door assemblies: Submit names and contact details of proposed supplier and installer.

#### Product warranties

Consult or negotiate with manufacturers for warranty terms, and specify only such terms as are actually available.

Automatic sliding door assemblies: Submit a warranty from the supplier and installer for the system and its installation, for a period of at least twelve months from the date of completion.

Hardware: Submit the warranties offered by the manufacturer for the hardware items provided in the works.

#### Keys

Key codes: Submit the lock manufacturer’s record of the key coding system showing each lock type, number and type of key supplied, key number for re-ordering, and name of supplier.

Keys: For locks keyed to differ and locks keyed alike, verify quantities against key records, and deliver all keys and records to the Employers representative at completion.

Hold points, if required, should be inserted here.

## Products

### Frames

#### Aluminium frames

To be assembled from aluminium sections, including necessary accessories such as buffers, strike plates, fixing ties or brackets, and suitable for fixing specified hardware.

#### Timber frames

- To be constructed with best quality timber. Obtain approval from the Employers representative for the timber selection before use. Construct as shown on the drawings and ensure that all joints are securely made to avoid distortion of the frame in use.

#### Steel frames

To be folded from metallic-coated steel sheet sections, joints to be continuously welded, including necessary accessories such as buffers, strike plates, spreaders, fixing ties or brackets, and suitable for fixing specified hardware.

Finish: Grind the welds smooth, prepare and paint the welded joints with primer. Then prime the entire frame.

Hardware and accessories: Provide for fixing hardware including hinges and closers, using 4 mm backplates inside the frame. Screw fix the hinges into the back plates.

Base metal thickness:

- General: ≥ 1.1 mm.

- Fire rated doorsets: ≥ 1.4 mm.

- Security doorsets: ≥ 1.6 mm.

### Doors

#### Flush doors

Cellular core flush doors:

- Provide a subframe of 25 mm minimum width timber around openings for louvres and glazing.

- Provide additional material to take hardware and fastenings.

- Cut outs: If openings are required in flush doors (e.g. for louvres or glazing) make the cut outs not closer than 120 mm to the edges of the doors.

Solid core flush doors:

- Core of timber strips laid edge to edge, fully glued to each other and to facings each side of no less than two sheets of timber veneer.

- Single thickness of moisture resistant general purpose particleboard.

- Refer to drawings and Flush doors schedule for details.

#### Joinery doors

Fabricate joinery doors as shown on the drawings and in the Joinery doors schedule.

#### PVC doors

Fabricate PVC doors as shown on the drawings and in the PVC doors schedule.

#### Construction

Omit thicknesses if noted in a door schedule.

Form rebates to suit standard rebated door hardware.

Louvre grilles: Construct by inserting the louvre blades into a louvre frame, and fix the frame into the door.

Metal louvre door “leaves” are specified in the Windows worksection, which covers a wide range of louvre types including those designed for inclusion in door frames. Transfer the relevant text to this worksection if required. Louvre grilles fitted to cutouts in door leaves are often defined in the mechanical specification and enumerated on door schedules.

#### Double doors

Provide rebated meeting stiles unless the doors open in both directions. Chamfer square edged doors to prevent binding between the leaves.

### Doorsets

#### Automatic sliding door assemblies

Provide auto sliding door assemblies in accordance with the Automatic door schedule.

#### Toughened glass door assemblies

Provide toughened glass door assemblies with matching concealed hinges and patch fittings as appropriate. Ensure that all glass edges are protected during installation and polish on completion.

Types other than metal are available. If fire resistance is required, specify as a fire-resistant doorset.

#### Fire-resistant doorsets

Provide fire resistant doors and frames as matched sets for door openings required to have a fire rating. Refer to the Fire and smoke resistant doorsets schedule for details.

Provide copies of test certificates from recognised authorities proving the performance of the doorsets.

#### Smoke-resistant doorsets

Provide smoke resistant doors and frames as matched sets for door openings required to have a smoke stopping capability. Refer to the Fire and smoke resistant doorsets schedule for details.

Provide copies of test certificates from recognised authorities proving the performance of the doorsets or seals to frames.

AS/NZS 1905.1 is cited in the BCA  Spec  C3.4. See also its requirements for radiation through glass.

The member sizes and thickness are typical only. For large hatches the members may need to be heavier, and should be shown on the drawings.

#### Security screen doorsets

Provide security screen doorsets in accordance with the Security screen doors schedule.

### Ancillary materials

#### Nylon brush seals

To be dense nylon bristles locked into galvanized steel strips and fixed in a groove in the edge of the door or in purpose-made anodised aluminium holders fixed to the door

#### Pile weather strips

To be polypropylene or equivalent pile and backing, low friction silicone treated, ultra-violet stabilised.

#### Door Seals

To be proprietary items as identified in Schedules and to approval of Employers representative.

### Hinges

#### Butt hinge sizes

Refer to Hinge table A and Hinge table B in which length (l) is the dimension along the knuckles, and width (w) is the dimension across both hinge leaves when opened flat.

- Steel, stainless steel, brass, bronze butt hinges for timber doors in timber or steel frames: To Hinge table A.

- Aluminium hinges for aluminium doors, or for doors of other materials in aluminium frames: To Hinge table B.

#### Hinge materials

Aluminium hinges: High tensile aluminium with fixed stainless steel pins in nylon bushes, and with nylon washers to each knuckle joint.

Doors fitted with closers: Provide low friction bearing hinges.

#### Hinge pins

Exterior or security doors opening out: Provide fixed pin hinges.

#### Hinge Table A

| Nominal hinge size l x w x t (mm) | Door leaves not exceeding any of the following |  |  |
| --- | --- | --- | --- |
|  | Mass (kg) | Width (mm) | Thickness (mm) |
| 70 x 50 x 1.6 | 16 | 620 | 30 |
| 85 x 60 x 1.6 | 20 | 820 | 35 |
| 100 x 75 x 1.6 | 30 | 920 | 40 |
| 100 x 75 x 2.5 | 50 | 920 | 50 |
| 100 x 75 x 3.2 | 70 | 1020 | 50 |
| 125 x 100 x 3.2 | 80 | 1220 | 50 |

Door mass is the criterion for hinge size. 16 kg covers light screen doors; 35 kg covers standard size (2040 x 820 mm) hollow or cell core doors; 40 kg covers standard size solid core doors, and standard size aluminium frame glazed doors; 68 kg covers 2700 x 900 mm solid core doors up to 40 mm thick; and 100 kg covers solid core doors up to 2700 x 1200 x 45 mm. Power transfer hinges for use with electric locks are non load bearing, and should only be used in addition to compatible hinges.

#### Hinge Table B

| Nominal hinge size l x w x t (mm) | Door leaf not exceeding mass (kg) | Minimum construction |  |
| --- | --- | --- | --- |
|  |  | Knuckles | Screws/hinge leaf |
| 100 x 70 x 3 | 30 | 3 | 3 |
| 100 x 80 x 3.5 | 50 | 5 | 4 |

#### Number of hinges

Follows the principle that height and width are criteria as well as mass.

Provide 3  hinges for doors up to 2200 mm high, and 4 for door leaves between 2200 mm and 3000 mm high.

#### Wide throw

If necessary, provide wide throw hinges to stop doors binding on obstacles such as nibs or deep reveals.

### Door hanging systems

#### General

Provide sliding door tracks in conformance with the schedules.

### Locks and latches

#### General Door Hardware

Provide hardware of sufficient strength and quality to perform its function, appropriate to the intended conditions of use and climate and fabricated with fixed parts firmly joined.

#### Bolts

Provide bolts including barrel bolts and tower bolts with associated hardware, including lock plates, ferrules or floor sockets.

#### Furniture

Provide lock and latch furniture suitable for use with the lock or latch to which it is installed with the corresponding level of performance.

Ten grades of security are available, SP 1 to SP 10 (see AS 4145.4 Table 3.1).

An example of padlock classification might be: SP 7-K 6-D 2-C 3.

The SP 7K 6D 2C 3 code denotes a padlock with highly physical and keying security, relatively high corrosion resistance but maximum durability, perhaps used in an exterior application of a storage area where valuable goods must be protected and the frequency of operation is small.

#### Strike plates

Use strike plates provided with the locks or latches.

#### Fire rated door closers

Provide closers tested and certified for use as components of fire door assemblies.

#### Door Controllers Performance

Provide door controllers, including door closers, floor or head spring pivots which are suitable for the door type, size, weight and swings required and the operating conditions, including wind pressure.

Specify or refer to a detail.

## Execution

For positioning of changes of floor finishes at doorways, refer to the relevant floor finish worksection in the FINISHES group.

Everything here should be detailed and scheduled. Do not rely on the specification for other than basic requirements. The installation methods described here are only some of those that may be used (if any) and should be coordinated with what is drawn.

### Frames

#### General

Install doors so that the frames:

- Are plumb, level and straight within acceptable building tolerances.

- Are fixed or anchored to the building structure to resist the wind loading.

- Will not carry any building loads, including loads caused by structural deflection.

- Allow for thermal movement.

#### Flashing and weatherings

Install moulds, sealant and cement pointing as required so that water is prevented from penetrating the building between the door frame and the building structure.

#### Aluminium frames

Building in to masonry: Screw galvanized steel brackets twice to jambs and build in.

Fixing to masonry openings: Use proprietary expansion anchors and screw through jambs at each fixing.

#### Frame fixing

Brackets: Metallic-coated steel:

- Width:  25 mm.

- Thickness:  1.5 mm.

Jamb fixing centres:  600 mm.

#### Fixing and fasteners

Materials: Use materials compatible with the item being fixed and of sufficient strength, size and quality to perform their function.

- Concealed fixings: Provide a corrosion resistant finish.

- Exposed fixings: Match exposed fixings to the material being fixed.

Support: Provide appropriate back support (for example blocking and backing plates) for hardware fixings.

Packing: Pack behind fixing points with durable full width packing.

Prepared masonry openings: If fixing timber door frames into existing prepared openings with fastenings through the frame face, make the fastener heads finish below the surface and fill the hole for a smooth surface finish.

#### Joints

Make accurately fitted tight joints so that neither fasteners nor fixing devices such as pins, screws, adhesives and pressure indentations are visible on exposed surfaces.

#### Operation

Ensure moving parts operate freely and smoothly, without binding or sticking and are lubricated.

#### Supply

Deliver door hardware items, ready for installation, in individual complete sets for each door.

- In a separate dust and moisture proof package labelled for the specific door.

- Including the necessary templates, fixings and fixing instructions.

- Refer to the drawings and Flush doors, Joinery doors, PVC doors, Security screen doors, Fire and smoke resistant doorset and Automatic door schedules for details of frames, doors and hardware.

### Completion

#### Cleaning

The Contractor is to clean all frames, doors, glass, hardware at completion. Any damage to frames and doors, or broken glass is to be repaired or replaced to the satisfaction of the Employers representative.

#### Adjustment

Leave the hardware properly adjusted with working parts in working order and lubricated where appropriate.

Action: E.g. single sliding panel, bi-parting sliding panels, revolving.

Glazing: Nominate here or refer to Glazing worksection.

Frame type: E.g. nominate an aluminium suite and finish, or frameless.

Refer to Door hardware for door control.