# Window coverings

Worksection application

This worksection is applicable to window coverings both decorative and blockout comprising, curtains and blinds, manual and power operated.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. See for example:

- Windows.

- Window hardware.

- Metalwork for the inclusion of sun screens and awnings.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

- Windows.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

Worksection cross-referencing

Other worksections that cross reference this worksection are:

- None.

Documenting this and related work

You may document this and related work as follows:

- Coordinate connections for power operated window coverings with the appropriate Electrical Services worksection.

- Locate and identify window coverings and operating systems on drawings to your office documentation policy.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes, do not repeat them here.). You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work. For example:

- Windows.

- Window hardware.

- Joinery for pelmets/recesses.

- Electrical design and install for connections for motorised units.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the building locations and surfaces prepared to receive window coverings before installation.

Hold points, if required, should be inserted here.

### Submissions

#### Samples

Submit 2 samples of each of the following where applicable, for approval by the Employers Representative:

- Sections proposed to be used for frames, louvres and slats.

- Finishes to prepared surfaces with trims.

- Colour range samples of fabrics, facings and production material.

- Manufacturer’s standard control system furniture items.

#### Fire hazard

Do not provide materials which, when subject to fire conditions, will emit excessive smoke or dangerous fumes. Submit certificates, if required, for approval by the Employers Representative.

2 is for “medium performance”, 1 is for short life fabrics, and 3 is for “high level of performance”. Edit as required.

## Products

### Internal

#### Curtains and fabric shades

Install curtains or shades to locations identified on drawings or in BOQ. Check all dimensions on site before fabricating track and making curtains. Refer to Curtain schedule.

Select the system, describe the functions and nominate the location of the equipment and switching.

#### Aluminium Venetian blinds

Install aluminium blinds to locations identified on drawings or in BOQ. Check all dimensions on site before fabricating track and making blinds. Refer to Aluminium venetian blind schedule.

Or as shown on drawings.

Omit the following if the product is nominated.

Slat material: High tensile aluminium alloy.

Thickness: 0.175 mm.

0.22 mm thick for 50 mm slats.

e.g. Required.

Cord: 1.5 mm thick polyester with braided jacket and safety tassel, and ladders for location and control.

Top and bottom rails: 0.5 mm aluminium zinc coated steel powdercoat finish.

Refer to manufacturer for 50 mm slat blinds.

e.g. Manual or motorised by system nominated by the manufacturer.

#### Plastic Venetian blinds

Install plastic blinds to locations identified on drawings or in BOQ. Check all dimensions on site before fabricating track and making blinds. Refer to Plastic venetian blind schedule.

Or as shown on drawings.

Omit the following if the product is nominated.

Slat material: Extruded polystyrene.

Thickness: 3 mm nominal.

Slat width: 50 mm nominal.

Cord: Internal 2.3 mm thick polyester.

Tape: To allow a 42 mm pitch between ladders and a 8 mm slat overlap.

Top and bottom rails: 0.5 mm aluminium zinc coated steel powdercoat finish.

e.g. Required, or as detailed.

Consult manufacturer’s catalogue for the available range.

e.g. Manual or motorised by system nominated by the manufacturer.

#### Timber Venetian blinds

Install timber blinds to locations identified on drawings or in BOQ. Check all dimensions on site before fabricating track and making blinds. Refer to Timber venetian blind schedule.

Omit the following if the product is nominated.

Slat material: Approved timber

e.g. Western red cedar, clear finish, or as nominated by the manufacturer for an opaque finish.

Thickness: 5 mm nominal.

Slat width: 50 mm nominal.

46 mm for clear finished timber, 50 mm for opaque finished timber.

Cord: Internal 2.3 mm thick polyester.

Tape: To allow a 42 mm pitch between ladders and a 8 mm slat overlap.

Top and bottom rails: 0.5 mm aluminium zinc coated steel powdercoat finish.

Consult manufacturer’s catalogue for the available range.

e.g. Manual or motorised by system nominated by the manufacturer.

Consult manufacturer’s catalogue for the available range.

#### Vertical louvre blinds

Install vertical blinds to locations identified on drawings or in BOQ. Check all dimensions on site before fabricating track and making blinds. Refer to Vertical louvre blind schedule.

Omit the following if the product is nominated.

Type: Louvres supported by a carrier system which traverses on wheels and operates with a friction spring loaded clutch mechanism.

Louvre blades: Heavy duty fabric or vinyl coated fabric blades in single, straight lengths finishing 10 mm above floor or sill level, without twists, warp, bows, edge ripples or fraying. Fix a weight into a pocket formed in the bottom of each blade.

Consult manufacturer’s catalogue for the available range.

Spacings: Space the blades evenly with plastic spacers which lock into the carrier rail to provide a continuous linkage, and fix with sealed plastic slat holders carried by plastic rotation pivots. Connect the bottoms of the blades by a plastic link chain with reversers.

Tracks: Fabricated to suit size of blades for width of window or within a pelmet extending past the face of the window.

e.g. “Anodised”, “Powder coated”.

Operation: Dual continuous loop chains controlling the functions of tilting and drawing.

e.g. “Centre opening”, “One way opening”.

## Execution

### Installation

#### General

Fixing: Secure the tracks with ceiling clamps or wall mounted brackets so that there are no fixings through the track.

Refer to details.

### Completion

Consult or negotiate with manufacturers for warranty terms, and specify only such terms as are actually available. Cross refer to Warranty schedule, General requirements.

#### Maintenance manual

Submit the manufacturers’ data as follows:

- Recommendations for service use, care and maintenance.

- List of manufacturers and suppliers of replacement parts.

Nominate items that require a maintenance manual.