# Lining

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to the installation of internal lining systems.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. See for example:

- Light timber framing for structural sheet bracing.

- Cladding for external cladding.

- Partitions – framed and lined for internal fitout partitions.

- Applied wall finishes for additional wall finishes applied over basic internal lining.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

- Light steel framing.

- Light timber framing.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

Worksection cross-referencing

Other worksections that cross reference this worksection are:

- None

Documenting this and related work

You may document this and related work as follows:

- Locate and schedule the sheet lay-up of framed partitions on drawings.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Light steel framing.

- Light timber framing.

- Cladding.

- Applied wall finishes.

### Inspection

#### Notice

Inspection: Give sufficient notice so that inspection may be made of the wall face or framing before installation of linings.

### Submissions

#### Samples

Plasterboard: Submit two 300x300mm samples of each type.

Fibre cement sheet: Submit two 300x300mm samples of each type.

Accessories: Submit samples of accessories, fasteners, trims and cornices.

### Tolerances

#### Surface

Flatness, twist and bow: ≤ 3.0 mm deviation from a 1.5 m straightedge placed in any position.

## Products

### Materials and components

#### Plasterboard

Plasterboard sheet lining is to be sheet material, size, type and thickness in accordance with the Sheet lining schedule, to the approval of the Employers representative.

i.e. square, recessed or bevelled to suit the specified joint finish. See Figure 1 of AS/NZS 2588.

#### Fibre cement

Fibre cement sheet lining is to be sheet material, size, type and thickness in accordance with the Sheet lining schedule, to the approval of the Employers representative.

#### Fasteners

Steel nails: Hot dip galvanized.

Screws: Coated steel cross head screws.

#### Adhesives

For plasterboard: Epoxy grout adhesive as supplied by the plasterboard sheet manufacturers.

For cement sheet: Mastic adhesive.

#### Sealants

Fire rated sealant: Non-hardening sealant compatible with the materials to be sealed and having a fire rating equal to that of the partition it seals.

Acoustic sealant: Non-hardening sealant compatible with the materials to be sealed and having a specific gravity of not less than 1.5 gm/cubic centimetre and of 100% polyurethane mastic.

## Execution

### Construction generally

#### Conditions

Do not commence lining work until such time as the building or zone in question is enclosed and weathertight and all wet trades have been completed.

#### Substrates or framing

Before fixing linings check and, if necessary, adjust the alignment of wall faces or framing. Make good any damaged areas that may affect the fixing of the lining. Ensure that there are no projections from the face of the wall structure that may affect the installation of the lining material.

#### Ceiling linings

Do not install ceilings until at least 14 days after the timber roof structure is fully loaded where this is used for support of the ceiling.

#### Accessories and trim

Provide accessories and trim necessary to complete the installation.

#### Adhesives

Provide adhesives of types appropriate to their purpose, and apply them so that they transmit the loads imposed, without causing discolouration of finished surfaces.

### Plasterboard lining

#### Supports

Install timber battens or galvanized steel channels as follows:

- Where framing member spacing exceeds the recommended spacing.

- Where direct fixing of the plasterboard is not possible due to the arrangement or alignment of the framing or wall face.

#### Installation

Gypsum plasterboard: Install strictly in accordance with manufacturers recommendations.

Framed construction: Screw or nail or combine with adhesive.

Masonry construction: Fix using adhesive direct to masonry.

Suspended ceilings: Fix using screw or screw and adhesive to ceiling members.

To steel channels: Fix using screw or screw and adhesive.

#### Multiple sheet layers

Application: Fire rated and acoustic rated walls.

Joints: Fill and flush up all joints and fixings in each layer and caulk up perimeters and penetrations before commencing succeeding layers. Stagger all sheet joints by minimum 200 mm.

#### Joints

Flush joints: Provide recessed edge sheets and finish flush using perforated paper reinforcing tape.

Butt joints: Make joints over framing members or otherwise provide back blocking.

External corner joints: Make joints over metallic-coated steel corner beads.

Control joints: Install purpose-made metallic-coated control joint beads at not more than 12 m centres in walls and ceilings and to coincide with structural movement joints.

### Fibre cement lining

#### Supports

Install timber battens or galvanized steel channels as follows:

- Where framing member spacing exceeds the recommended spacing.

- Where direct fixing of the fibre cement is not possible due to the arrangement or alignment of the framing or substrate.

#### Installation

Run sheets across the framing members. In flush jointed applications, stagger end joints in a brick pattern and locate them on framing members, away from the corners of large openings. Provide supports at edges and joints.

Timber framed construction: Nail only or combined with adhesive.

Steel framed construction: Screw only or combined with adhesive.

Masonry construction: Fix using adhesive direct to masonry.

Suspended flush ceilings: Fix using screw or screw and adhesive to ceiling members or support frame.

Ceilings and soffits: Provide battens where fixing to underside of rafters, roof trusses and purlins.

#### Multiple sheet layers

Application: Fire rated and acoustic rated walls.

Joints: Fill and flush up all joints and fixings in each layer and caulk up perimeters and penetrations before commencing succeeding layers. Stagger all sheet joints by minimum 200 mm.

#### Joints

Flush joints: Provide recessed edge sheets and finish flush using perforated paper reinforcing tape.

- Movement joints in walls: Position a stud parallel to the joint on each side.

- Movement joints in ceilings and soffits: Provide movement joints to divide ceilings into bays not larger than 10.8 x 7.2 m and soffit linings into bays not larger than 4.2 x 4.2 m or 5.4 x 3.6 m. Provide framing parallel to the joint on each side. Do not fix the lining to abutting building surfaces.

External corner joints: Make joints over metallic-coated steel corner beads.

Control joints: Provide purpose-made metallic-coated control joint beads at ≤ 7.2 m centres in walls and ceilings and to coincide with structural movement joints.

### Ceiling access

#### General

Location: Provide personnel access ways to each separate ceiling space.

Size (mm): Minimum of 600 x 600 mm

Material: Match adjacent ceiling lining.

Type: Plain cover supported on all sides by timber trim fixed to underside of ceiling.

### Cornices

#### General

Plasterboard cornice: Install plasterboard trims to the junctions between wall surfaces and ceilings as shown on the drawings.

Timber cornices: Install timber trims to the junctions between wall surfaces and ceilings as shown on the drawings.

### Completion

#### General

Ensure that all surfaces are protected, dry and free from damage before paint finishes are to be applied. All plasterboard and fibre cement surfaces must dry for at least 7 days before painting is commenced.

Fire rated materials (e.g. expanded vermiculite boards) can be most easily identified by proprietary item.

Location: e.g. bulkheads, fire-rated ceiling, room type or identifier, or use Lining designation on drawings or in a Finishes schedule or Wall type schedule.

Configuration: e.g. “Horizontal”, “Vertical”, “Double thickness”.

Joint type: e.g. “Butt” or “Flush”.

Level of finish: To AS/NZS 2589.1

Level 0	Temporary construction	Appendix C

Level 1	Concealed work	Appendix D

Level 2	Warehouse, storage area	Appendix E

Level 3	For heavy texture or paper finish	Appendix F

Level 4	Domestic construction	Clause 7

Level 5	For gloss or semi-gloss paint finish	Appendix G