# Insulation and vapour barriers

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to insulation, sarking and vapour barriers for roofs, ceilings, walls and floors. It is generally specified around AS 3999, AS/NZS 4200.1, AS/NZS 4200.2 and AS/NZS 4859.1.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- There are no directly related NATSPEC worksections.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection. For example:

- Insulation forming part of a proprietary roofing, cladding or lining system.

- Worksection cross referencing

- Other worksections that cross reference this worksection are:

- None.

- This worksection does not relate to insulation for Services (e.g. for ductwork, which is dealt with in Ductwork insulation).

- Documenting this and related work

- You may document this and related work as follows:

- Insulation and vapour barriers may be integral to other worksections such as Concrete in situ, Waterproofing – external, Roofing, Cladding, Curtain walls, Partitions – framed and lined, Suspended ceilings and Waterproofing – wet areas. Cross reference from related worksections to this worksection or take relevant text from here.

- Show of the drawings the extent, type, location, arrangement, fixing and support details of all insulation, sarking and vapour barriers. Particular requirements, if any, which are not covered by the worksection text and not shown on the drawings may be specified in this worksection.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Concrete in situ for underslab insulation.

- Waterproofing – external.

- Roofing.

- Suspended ceilings.

- Partitions – framed and lined.

- Waterproofing – wet areas.

### Interpretation

#### Definitions

This definitions subclause can be edited to suit the project or deleted if not required.

General: For the purposes of this worksection the definitions given below apply.

This is the BCA’s interpretation of ‘Fire hazard properties’ at clause A 1.1.

- Sarking-type material: Flexible reflective foil membrane material normally used for waterproofing, vapour proofing or thermal reflectance.

Derived from the BCA clause A 1.1.

- Mineral wool (including glasswool and rockwool): Entangled mat of fibrous non-crystalline material derived from inorganic oxides or minerals, rock, slag or glass, processed at high temperatures from a molten state.

In Australia, two classes of mineral wool are produced, rockwool and glasswool.

- Vapour barrier: A material or system that adequately impedes the transmission of water vapour.

This definition is from ASTM C168 which uses the term ‘water vapor retarded (barrier)”. To qualify as a sarking membrane or vapour barrier, the material or system has to be continuous i.e. any joints in the membrane must be sealed airtight.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the insulation to roof space in areas which will be covered up or concealed.

Hold points, if required, should be inserted here.

## Products

### Materials and components

A BCA requirement for Class 2 to 9 buildings, here extended to all buildings, see BCA specification C 1.10 clause 2(a). A flammability index for sarking greater than 5 is generally not permitted. Reflective foil to AS/NZS 4200.1 classification “low” satisfies this requirement.

Note the BCA requirement for exposed sarking-type material in fire-isolated exits to have a flammability index of 0 – not covered in AS/NZS 4200.1. See BCA Specification C 1.10 clause 3(b).

#### Bulk insulation

Mineral wool blankets and cut pieces: Provided in bulk rolls for laying over roof structures or on roof slabs and batts to suit ceiling member spacing.

Polystyrene (extruded rigid cellular sheets): Provided in modular panels for fixing to walls and roof slabs.

Reflective insulation: Provided in bulk foil rolls for laying over roof structures and foil batts to suit ceiling member spacing.

Delete standards which are not applicable to the project.

#### Sarking-type material

Sarking: Reflective foil fixed as a membrane to reduce liquid water transfer in walls or roof structures but allow water vapour to move through the building envelope.

Vapour barrier: Reflective foil sealed as a membrane to stop all liquid water and water vapour transfer.

AS 3999 is specified because AS/NZS 4200.1 and AS 3999 give different figures. The upper limit in AS 3999 (.06 g/N.s) corresponds to the middle of the “medium” range in AS/NZS 4200.1 (see AS 3999 clause  1.6.3). Different permeances apply to vapour and water barriers.

To permit condensate to drain away, maintaining the performance of the membrane. An airspace is also essential.

For reflective foil laminate in critical situations, such as external exposure under unlined eaves, consider specifying material carrying the appropriate Australian StandardsMark.

#### Fasteners and supports

Metallic-coated steel.

AS 2423 covers wire netting and chainwire and requires that all products be protected against corrosion by application of a metallic-coating during manufacture, optionally overlaid with an organic coating, see AS 2423 clause  1.3.13.

Welded safety mesh may be required by OH&S authorities for commercial and industrial buildings, along with perimeter guard rails. Coordinate with the Roofing worksection which also cites AS/NZS 4389.

## Execution

### General

#### Bulk insulation

AS 3999 deals in part with vapour barriers used in conjunction with bulk insulation.

Batts: Fit tightly between framing members. If support is not otherwise provided, fix over wire mesh stapled to the framing and stretch tight.

AS 3999 gives some directives on fixing of insulation often deferring to the manufacturer’s recommendations on the type and spacing of fixing devices. Preferably show fixing details on the drawings.

AS/NZS 4200.2 requires wire support for extra heavy duty sarking over framing exceeding 900 mm centres for tiled roofs. For other duty ratings and applications, see the standard. Mesh support for sound (rain noise) insulation for metal roofing must be taut in order to keep the insulation in contact with the roof sheeting. Use a sarking membrane to dispose of water which leaks through the roofing, or which condenses on the underside of the roofing (e.g. metal roof sheets) under cold conditions.

#### Reflective foil laminate

To timber: Metallic-coated flat head nails or staples at 300 mm maximum centres.

AS 2334 defines clouts as nails having a plain cylindrical shank with an integrally formed flat circular head. They are normally supplied with a hot-dip galvanized coating or with an electro-deposited or mechanically deposited zinc coating giving equivalent corrosion protection, and are mainly intended for the general fixing of thin sheet materials.

To steel or aluminium: Double sided pressure sensitive tape.

Normally 50%.

Overlap (minimum): 150 mm and adhesive fix.

#### Roof sarking locations

Location: Provide sarking under metal sheet roofs. Fix over timber supports and run rolls across roof plane. Overlap each layer of foil so that any water will run down the slope and discharge into the gutter without dripping into the roof space.

Welded safety mesh, wire netting, steel or nylon wires.

For cavity wall insulation, usually held in place with purpose-made plastic clips over purpose-made wall ties. Sheet size should suit wall tie row spacings, and maintain a clear cavity.

### Roof insulation

Refer to Instruction text in Floor insulation.

#### General

Location: The whole of the ceiling area, except the following:

State if insulation is not required in semi-enclosed spaces (balconies, verandahs) or ancillary buildings (garages, workshops, carports etc.).

- Eaves, overhangs, roof lights, vents and openings.

- Roofs to outbuildings, garages, and semi-enclosed spaces such as verandahs, porches.

Separate or combined bulk insulation and vapour barrier are alternatives. Edit as required.

#### Installation

Welded safety mesh, wire netting, steel or nylon wires.

Refer to the drawings and Insulation schedule for details of insulation requirements for the works.