# Roofing

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to the provision of roof coverings of metal and plastic sheet; concrete, clay and metal tiles; slate roofing and timber shingles and shakes; and associated elements such as roof plumbing, rainwater tanks, roof lights, ventilators and metal wall cladding.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Tensioned membrane structures for suspended fabric roofing.

- Waterproofing – external for membrane roofs.

- Cladding for wall cladding other than metal sheet.

NATSPEC worksection(s) on which this worksection relies

The following worksections must be included, if this worksection is used, to provide a complete specification:

- General requirements.

Associated material not included in NATSPEC

Some projects may include items not covered by this worksection. You may need to create new text, modify this text or use an applicable worksection. Examples include:

- In situ installed pure zinc folded seam and pan roofing.

Worksection cross-referencing

Other worksections that reference this worksection are:

- Hydraulic – design and install.

- Wastewater.

Documenting this and related work

You may document this and related work as follows:

- Locate the extent of roofing types, accessories, and finishes on drawings to your office documentation policy.

- Roof plumbing: Show on the drawings the arrangement of the rainwater system including the type and size of the main components (gutters, downpipes, sumps, rainheads, etc.) and the size and spacing of supports and fixings.

- For smoke and heat venting, see AS 2665 which is cited in the BCA as a primary reference Standard.

- Specify stormwater disposal in Hydraulic – design and install or Wastewater.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes, do not repeat them here.). You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Hydraulic – design and install or Wastewater.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of:

- Roof framing during construction.

- Those parts of the roofing, vapour barrier, insulation and roof plumbing installation which will be covered up or concealed.

Hold points, if required, should be inserted here.

### Submissions

#### Samples

List here any samples required other than those specified in the Template. Approved samples are retained on site and define the acceptable limits of colour and texture variation. If particular or additional samples are required, e.g. samples for testing, list them here. See AS 4046 Parts 0 to 9 for testing of roof tiles.

Submit samples of the following showing the range of variation available:

- Corrugated and flat metal roof sheeting

- Roof cappings, flashings, gutters, downpipes

## Products

### Materials and components

#### Fasteners

Self-drilling screws: Corrosion resistant screws to approval of Employers representative.

Nuts and bolts: Corrosion resistant fastenings to approval of Employers representative.

Fastenings to timber battens or purlins: Provide fastenings just long enough to penetrate the thickness of the timber without piercing the underside.

Add locations as required.

### Corrugated metal roofing and cladding

#### General

Consult manufacturer’s literature for the correct description of the roofing system and available finishes. Vary as required.

Provide a proprietary system of preformed corrugated sheets and all purpose-made accessories required to complete the installation to roof framing or wall framing. Refer to Corrugated sheet roofing materials schedule for details.

Use manufacturer’s description.

### Flat Sheet metal Roofing and cladding

#### General

Provide a system of flat metal sheets and all purpose-made accessories required to complete the installation to roof framing or wall framing. Refer to Flat sheet roofing materials schedule for details.

### Glazed roofing and skylights

#### General

Provide a proprietary overhead glazing system fixed to glazing bars or directly to the roof framing. Provide all purpose-made accessories required to complete the installation. Refer to Glazed roofing materials and skylight schedule for details.

Add SHGC and U-value where called for in the BCA.

e.g. BCA Fire hazard properties – general if GRP (FR) is nominated.

Add SHGC and U-value where called for in the BCA.

### Roof ventilators

For electric fan powered ventilators specify the necessary electrical connection in the electrical services worksection. Specify any particular requirements, material, type (e.g. static, wind driven, electric fan powered), size, etc. if not shown on the drawings. For roof mounted heat exhaust vents, see AS 2427.

#### General

Provide proprietary roof mounted ventilators and all purpose-made accessories required to complete the installation where shown on the drawings to ventilate the roof space.

Provide fabricated ventilators in walls as shown on the drawings to ventilate the roof space.

Refer to Roof ventilators schedule for details.

## Execution

### Installation

#### Protection

Keep the roofing and rainwater system free of debris and loose material during construction, and leave them clean and unobstructed on completion. Repair damage to the roofing and rainwater system.

If it is necessary to repair minor damage to metal roofing, do so immediately after the damage has occurred. The Contractor is take care to not damage other surfaces during the repair works.

#### Thermal movement

Provide for thermal movement in the roof installation and the structure, including movement in joints and fastenings.

These tolerances are from AS 1562.1.

Coordinate with the Insulation and vapour barriers worksection. Do not call up welded safety mesh in more than one spot.

### Sheet metal roofing and cladding

#### Roofing sheet installation

Fixings: Provide all fixings required to fix the roof sheeting to the framing so that the entire roof covering is waterproof and secure. All loose edges are to be fixed down to ensure that they cannot get loose in high winds.

For work in cyclone areas of the Northern Territory compliance with NT “Deemed to Comply Standard” will be required. Cyclone washers must be used unless evidence is produced to prove they are not required.

Expansion joints: refer to drawings for locations of expansion joints in roofs and details of construction.

Expansion joints should be provided every 35 m in sheet length for roofs with concealed fixings and 24 m in sheet length for roofs with exposed fixings.

#### Ridges and eaves

On birds and buildings, see BDP EDG Note DES 56.

Treat ends of sheets as follows:

- Project sheets 50 mm into gutters.

- Close off ribs of ribbed sheeting at bottom of sheets using mechanical means or with purpose-made end caps.

- Turn pans of ribbed sheeting up at tops and down into gutters by mechanical means.

- Provide pre-cut notched eaves flashings and bird proofing wire mesh where necessary.

#### Ridge and eaves capping

Finish off along ridge and side eaves edges with purpose-made ridge capping and eaves cappings.

This is possible only with certain sheeting profiles and roof slopes. Consult the manufacturer about recommended purlin spacings at the ridge to achieve the required curvature. Show the purlin locations on the drawings.

#### End laps

Where end laps are unavoidable in roof sheeting, and the sheet profile is not suitable for interlocking or contact end laps, construct a stepped type lap. Refer to details on drawings as required.

Length of lap (mm): Laps to ends of sheets should not be less than 150mm and sealed with a continuous line of silicone sealant between the sheets of roofing.

Specify the laps required, if applicable.

#### Curved corrugated sheet

Form by rolling from material recommended for curving. Minimise crimping or creasing across the face of the sheet. Trim off crimped or creased edges and ends.

K-Span roofing where identified on the drawings is to be strictly controlled during the installation process to ensure that the completed work is of a high standard.

#### Cladding sheet installation

Fixings: Provide all fixings required to fix the wall cladding sheeting to the framing so that the entire wall is waterproof and secure. All loose edges are to be fixed to ensure that they cannot get loose in high winds.

For work in cyclone areas of the Northern Territory compliance with NT “Deemed to Comply Standard” will be required. Cyclone washers must be used unless evidence is produced to prove they are not required.

Expansion joints: refer to drawings for locations of expansion joints in walls and details of construction.

Flashings: Flashings are required at the top, sides and bottom of all metal wall cladding to ensure that the wall is waterproof in all weather conditions.

#### Metal separation

The design process should ensure compatibility or detail separation.

Prevent direct contact between incompatible metals, and between green hardwood and aluminium or coated steel, by either:

- Applying an anti-corrosion, low moisture transmission coating to contact surfaces.

- Inserting a separation layer.

Tile verge or Pointed verge: Select alternatives. Use tile verge in high wind areas.

Excluding safety mesh, to AS/NZS 4389. AS/NZS 1562.2 is cited in the BCA for weatherproofing.

### Glazed roofing and skylights

#### Installation

Fixing: Fix all glazed roof panels and skylights in accordance with the drawings.

Flashings: Flashings are required at the top, sides and bottom of all glazed roof panels and skylights to ensure that the roof is waterproof in all weather conditions.

Specify and detail to the recommendations of the glazing bar manufacturer.

e.g. Roofing screws with neoprene washers in oversized holes. Consult manufacturer.

Specify and detail to the recommendations of the skylight manufacturer.

### Roof ventilators

#### Installation

Fixing: Fix roof ventilators in accordance with the manufacturer’s construction details or in accordance with the drawings for fabricated ventilators.

Specify and detail to the recommendations of the roof window manufacturer.

### Roof plumbing

Refer to the Gutter and downpipe schedule for details.

#### General

Provide the flashings, cappings, gutters, rainwater heads, outlets and downpipes necessary to complete the roof system.

#### Jointing sheet metal rainwater goods

Butt joints: Make joints over a backing strip of the same material.

Soldered joints: Do not solder aluminium or aluminium/zinc-coated steel.

Sealing: Seal fasteners and mechanically fastened joints. Fill the holes of blind rivets with silicone sealant.

Jointing system: Refer to the Gutter and downpipe schedule for specific jointing details for each type of element.

e.g. Blind rivet and seal or a proprietary system.

#### Flashings and cappings

Installation: Flash roof junctions, upstands, abutments and projections through the roof. Preform to required shapes where possible. Cut, notch, bend or dress down as necessary to follow the profile of adjacent surfaces. Lap joints 150 mm in running lengths. Provide matching expansion joints at 6 m maximum intervals.

6 m corresponds to the manufacturing length, but movement at these joints would be less than 1 mm so they perhaps do not all need to be fully-fledged expansion joints.

Upstands: Flash projections above or through the roof with two part flashings, consisting of a base flashing and a cover flashing, with at least 100 mm vertical overlap. Provide for independent movement between the roof and the projection.

Wall abutments: Provide overflashings where roofs abut walls, stepped to the roof slope in brickwork.

- In masonry: Build cover flashing at least 100mm into the wall at least 250mm above the roof level. Provide base flashing on roof and provide at least 100mm vertical overlap.

- In concrete: Turn cover flashing at least 30 mm into sawcut grooves at least 250mm above the roof level, wedge at 200 mm centres with compatible material and render over top of flashing. Provide base flashing on roof and provide at least 100mm vertical overlap.

Fixing to pipes: Solder, or seal with neutral cured silicone rubber and either of the following:

- Secure with a clamping ring.

- Provide a proprietary flexible clamping shoe with attached metal surround flashing.

#### Gutters

Prefabricate gutters to the required shape where possible. Form stop ends, bends and returns. Provide overflows to prevent back-flooding.

Gutter and sump support: Provide framing and lining to support valley gutters, box gutters and sumps. Line the whole area under the gutters and sumps.

Support: Steel straps as shown on drawings or as approved by the Employers representative.

e.g. Proprietary metallic-coated adjustable strap and channel system.

Lining: Timber boards or plywood as shown on drawings or as approved by the Employers representative.

e.g. Spandeck

Valley gutters: Profile to suit the valley boarding. Nail or screw to the valley boarding at the top end to prevent the gutter creeping downwards.

Gratings and guards: Provide removable gratings over rainwater heads and sumps:

- Type: Wire mesh cages reinforced with steel bars where required due to size and expected snow loads. Refer to drawings for details.

e.g. Plastic mesh or proprietary metal guards to match the gutter profile.

Expansion joints: Provide expansion joints in guttering longer than 30 m:

- Type: Refer to drawings for details.

e.g. As detailed or proprietary elastic expanding adhesive fixed type.

#### Downpipes

Specify the material, profile and size in the Gutter and downpipe schedule if not shown on the drawings. Refer to commentary for Flashings and cappings schedule.

In high wind areas consider the degree of exposure of gutters and downpipes to wind loading and the need to provide additional fixings.

Prefabricate downpipes to the required section and shape where possible. Connect heads to gutter outlets and, if applicable, connect feet to rainwater drains.

Access cover: Provide a removable watertight access cover at the foot of each downpipe stack if the downpipe is connected to rainwater drains.

Downpipe support: Provide supports and fixings for downpipes.

Specify here requirements not shown on the drawings. If there is more than one size of gutter or downpipe list them separately against the designation used on the drawing.

### ROOF mounted equipment ACCESS

#### Walkway

Product: Provide proprietary walkway system to locations as shown on drawings. Provide fabricated system constructed as shown on drawings. Fabricate in accordance with metalwork section of the specification.

Installation: Install proprietary systems in accordance with manufacturer’s details and as identified on drawings.

Cross refer ladders, platforms and balustrades to the appropriate worksection i.e. Metalwork, Structural steel.

e.g. Multiple side fixings to roofing ribs, or direct fixing to roof structure with flashings at penetrations.

### Completion

#### Roof Inspection

The Contractor is to closely inspect the entire roof covering and metal cladding to walls at completion of the works.

Make good any defects or damage to the sheeting, cappings or flashings. Remove all loose metal and other rubbish, spare nails, screws, filings and other debris.

Clean down the roof, gutters, and downpipe outlets to ensure that it is good condition ready for occupation.