# fire Extinguishers and blankets

Worksection application

This worksection is applicable to fire extinguishers and fire blankets.  Products used must be on the Scientific Services Laboratory Register of Accredited Products - Fire Protection Equipment.  The specification should reflect any extra requirements of local statutory authorities.  The principal product standards are the AS/NZS 1841 series and AS 2444.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Fire-stopping, for fire stopping materials and accessories.

- Hydrants, Hose reels, Sprinklers and Fire detection and alarms, for fire systems.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

- Worksection cross referencing

- Other worksections that cross reference this worksection are:

- Signs and display.

- Documenting this and related work

- You may document this and related work as follows:

- Statutory signs related to fire systems in Signs and display.

- Statutory signs related to the location of portable fire extinguishers in Signs and display.

- Locate extinguishers and blankets on drawings to your office documentation policy.

## General

### Samples

#### General

Provide samples of all fire extinguishers proposed for use in the project for approval of the Employers representative.

The General requirements worksection contains umbrella requirements for referenced documents, inspections, tests, samples, contractor’s submissions, proprietary items, manufacturers’ recommendations, warranties, record drawings, operation and maintenance manuals, and the like.

They need not be specified here unless you need to relocate them, for example if this is to be the specification for a package contract. In this case make sure that the interlocking contracts leave no loopholes, such as work by non-existent ‘others’, and have no overlaps.

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Fire-stopping.

- Signs and display.

- Hydrants.

- Hose reels.

- Sprinklers.

- Fire detection and alarms.

### Authorised products

#### General

Provide equipment from Certified manufacturers only. Provide copies of the test certificates if requested by the Employers representative.

The Register of Fire Protection Equipment, as issued by Scientific Services Laboratory under its ActivFire Scheme includes portable extinguishers and residential fire protection equipment.

## Products

### Extinguishers

#### Extinguisher type and location

Provide portable fire extinguisher types and matching signs to the locations identified in the Fire extinguishers schedule.

Normally selection and location is the designer’s responsibility, not the contractor’s.

Location: and Number: Usually shown on the drawings.

Unit type: Refer to AS 2444 and AS/NZS 1850 for available options, listed under Standards.

Classification and rating: Refer to AS/NZS 1850 for available options e.g. 4A, 40B.

Maximum force to operate lever: Use where this is likely to be critical e.g. 100 N.

Cabinet:

- Type: Details may be shown on the drawings.

- Security: e.g. break glass.

Waterproof cover: Size and type.

### Blankets

#### Fire blanket type and location

Provide fire blanket types and matching signs to the locations identified in the Fire blanket schedule.