# Site preparation

Worksection application

This worksection is applicable to clearing, environmental protection and retention of trees.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Earthwork.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection. For example:

- None.

- Worksection cross referencing

- Other worksections that cross reference this worksection are:

- None.

- Documenting this and related work

- You may document this and related work as follows:

- Show the location and type of temporary and permanent erosion and sediment control on the drawings.

## General

### Aims

#### Responsibilities

The aim of this worksection is to clear the site and put in place adequate environmental controls to allow the commencement of earthworks and/or building works.

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

Add Hold points, if required.

### Submissions

#### Execution

Submit the methods and equipment proposed for the earthworks, including the following:

- Dewatering and groundwater control and disposal of surface water.

- Control of erosion and contamination of the site, surrounding areas and drainage systems.

- Dust control.

- Noise control.

## execution

### Trees

#### Work near trees

Keep the area within the dripline free of construction material and debris. Do not place bulk materials and harmful materials under or near trees. Do not place spoil from excavations against tree trunks. Prevent wind-blown materials such as cement from harming trees and plants.

Prevent damage to tree bark. Do not attach stays, guys and the like to trees.

If excavation is required near trees to be retained, give notice and obtain instructions. Open up excavations under tree canopies for as short a period as possible.

Use hand methods to locate, expose and cleanly remove the roots on the line of excavation. If it is necessary to excavate within the drip line, use hand methods such that root systems are preserved intact and undamaged.

Backfill to excavations around tree roots with backfill free from weed growth and harmful materials. Place the backfill layers, each of 300 mm maximum depth, compacted to a dry density similar to that of the original or surrounding soil. Do not backfill around tree trunks to a height greater than 300 mm above the original ground surface. Immediately after backfilling, thoroughly water the root zone surrounding the tree.

Water trees as necessary, including where roots are exposed at ambient temperature > 35C.

### Existing services

#### Marking

Before commencing earthworks, locate and mark existing underground services in the areas which will be affected by the earthworks operations including clearing, excavating and trenching.

#### Excavation

Do not excavate by machine within 1 m of existing underground services.

DIAL 1100 BEFORE YOU DIG is a free service, from anywhere in Australia, of locating underground pipe and cables (possible within two working days). See www.dialbeforeyoudig.com.au.

### Environmental protection

See AS ISO 14050 for definitions of fundamental concepts related to environmental management and BDP DES29 for a checklist of environmental issues on major construction sites.

#### Dust protection

Provide dust-proof screens, bulkheads and covers to protect existing finishes and the immediate environment from dust and debris.

Permanent erosion control measures should be designed as part of the works. Temporary measures are the responsibility of the contractor; however in special cases it may be necessary to specify particular requirements for some of the temporary measures to be adopted.

#### Dewatering

Keep groundworks free of water. Provide and maintain slopes and drains on excavations and embankments to ensure free drainage. Place construction, including fill, masonry, concrete and services, on ground from which free water has been removed. Prevent water flow over freshly laid work.

Dewatering operations as necessary to keep the works free of surface water. These could include, where necessary, pumping from excavations or external dewatering by deep wells or well points.

Where a particular dewatering system is necessary for design reasons or as part of the permanent work, specify requirements here or show on the drawings.

### Site clearing

#### Extent

Ensure that landscaped areas are shown as such on the drawings if they are to be cleared as specified. If site clearing is sufficiently variable and extensive, consider making a special drawing.

Clear only the following site areas:

- Areas to be occupied by works such as buildings, paving, excavation, regrading and landscaping.

- Other areas designated to be cleared.

Contractor’s site areas: If not included within the areas specified above, clear generally only to the extent necessary for the performance of the works.

#### Clearing and grubbing

Remove everything on or above the site surface, including rubbish, scrap, grass, vegetable matter and organic debris, scrub, trees, timber, stumps, boulders and rubble.

Remove tree stumps and roots over 75 mm diameter to a minimum depth of 500 mm below subgrade under buildings, embankments or paving, or 300 mm below finished surface in unpaved areas. Holes remaining after grubbing shall be backfilled with sand material to prevent ponding of water. The material shall be compacted to the relative density of the existing adjacent ground material.

Clearing: and Grubbing: Will apply to most areas.

Old works: Remove old works, including slabs, foundations, pavings, drains and manholes found on the surface unless identified on the drawings to remain intact.

Topsoil

All topsoil shall be stripped over the area on which construction or grading takes place. This topsoil shall be carefully stockpiled to be reused for landscaping on completion of the building operations or otherwise disposed of as directed.

### Disposal of materials

#### Disposal

General: Remove cleared and grubbed material from the site.

If cleared and grubbed material is to be disposed of other than by removal from the site specify here.

Include if mulch can be reused on site. Exclude noxious weeds and material which it may not be possible to chip (e.g. ironbark).