# Glazing

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to glazing in framed openings to windows, doors and curtain walls based around AS 1288, and to glass products such as shower screens and mirrors.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. See for example:

- Curtain walls for curtain walls.

- Structural glazing for adhesive fixed glazing.

- Glass blockwork for glass blockwork.

- Partitions – glazed for glazed internal partitions.

- Applied wall finishes for glass lining to sheeted partitions.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

- Windows.

- Doors and hatches.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

- Glazed balustrades as proprietary items or complete assemblies.

Worksection cross-referencing

Other worksections that cross reference this worksection are:

- Curtain walls.

- Joinery.

- Metalwork.

Documenting this and related work

You may document this and related work as follows:

- Show glass types as part of a window schedule to your office documentation policy.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Windows.

- Doors and hatches.

### Inspection

#### Notice

Inspection: Give sufficient notice so that inspection may be made of the following:

- Glass products before they are installed.

Nominate items to be inspected .Hold points, if required, should be inserted here.

## Products

### Glass

#### Glass and glazing materials

Glass and glazing materials generally: Free from defects which detract from appearance or interfere with performance under normal conditions of use.

Glazing plastics: Free from surface abrasions, and warranted by the manufacturer for 10 years against yellowing or other colour change, loss of strength and impact resistance, and general deterioration.

Refer to Annealed glasses, Processed glasses and Fabricated glass units schedules for specific details for the works.

Glass types (constituent glasses): In principle any of the above annealed and processed glasses may be used; consult manufacturers for combinations actually available.

Laminated safety glass:

- Interlayer: May be coloured, reflective, etc. Consult manufacturer.

Laminated security: See AS/NZS 2343 for transparent, opaque and translucent panels (glass or plastics).

- Ballistic attack resistance: See AS/NZS 2343 clause 4 for classifications.

### Glazing materials

#### General

Glazing materials (including putty, glazing compounds, sealants, gaskets, glazing tapes, spacers, setting blocks): Appropriate for the conditions of application and the required performance.

#### Jointing materials

Provide recommended jointing and pointing materials which are compatible with each other and with the contact surfaces and non staining to finished surfaces. Do not provide bituminous materials on absorbent surfaces.

#### Pile weather strips

Materials: Polypropylene or equivalent pile and backing, low friction silicone treated, ultra violet stabilised.

Finned type: A pile weather seal with a central polypropylene fin bonded into the centre of the backing rod and raised above the pile level.

#### Extruded gaskets and seals

Type: Non cellular (solid) seals to exclude water from glass/frame junctions.

Material:

- Rubber products to be neoprene, ethylene propylene diene monomer (EPDM) or silicone rubber.

- Flexible polyvinyl chloride (PVC)

#### Priming

Apply the recommended primer to the surfaces in contact with sealant materials.

#### Movement joints

Depth of elastomeric sealant: One half the joint width, or 6 mm, whichever is the greater.

Foamed materials (in compressible fillers and backing rods): Closed-cell or impregnated types which do not absorb water.

Bond breaking: Provide backing rods, and other back-up materials for sealants, which do not adhere to the sealant.

#### Glazing films

Supply films identified in the schedules to approval of the Employers representative. All films are to be proprietary products installed strictly in a accordance with the manufacturers instructions.

### Mirrors

Refer to Mirrors schedule for details.

#### Reflective surface

Type: Silver layer deposited on the glass or glazing plastic.

Size: Check AS 1428.1 for disabled persons facilities.

## Execution

### Glass processing

#### General

Perform required processes on glass, including cutting, obscuring, silvering and bending. Form necessary holes, including for fixings, equipment, access holes and speaking holes. Process exposed glass edges to a finish that will reduce the risk of injury.

Glass element: e.g. “Mirrors”, “Shelves and divisions”, “Sliding glass doors”.

Type of process: For example:

- Edge finishes: Arris, flat, round, bullnose, etc. Ground arrised edges are most common. Edges of louvre blades are usually “rubbed” or “ground”.

- Bevelling: Plain, mitre, feather edge, double, etc. Specify a width for plain and feather. Finish is usually polished.

- Finger slots: Specify size, e.g. “65 x 20 x 3 mm”.

- Finish: Smooth ground or polished.

- Obscuring: Sandblast obscuring, emery obscuring, acid embossing.

- Various finishes: See BS 952-2, or consult glass merchant.

- Safety marking: As recommended in AS 1288 clause 5.1 note 6.

### Installation

#### General

Install the glass so that:

- Each piece is held firmly in place by permanent means which enable it to withstand the normal loadings and ambient conditions at its location without distortion or damage to glass and glazing materials.

- Building movements are not transferred to the glass.

- External glazing is watertight and airtight.

Toughened glass: Do not cut, work, or permanently mark after toughening. Use installation methods which prevent the glass making direct contact with metals or other non-resilient materials.

Frameless installations: Join the vertical edges of adjacent glass panels with silicone jointing compound.

External timber framed glazing: Glaze with putty. Do not dry bead into timber frames.

### Fixing mirrors

The worksection gives four examples of fixing methods. Select one, or vary to suit the design.

#### Screw fixing

Direct to wall plugs with dome-headed chromium-plated screws in each corner and at 900 mm maximum centres around perimeter. Provide polyethylene sleeves and washers to prevent contact between screw and glass. Do not over-tension the screws.

#### Frame fixing

In wet or moist areas the space behind the mirror should be either well ventilated or entirely sealed.

Proprietary aluminium frames to mirror perimeter, corners mitred. Attach the frame to the wall with concealed screw fixings. Frames and finish to approval of the Employers representative.

#### Bead fixing

Rebated timber beads to mirror perimeter, corners mitred. Screw fix the beads to the substrate.

### Glazed shower screens

#### Type

Proprietary system comprising frames of extruded aluminium, stainless steel, or PVC, assembled around safety glass to form fixed panels and sliding, hinged or pivoted doors.

Specify if different from the Template.

#### Water shedding

Provide an assembly which sheds water to the inside without retaining it on the frame surfaces. Seal the edge of the frame to adjoining surfaces with a resilient strip.

#### Sliding assemblies

Hanging: Hang the sliding sash on stainless steel or nylon sheaves on overhead channel track formed in the frame head, and fit nylon or equivalent bottom guides.

Hardware: Pull handles on both sides of sash, or of leading sash in multiple sash arrangements.

### Completion

#### Cleaning

Replace damaged glass and leave the work clean, polished, free from defects, and in good condition.

Glass type: As nominated in Products

Glass properties:

- Total U-value of the transparent or translucent element in each glazing element.

- SHGC: Solar heat gain coefficient of the transparent or translucent element in each glazing element.

These properties together with the frame factor (BCA Table J2.3b) are required for the calculation of deemed to satisfy provisions under J2.3.