# Joinery

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to the shop fabrication and site installation of custom made joinery fixtures.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Metalwork for custom-made metal fixtures.

- Applied wall finishes for fabricated and in situ applied wall finishes.

- Painting for site painting.

Worksection(s) on which this worksection relies

The following worksections must be included, if this worksection is used, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by this worksection. You may need to create new text, modify this text or use an applicable worksection. Examples include:

- Shop painting opaque lacquers and enamels, and clear finishes.

Worksection cross-referencing

Other worksections that reference this worksection are:

- None.

Documenting this and related work

You may document this and related work as follows:

- Note additional noggings or stud stiffeners required for the support of wall hung units in Partitions – framed and lined.

- Import applicable material from Stainless steel benching, or Metalwork, for stainless steel features to joinery fixtures.

- Import applicable material from Stone and terrazzo tiling, for stone finishes to joinery fixtures.

- Import applicable material from Glazing for processed glass features or finishes.

## General

### tolerances

#### Responsibilities

Fabricate and install joinery items. Items to be undamaged, plumb, level, straight and free of distortion and to the Tolerances table.

#### Tolerances Table

| Property | Tolerance criteria |
| --- | --- |
| Plumb and level | 2 mm in 800 mm |
| Offsets in flush adjoining surfaces | < 1 mm |
| Alignment of adjoining doors | < 1.5 mm |

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Glazing.

- Metalwork.

- Stainless steel benching.

- Stone and terrazzo tiling.

- Applied wall finishes.

- Painting.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following:

- Shop fabricated or assembled items ready for delivery to the site.

- Site erected assemblies on completion of erection.

Hold points, if required, should be inserted here.

### Submissions

#### Samples

Submit samples to the Sample table for approval by the Employers representative.

Sample Table

| Description | No. of samples |
| --- | --- |
| Each type of board to be used complete with finish and edge stripping | 2 |
| Typical item of hardware indicating each finish | 2 |
| Stone benchtop indicating range of colours | 2 |
| Timber balustrade section | 1 |
| The finish to all stainless steel items | 2 |
| Complete timber bench cupboard door, including hardware | 1 |
| Complete drawer front, including hardware | 1 |

Samples cost money and take up time. Delete or amend this clause if sheets, finishes and edges are conventional and if proprietary hardware is specified.

If delivery involves a goods lift nominate the car size and capacity.

These shop drawing requirements assume the installation is fully detailed. Edit as required.

## Products

The prompts for this subsection set out the range of characteristics which need to be considered when specifying sheet products. If there are several locations or applications requiring differing types or grades of sheet, tabulate or complete a separate schedule for each, so that a notation on a drawing would call up the appropriate type of sheet.

### Joinery materials and components

#### Joinery timber

- All joinery timber shall be to approval of the Employers representative.

Separate grades are given for pinus and non-pinus species of clear (FCL), select (FSEL), standard (FSTD) and utility (FUTL). An additional furniture appearance grade (FAP) is specified for the pinus species.

#### Plywood

All plywood shall be to approval of the Employers representative.

E.g. Decorative, select and general purpose.

AS/NZS 2270 grades plywood by the quality of the face and back veneers as set out in Table 1.1

#### Decorative overlays

Timber veneer or laminate to approval of the Employers representative.

Thickness (minimum):

- For horizontal surfaces fixed to a continuous background: 1.2 mm minimum.

- For vertical surfaces fixed to a continuous background: 0.8 mm.

- For edge strips: 0.8 mm.

#### Stone facings

Provide stone slabs to benchtops within the visual range of the approved samples.

#### Timber veneers

Provide veneers falling within the visual range of the approved samples.

Cross refer to selections in Glazing.

### JOINERY ITEMS

If joinery work is fully detailed you could cross refer to the drawings and add the following notes as an alternative to the materials and components listed separately below for typical joinery items or assemblies.

#### General

Provide materials noted on drawings as follows:

- Joinery components and their location, indicative construction details, trims, materials, dimensions and thicknesses, and finishes shall be as detailed.

- All dimensions noted on drawings shall be confirmed on site before construction of the joinery.

- Finishes selections and hardware are noted in the Joinery fixtures schedule.

Delete if joinery is not fully detailed.

### kitchen assemblies

The selection of medium density fibreboard or particleboard can in most cases be left to the fabricator. Vary the specification if you have a preference. Colour availability is more restricted in particleboard substrates. High moisture resistant particleboard is the default. Its use is essential in all domestic kitchen locations. Vary if a standard particleboard substrate is acceptable. Only the high moisture resistant grade of medium density fibreboard is available in the melamine overlaid products.

See Commentary to Workstations in the Miscellaneous furniture worksection.

#### Plinths

Built-in cupboards, particularly in domestic kitchens, usually consist of a plinth, a carcass and a top. The plinth is scribed to the floor to provide a level base for the carcasses. Usually the plinth forms the toe recess. Given the needs of on site working, moisture resistance and durability, plywood is probably the best general substrate. Other materials may be required e.g. to match timber or tile skirtings.

Material: Construct from exterior grade general purpose plywood unless already in place as a concrete plinth.

Thickness: 16 mm.

Fabrication: Form up with front and back members and full height cross members at not more than 900 mm centres.

Finish: Decorative laminated sheet or ceramic/ stone tile finish.

Installation: Fix to floor and secure to wall to provide level platform for carcasses.

#### Carcasses

Material: Select from the following:

- Melamine overlaid high moisture resistant particleboard.

- Approved solid timber sections.

Thickness: 16 mm minimum.

16 mm board is generally adequate for most construction. Vary the thickness if appropriate.

Joints: Select from the following:

- Proprietary mechanical connections.

- Screws and glue.

Shelves: Support on battens or fix directly into grooves in side walls of joinery units.

A variety of hole size and drilling centres are in use, vary as required. For bookshelves a maximum span of 600 mm is recommended.

Finish: Decorative laminated sheet or solid timber finish.

A common practice is to use a white melamine overlaid board for construction generally and to laminate gable ends, exposed backs and insides to the selected colour. This eliminates the need for secret fixings and gives the widest choice of colour. Alternatively, secret fixings can be used and melamine overlaid board used throughout except for bench tops. Heavy duty melamine overlaid boards are available which are claimed to match the performance of laminated surfaces. Specify by proprietary item if required.

Fasteners: Conceal with finish.

Installation: Secure to walls at not more than 600 mm centres.

#### Drawer fronts and doors

Material: Refer to the drawings for specific details of joinery or select from the following:

- Melamine overlaid high moisture resistant particleboard.

- Approved solid timber sections with or without inset glass panels.

- Metal grille or sheet metal panels fixed over timber frames

Thickness: 16 mm minimum.

Maximum door size: 2400 mm high, 900 mm wide, 1.5 m2 on face.

Finish: Decorative laminated sheet, solid timber finish or paint.

#### Drawer backs, sides and bottoms

A variety of proprietary systems as well as traditional construction is available; the default here gives good construction and finish.

Material: Select from the following:

- Melamine overlaid high moisture resistant particleboard.

- Approved solid timber sections.

i.e. PVC on both faces and all edges.

Thickness: 12 mm minimum.

Finish: Decorative laminated sheet or solid timber finish.

#### Laminated benchtops

If you give a layout of joints, bear in mind the practicalities of transport and on site handling. Edit Submissions accordingly.

Material: High moisture resistant particleboard.

Benchtop thickness: 33 mm.

Finish: Decorative laminated sheet.

Exposed edges: Extend laminate over shaped nosing, finishing > 50 mm back on underside or provide solid timber edge trim.

Installation: Fix to carcass at least twice per 600 mm length of benchtop.

Joint sealing: Clamp with proprietary mechanical connectors to ensure high quality connection between benchtop sections. Ensure joints in benchtops are clear of sinks to avoid water damage to joint.

#### Stone or concrete benchtops

Material:

Natural stone or Re-constituted stone

- Thickness is to be minimum of 40mm unless noted otherwise on the drawings.

- Concrete benchtops may have a polished finish or be covered with ceramic tiles.

#### Splashback:

- Material is identical to benchtop unless noted otherwise in the Joinery fixtures schedule.

- Thickness is to be 16mm for high moisture resistant particleboard with laminate finish.

- Thickness is to be 20mm minimum for stone.

- Thickness is to be 40mm minimum for concrete. Alternatively use ceramic tile splashback for concrete benchtops.

- Waterproof silicone sealant is to be used as a continuous seal between the benchtop and splashback.

Nominate or refer to detail:

- Edge treatment.

- Joints.

- Junctions with other materials.

Solid plastic benchtops: Import material as required from Stone benchtops.

#### Drawer and door hardware

Specify by proprietary item and schedule if required. Refer to the Door and window hardware worksection which covers items such as hinges, keys, locks, latches, furniture and door controllers. Also consider locks and keys, if required.

Hinges, drawer runners, door handles and locks are to be to the approval of the Employers representative.

There is a wide range of proprietary hardware available and those that may need to be considered include:

- Knobs and handles (pulls).

- Stays and flap hinges.

- Catches (particularly child-proof safety catches - recommended where drugs and chemicals are stored).

- Locks.

- Shelf supports and revolving units.

- Wardrobe and hanging accessories.

- Wireware.

- Extension and swivel mechanisms.

### Timber balustrades

Provide materials for the approval of the Employers representative before installation. Ensure all dimensions are checked on site before construction starts. Refer to BOQ and drawings for extent of work.

## Execution

### Joinery

#### General

Joints: Provide materials in single lengths whenever possible. If joints are necessary make them over supports.

Framing: Frame and trim where necessary for openings, including those required by other trades.

Provide openings for the following e.g. equipment: If scheduling required openings be sure to be comprehensive, or leave this trade coordination matter to the contractor.

#### Accessories and trim

Provide accessories and trim necessary to complete the installation.

#### Fasteners

The range of fasteners available is so wide that it is not recommended that specifiers nominate types unless they have a particular requirement in mind. Some fasteners have the advantage of a knock-down capability and this may be a consideration. Stapling and nailing are excluded but may be acceptable in some classes of work. If sufficient for your purposes, say so.

Visibility: Do not provide visible fixings except in the following locations:

- Inside cupboards and drawer units.

- Inside open units.

Visible fixings: Where fastenings are unavoidable on visible joinery faces, sink the heads below the surface and fill the sinking flush with a material compatible with the surface finish. In surfaces which are to have clear or tinted timber finish provide matching wood plugs showing face (not end) grain. In surfaces which are to have laminate finish provide proprietary screws and caps finished to match.

Fixings: Screws with washers into timber or steel framing, or masonry anchors to brickwork.

#### Adhesives

As in the case of fasteners, the choice of these is best left to the fabricator who should rely on the adhesive manufacturer’s recommendations.

Provide adhesives to transmit the loads imposed and to ensure the rigidity of the assembly, without causing discolouration of finished surfaces.

#### Finishing

Edge strips: Finish exposed edges of sheets with edge strips which match sheet faces or use solid timber trims as noted on the drawings.

Specify or detail any special edge strip requirements e.g. solid timber throughout, or to bench tops only. Nominate if veneer or plastic.

Matching: For surfaces which are to have clear or tinted finish, arrange adjacent timber pieces to match the grain and colour.

Hygiene requirements: To all food handling areas and voids at the backs of units to all areas, seal all carcass junctions with walls and floors, and to cable entries, with silicone sealant for vermin proofing. Apply water resistant sealants around all plumbing fixtures and ensure the sealants are fit for purpose.

### Delivery and storage

#### General

Deliver joinery units to site in unbroken wrapping or containers so that its moisture content is not adversely affected. Do not store in areas of wet plaster. Keep storage to a minimum by delivering items only when required for installation.

Examine joinery units for completeness and repair defects before installing in place.

#### Background

Clean all background surfaces that will be permanently concealed behind joinery before installing in place.

### Timber balustrades

Determine design loads for balustrades in accordance with AS/NZS 1170.1 and AS/NZS 1170.2.

#### General

Provide a balustrade to the stair and landing, consisting of posts, handrail, infill panels, and associated mouldings as noted in the BOQ and drawings.

### Completion

#### Cleaning

Temporary coatings: On or before completion of the works, or before joining up to other surfaces, remove all traces of temporary coatings used as a means of protection.

General: Remove all dust, marks and rubbish from all surfaces and internal spaces. Clean and polish all surfaces such as solid timber, anodised or painted metals, glass, stone, concrete, ceramic tiles and laminates.

Refer to the Joinery fixtures schedule for locations, type and finishes of joinery items.