# Windows AND WINDOW HARDWARE

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to windows and/or sliding glazed doors consisting of proprietary lines (or “suites”) of manufactured products, supplied as complete systems fabricated and assembled by specialist firms to their standard designs, in specified materials such as metal, timber and plastics, and including the following:

- Glazing, hardware and associated screens, shutters, integral blinds, louvres, grilles.

- Installation accessories, such as fixings, flashings, sealants, caulking and weather-stripping, necessary for the satisfactory functioning of the whole.

Related material located elsewhere in NATSPEC

There are no directly related NATSPEC worksections.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

- Window hardware.

- Glazing.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

Worksection cross-referencing

Other worksections that reference this worksection are:

- None.

Documenting this and related work

You should document related work as follows:

- Schedule windows on drawings to your office documentation policy.

## General

To use AS 2047 the building class needs to be nominated as follows:

- Housing: BCA Class 1 and 10.

- Residential: BCA Class 2, 3 and 4.

- Commercial: BCA Class 5, 6, 7, 8 and 9.

- See Commentary for a discussion on the Australian Window Association (AWA), AS 2047 and the BCA requirements.

### Interpretation

#### Definitions

For the purposes of this worksection windows also includes louvres, either vertical or horizontal, set into frames.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following:

- Openings prepared to receive windows (where windows are to be installed in prepared openings).

- Fabricated window assemblies delivered to the site, before installation.

- Commencement of window installation.

Hold points, if required, should be inserted here.

## Products

### Louvre assemblies

Coordinate with the Ductwork worksection, if applicable.

#### General

Provide louvre blades mounted in a surround frame and able to withstand the wind pressure for that location without failure or permanent distortion of blades, and without blade flutter.

#### Adjustable louvres

Provide louvre blades clipped into holders which pivot, linked together in banks, each bank operated by an operating handle incorporating a latching device, or by a locking bar.

#### Screens

Provide metallic coated steel wire mesh screens behind louvres to prevent the entry of birds, rodents and wind blown leaves and papers.

AS/NZS 3666.1 clause 2.2.1 makes the provision of screens behind air intake louvres.

### Insect screens

#### Aluminium framed screens

Provide insect screens with mesh frame channel. Provide an extended frame section where necessary to adapt to window opening gear.

- Mesh: Fix the mesh into the frame channel with a continuous resilient gasket, so that the mesh is taut and without distortion.

#### Fixed screens

Provide fixed screens to the window frames with a clipping device which permits removal for cleaning.

#### Hinged screens

Hinge at the side to give access to opening sash.

#### Sliding screens

Provide a matching aluminium head guide, sill runner, and frame stile sections for screens not part of the window frame.

- Hardware: Nylon slide runners and finger pull handle.

### Security window grilles

#### General

Provide security grilles in accordance with the drawings or proprietary metal security grille screens, fixed to the building structure with tamper resistant fastenings.

### Window Hardware

#### Hardware

Provide hardware of sufficient strength and quality to perform its function, appropriate to the intended conditions of use and climate and fabricated with fixed parts firmly joined.

## Execution

### INSTALLATION

#### General

Install windows so that the frames:

- Are plumb, level and straight within acceptable building tolerances.

- Are fixed or anchored to the building structure to resist the wind loading.

- Will not carry any building loads, including loads caused by structural deflection.

- Allow for thermal movement.

#### Flashing and weathering

Install moulds, sealant and cement pointing as required so that water is prevented from penetrating the building between the window frame and the building structure.

#### Fixing and fasteners

Materials: Use materials compatible with the item being fixed and of sufficient strength, size and quality to perform their function.

- Concealed fixings: Provide a corrosion resistant finish.

- Exposed fixings: Match exposed fixings to the material being fixed.

Support: Provide appropriate back support (for example blocking and backing plates) for hardware fixings.

Window fastener spacing (nominal): 600 mm.

Window fasteners: Conceal fasteners where possible.

Packing: Pack behind fixing points with durable full width packing.

Prepared masonry openings: If fixing timber windows into existing prepared openings with fastenings through the frame face, make the fastener heads finish below the surface and fill the hole for a smooth surface finish.

#### Joints

Make accurately fitted tight joints so that neither fasteners nor fixing devices such as pins, screws, adhesives and pressure indentations are visible on exposed surfaces.

#### Operation

Ensure moving parts operate freely and smoothly, without binding or sticking and are lubricated.

#### Supply

Deliver window hardware items, ready for installation, in individual complete sets for each window.

- In a separate dust and moisture proof package labelled for the specific window.

- Including the necessary templates, fixings and fixing instructions.

- Refer to the drawings and Window, Louvre and Security grille/shutter schedules for details of windows. Refer to the Window hardware schedule for details of window hardware.

State a particular method here, or delete to leave the choice of method to the contractor.

Coordinate with the Air grilles worksection, if applicable.

### Completion

#### Cleaning

The Contractor is to clean all frames, glass, hardware at completion. Any damage to frames, or broken glass is to be repaired or replaced to the satisfaction of the Employers representative.

#### Adjustment

Leave the hardware properly adjusted with working parts in working order and lubricated where appropriate.

Security screen code: If there are several different types of security screen grille in the project, give each an SS code designation (mark) by which it can be referred to in the Security window grille schedule.

Type: Hinged to Type I or II or III. Sliding to Type I, II or III. AS 5039 describes the three types at clause 5.2.

- Type I prevents an arm from passing through.

- Type II allows an arm but prevents bodily entry.

- Type III prevents insects passing through.

Material: Steel or aluminium.

Finish: See AS 5039 clause 6.2 for corrosion protection finishes.

Hinges:

- Material: e.g. Aluminium or steel.

- Fixing: Rivets or fastening devices. See AS 5039 clauses 6.7, and 6.8.

Lock: See AS 5039 clause 6.5. If the manufacturer’s standard lock and hardware are not acceptable, specify non-standard hardware using the titles of items specified in detail in the Window hardware worksection, or by the designations (trade names, etc.) of proprietary products.