# Earthwork

Worksection application

This worksection is applicable to topsoil removal, excavation, filling and compaction.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Demolition.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

- Site preparation.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection. For example:

- Gabions and retaining structures.

- Worksection cross referencing

- Other worksections that cross reference this worksection are:

- Pavement base and subbase.

- Documenting this and related work

- You may document this and related work as follows:

- Show earthwork levels, extent and slope of permanent/temporary batters and location of temporary/permanent retaining structures required as part of the earthworks on the drawings.

- Approximate locations of unsuitable materials/contaminated materials identified in the geotechnical report should be shown on the drawings.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

### Interpretation

#### Definitions

For the purposes of this worksection the definitions given below apply.

- Bad ground: Ground unsuitable for the purposes of the works, including fill liable to subsidence, ground containing cavities, faults or fissures, ground contaminated by harmful substances and ground which is or becomes soft, wet or unstable.

- Line of influence: A line extending downward and outward from the bottom edge of a footing, slab or pavement and defining the extent of foundation material having influence on the stability or support of the footings, slab or pavement.

See AS 3798 clause 1.3.11, where “pavement” has a wider meaning.

Refer also to Records of measurement, below.

The Definitions subclause can be edited to suit the project or deleted if not required.

Include the report in the tender documents, or inform tenderers where the report is available. If no geotechnical and environmental site investigation has been carried out, delete the clause. See AS 1726. See also “Guidelines for the provision of geotechnical information in construction contracts” published by the Institution of Engineers, Australia. On selective provision of available data, see AS 3798 clause 3.3(b).

For potentially contaminated soil see ANZECC/NRMMC, the guide standards AS 4874, AS 4482.1 and AS 4482.2 and BDP EDG GEN32. On contaminants, see AS 3798 clauses 2(x) and 4.2(b), AS 4482 Parts 1 and 2 and ANZECC/NRMMC.

AS 4964 provides laboratories with a method of qualitative identification of amosite, crocidolite and chrysotile asbestos in bulk building materials and soils by polarised light microscopy and dispersion staining techniques.

### Records of measurement

Notice that the items are ready for measurement is required to be given in Inspection. Once notice has been given the onus is on the contract administrator to act so as to avoid delay, either by completing the measurement or by giving a direction to proceed.

You may care to state that the contractor is not entitled to contract variation or extension of time for excavation in excess of that required by the contract, including excavation below required depths, or additional excavation which the contractor may elect to undertake to permit the use of certain constructional plant, and any consequent additional backfilling, compacting or testing.

#### Excavation and backfilling

Do not commence backfilling or place permanent works in the excavation until the following have been agreed and recorded:

- Depths of excavations related to the datum.

- Final plan dimensions of excavations.

Method of measurement: To be jointly agreed between the Employers representative and Contractors Site Manager unless otherwise agreed.

If excavation or rock excavation is to be measured for payment purposes ensure that the method of measurement is specified, and that the conditions for measurement and payment are defined in the contract. Method of measurement may be deleted here if covered in the conditions of contract or preliminaries.

If rock is not to be paid for at a separate rate, delete or substitute a subclause such as the following:

“Rock excavation

Excavate the ground as found. No additional payment will be made for rock excavation.”

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following:

- Excavation completed to contract levels or founding material.

- Filling completed to contract levels.

Add Hold points, if required.

Consult the geotechnical and environmental engineer in relation to geotechnical and environmental tests required and the frequency of these test and outline here. Delete where imported fill is not required.

### Tolerances

#### Tolerances

Finish the surface to the required level, grade and shape within the following tolerances:

- Under building slabs and loadbearing elements: + 0, -40 mm.

- Pavement subgrades; + 0, - 60 mm.

- Other ground surfaces:  50 mm, provided the area will drain and matches adjacent construction where required.

AS 3798 clause 3.3(a)(v) refers to “standard of surface trim”. State any other tolerances on levels or dimensions. Particular tolerances may be required for work prepared for separate or future contracts.

## Products

### Fill materials

See AS 3798 Section 4.

#### Fill material generally

Fill material is to be inorganic, non-perishable material.

On terminology, see AS 3798 clause 3.3(a)(iv).

Excluded materials:

- Organic soils.

- Materials contaminated through past site usage.

- Silts or silt-like materials.

- Fill containing wood, metal, plastic, boulders or other deleterious material.

Classifications for structural fill are based on the intended use of the fill, and defined as follows:

Class I structural fill - used as support for shallow foundations, paved areas, and slabs each with loadings of 3660 kgf/m2 or more, for storage tanks, truck turnarounds, and base course and sub-base course for roadway pavements.

Class II structural fill - used as support for shallow foundations, paved areas, and slabs each with loadings of less than 3660 kgf/m2 and for parking areas, backfill around foundations, for the construction of embankments, and for roadways pavement subgrades.

Class III non-structural fill - used in areas where installation of structures or equipment is not planned and in open areas where grading is only required to reach levels noted on the drawings.

#### Re-use of material recovered from excavation

Re-use excavated material elsewhere on site if approved by the Employers representative.

“Permitted” or “Not permitted” based on advice from the geotechnical and environmental engineer. Excavated material may not be suitable for reuse where it is contaminated or possesses problematic geotechnical properties.

Where use of site excavated material is permitted, describe here any areas of contamination or unsuitable material which are excluded from reuse or make reference to the drawings. Delete if not applicable.

Consult the geotechnical and environmental engineer for specific requirements applicable to the site. Delete where no importation of fill is required.

## Execution

### Removal of topsoil

#### General

From AS 3798 clause 6.1.5.

Remove topsoil to all areas to be cut, areas to be filled, areas to be occupied by structures, pavements, embankments and the like.

Maximum depth: 100 mm.

The depth specified should be varied to suit the recommendations of the geotechnical report.

#### Re-use of removed topsoil

Re-use removed topsoil elsewhere on site as directed by the Employers representative.

“Permitted’ or “Not permitted” based on available area for respreading and advice from environmental engineer in relation to contamination.

Delete this subclause where all topsoil is being reused.

### Excavation

Ensure that the drawings give the information necessary to enable the contractor to perform the groundworks accurately. Show the required finished surface levels of slabs, paving, filled or landscaped areas and the like, and show or specify the thickness of the materials superimposed on the subgrade surface - slabs, slab bases, topsoil layers, and so on. It is the contractor’s responsibility to make allowance for these, and for compaction or settlement, when determining the site surface levels and profiles. See AS 3798 clauses 6.1.6 and 6.1.7.

#### Extent

Excavate over the site to give correct levels for construction, pavements, filling and landscaping.

Arrange levels and grading to prevent surface water flowing into or under habitable areas during storms.

Excavate for footings, pits and shafts, to the required sizes and depths. Confirm that bearing capacity is adequate.

Or 600 mm. May be specified here or shown on the drawings. Local regulations may apply.

#### Proof rolling

Proof roll excavations for pavements, filling and non-spanning slabs on ground to determine the extent of any bad ground.

#### Disposal of excess excavated material

Remove excess excavated material from the site and dispose of legally.

Include this subclause where volume of excavated material exceeds the fill required for the site. Where the fill is contaminated consult the environmental engineer for advice in relation to classification of fill and disposal requirements.

### Bearing surfaces

#### General

Provide flat bearing surfaces for loadbearing elements including footings. Step to suit changes in levels. Make the steps to the appropriate courses if supporting masonry.

Show steps in footings on the drawings.

### Reinstatement of excavation

#### General

Where excavation is deeper than the required depth, fill and consolidate to the correct depth.

### Supporting excavations

#### Provision of supports

Provide temporary supports to all excavations greater than 1.8m deep. Confirm type of supports and level of protection required with the Employers representative.

#### Removal of supports

Remove temporary supports progressively as backfilling proceeds.

### Adjacent structures

#### Temporary supports

Provide supports to adjacent structures where necessary, sufficient to prevent damage arising from the works. This applies to all structures where the line of influence is interfered with by the proposed excavation works.

Lateral supports: Provide lateral support using shoring.

Vertical supports: Provide vertical support where necessary using piling or underpinning or both.

#### Permanent supports

If permanent supports for adjacent structures are necessary and are not described, give notice and obtain instructions.

Major work, if known in advance to be necessary, should usually be designed by the structural engineer and included as part of the contract, in which case it should be shown on the drawings.

### Preparation for filling

See AS 3798 clause 6.1.

#### General

Prepare the ground surface before placing fill (including topsoil fill), ground slabs or load bearing elements. Shape to assist drainage. Compact the ground exposed after stripping or excavation.

### Placing fill

See AS 3798 clause 6.2.2.

AS 2870 and AS 3798 provide guidance on construction requirements and the specification of earthworks generally, including filling and compaction. AS 2870 is cited in the BCA at clause F1.10, for damp-proofing under slabs on ground. AS 3798 clause 5.1 recommends placing material directly from cut to fill, to minimise problems with moisture content.

#### General

Layers: Place fill in maximum 15cm horizontal layers across the fill area.

Mix: Place fill in a uniform mixture.

Protection: Protect the works from damage due to compaction operations. Where necessary, limit the size of compaction equipment or compact by hand. Commence compacting each layer at the structure and proceed away from it.

Partially completed structures are vulnerable to damage during compaction.

Their protection is the contractor’s responsibility, but consider whether special precautions need to be taken.

Protective boarding for membranes is specified in the Waterproofing worksection.

### Compaction requirements FOR FILL AND SUBGRADE

See AS 3798 Section 5 for guidelines on compaction criteria for commercial and residential developments.

Testing to AS 1289 generally does not apply to use of ripped rock or coarse material as fill i.e. soils with more than 20% by mass of particles coarser than 37.5 mm. See AS 3798 clauses 5.3 and 5.4. Where use of coarse fill material is proposed seek geotechnical advice.

#### Density

Compact the subgrade exposed by excavation to a minimum depth of 15cm. Compact each layer of fill to the required depth and density, as a systematic construction operation. Shape surfaces to provide drainage and prevent ponding.

Density of all layers of filling are to be approved by the Employers representative before subsequent layers are placed.

From AS 3798 Table 5.1 Note 7.

Maximum rock and lump size in layer after compaction: 2/3 compacted layer thickness.

AS 3798 clause 6.2.2.

#### Moisture content

Adjust the moisture content of fill during compaction in order to achieve the required density. Do not allow subgrade or fill layers to dry out after compaction before placing subsequent layers of fill. Do not over water filling to greater than moisture content of adjoining undisturbed ground.

This worksection requires adjustment of the moisture content of filling at time of compaction to 85 – 115% of optimum. If different or more particular requirements apply specify them here. If specifying moisture content in Compaction schedule delete this subclause.