# Metalwork

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to what is generally known as architectural metalwork such as handrails, access ways, screens, guards, furniture, flagpoles, ladders, sunscreens, and decorative items.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Structural steel for structural steel items.

- Roofing for rainwater goods.

- Glazing for toughened glass component of balustrades.

- Stainless steel benching for stainless steel benching and associated fitments and bench/sink units that form part of custom made metal fixtures.

Worksection(s) on which this worksection relies

The following worksections must be included, if this worksection is used, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by this worksection. You may need to create new text, modify this text or use an applicable worksection. Examples include:

- Pure zinc items, polished or weathered finish.

Worksection cross-referencing

Other worksections that cross reference this worksection are:

- None.

Documenting this and related work

You may document this and related work as follows:

- Import applicable material from this worksection to Applied wall finishes and Joinery for metal trims and fixings.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Structural steel.

- Glazing.

- Stainless steel benching.

- Applied wall finishes.

- Painting.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following:

- Shop fabricated or assembled items ready for delivery to the site.

- Site erected assemblies on completion of erection.

Hold points, if required, should be inserted here.

### Submissions

#### Samples

Submit samples to the Sample table for approval by the Employers representative.

Sample Table

| Description | No. of samples |
| --- | --- |
| Each type of metal item to be purchased | 2 |
| Typical joints of welded or fabricated items | 2 |
| Finished sample of each type of painted or anodised metalwork indicating range within colour specified and finish | 2 |
| The finish to all stainless steel items | 2 |

If delivery involves a goods lift nominate the car size and capacity.

These shop drawing requirements assume the installation is fully detailed. Edit as required.

Nominate the item or component.

See AS/NZS 1554.6 clause 4.6 on the method of qualification of welding procedure by testing.

Manufacturer’s data: Submit manufacturer’s published product data and details for purchased items.

Stainless steel: For each batch of stainless steel supplied to the works, submit the certificate of compliance specified for the applicable standard.

## Products

### Materials and components

Component member sizes including wall and sheet thicknesses should be shown on the drawings. Otherwise a series of component schedules, on a project specific basis, will need to be prepared.

#### Metals

Performance: Provide metals suited to their required function, finish and method of fabrication, in sections of strength and stiffness adequate for their purpose.

#### Rivets

Use blind rivets where available in the required metal.

A blind rivet is one which can be used where the back or inside of the material being riveted is not accessible. It is not necessarily a sealed rivet; the method of forming can leave a hole through the centre of the rivet. Sealed type blind rivets are generally available only in aluminium and copper. Stainless steel and monel metal rivets must be sealed with a silicone sealant inserted in the hole and finished flush.

#### Masonry anchors

Proprietary types comprising screws or bolts in self-expanding sockets.

#### Masonry plugs

Screws in purpose-made resilient plastic sockets or fixed to timber plugs built into the wall surface.

Many specialist decorative finishes exist:

- Specialist grinding and polishing: e.g. random ground, hairline finish, tartan and engine turned.

- Patterned and bead blasted.

- Acid etched on a silk screened pattern.

## Execution

### Construction generally

The general provisions of the template may need to be supplemented by specific requirements for particular installations. Table 3.2 of AS 1562.1 is a guide to compatibility of metals. AS 1562.1 is cited in the BCA, but not in connection with this matter.

#### Metals

Provide metals so that they transmit the loads imposed and ensure the rigidity of the assembly without causing deflection or distortion of finished surfaces.

#### Fasteners

Materials: Provide fasteners in materials of mechanical strength and corrosion resistance at least equal to that of the lowest resistant metal joined.

To copper and copper alloys: Provide copper or copper-alloy fixing devices only.

To aluminium and aluminium alloys: Provide aluminium alloy or stainless steel fixing devices only.

To stainless steel: Provide appropriate stainless steel materials only.

#### Fabrication

Workshop: Fabricate and pre-assemble items in the workshop wherever practicable.

Edges and surfaces: Keep clean, neat and free from burrs and indentations. Remove sharp edges without excessive radiusing.

Tube bends: Form bends in tube without visibly deforming the cross section.

Colour finished work: Match colours of sheets, extrusions and heads of fasteners.

Thermal movement: Accommodate thermal movement in joints and fastenings.

#### Fabrication tolerances

Structural work generally:  2 mm from design dimensions.

From AS 4100 clause 14.4.1.

#### Joints

Fit joints to an accuracy appropriate to the class of work. Finish visible joints made by welding, brazing or soldering using grinding, buffing or other methods appropriate to the class of work, before further treatment.

Self-finished metals: Free of surface colour variations, after jointing.

Joints: Fit accurately to a hairline where feasible.

#### Marking

Provide suitable and sufficient marks or other means for identifying each member of site-erected assemblies, and for their correct setting out, location, erection and connection.

#### Splicing

Provide structural members in single lengths where possible. Obtain approval of the Employers representative for locations of joints where splices in metalwork cannot be avoided.

### Welding and brazing

#### General

Quality: Provide finished welds which are free of surface and internal cracks, slag inclusion, and porosity.

If the minimum quality level specified here is not acceptable, show the required level on the drawings.

#### Brazing

General: Ensure brazed joints have sufficient lap to provide a mechanically sound joint. Do not use butt joints relying on the filler metal fillet only.

### Stainless steel fabrication

For general advice, refer to the current edition of “Australian Stainless Reference Manual”, produced by the Australian Stainless Steel Development Association (ASSDA). Refer also to the Stainless steel benching worksection, for commercial kitchen benching. See also WTIA Technical Note TN 16.

#### Welding stainless steel

All tube, angle or thick plate material is to be welded unless noted otherwise on the drawings. Ensure that welds do not discolour the final surface finish in the welding process.

#### Riveting

Riveting may be used only to join stainless steel sheet or strip less than 1 mm thick. Drill (not punch) the rivet hole, and drive the rivet cold. On completion, clean and passivate the riveted assembly.

#### Soldering

Do not solder stainless steel.

### Metal fixtures

If metalwork is fully detailed you could cross refer to the drawings and add the following notes as an alternative to the materials and components listed separately below for typical metal fixtures or assemblies.

#### General

Provide metal fixtures where noted on drawings and in the Metal fixtures schedule as follows:

- Components such as toilet roll holders, towel rails, soap dishes and their location, indicative construction details, trims, materials, dimensions and thicknesses, and finishes shall be as detailed or described in the schedule.

- All dimensions noted on drawings shall be confirmed on site.

Delete if metalwork is not fully detailed.

### Pipe handrails, stairs, ladders and balustrades

#### Assembly

The following prompt may suit a simple access stair. Vary and/or repeat the clause as required for other applications.

Materials, design and construction: Comply with the recommendations of AS 1657.

Material: Refer  to drawings and BOQ for details of member sizes and assembly of components.

Finish: Nominate paint or unfinished galvanizing.

#### Fabrication

Method: Welding.

Joints: Produce smooth unbroken surfaces at joints. Make end-to-end joints over an internal sleeve.

Bends: Make changes of direction in rails by evenly curved pipe bends.

Free ends: Seal the free ends of pipes with fabricated or purpose-made end caps.

#### Fixing to structure

Provide fabricated predrilled or purpose-made brackets or post bases, and attach the pipework to the building structure with fixings, including bolts into masonry anchors, and coach screws or bolts into timber, of metal compatible with the pipework.

#### Galvanizing

If possible, complete fabrication before galvanizing; otherwise apply a zinc-rich primer to affected joint surfaces.

#### Painting

If possible, complete fabrication before painting; otherwise apply paint to affected joint surfaces after fixing on site. Make good all damaged painted surfaces before completion of the building works. Paint finish in accordance with the Exterior and Interior painting schedules.

Location: If not shown on the drawings.

Diameter: For helical stairs, if not shown on the drawings.

Ladder cage: and Safety device: These are alternatives. The latter applies to rung type ladders only. Delete either or both if not required.

### Corner guards and vehicle guards

#### Corner Guards

Where corners of the structure are required to be protected from mechanical damage, provide metal corner guards as follows and as identified on the drawings or in the BOQ:

- Consisting of angle sections or sections fabricated from metal sheet bent to the radius or angle of the corner.

- Fitting close to adjoining surface finishes.

- Solidly grouted up at the back to eliminate voids.

- Securely fixed by a method which does not cause distortion in the guard surface, and consists of either concealed built in lugs, or flush countersunk head fixings into masonry anchors.

- Paint finish in accordance with the Exterior and Interior painting schedules.

#### Vehicle Guards

Where external features such as lamp posts, fire hose reels or pedestrian walkways are required to be protected from vehicle damage, provide metal guards as follows and as identified on the drawings and in the BOQ:

- Consisting of steel pipe posts set in deep concrete pads with welded end caps or bent to form a rail and two posts.

- Steel barrier rails securely bolted to the posts.

- Heavy duty protection posts will be large diameter steel pipe posts filled with concrete.

- Paint finish in accordance with the Exterior and Interior painting schedules.

### Water storage tanks and stands

#### Water Tanks

Fabricate metal water storage tanks to sizes shown on drawings and as identified in the BOQ. Allow for all reinforcement of tank walls, floors, and around fixtures projecting from the tank.

Bolt together prefabricated plastic or metal water storage tanks to sizes shown on drawings and as identified in the BOQ.

- Fabricate metal tank stands for the water storage tanks as identified on the drawings and in the BOQ.

- Refer to the Metal fixtures schedule for details.

- Paint finish in accordance with the Exterior and Interior painting schedules.

### Completion

#### Maintenance manual

General: Submit manufacturer’s published recommendations for service use.

#### Cleaning

Temporary coatings: On or before completion of the works, or before joining up to other surfaces, remove all traces of temporary coatings used as a means of protection.