# Landscape – soils AND PLANTING

Worksection application

This worksection is applicable to soils for landscaping including, rock work, filling, topsoil, topsoil additives, compost and mulch. Geotechnical site investigation and soil related earthworks are also covered.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. See for example:

- Site preparation.

- Earthwork for excavation and filling.

- Landscape – walling for walls under 750 mm in height.

- Landscape – fences and barriers for lightweight fences and barriers.

- Landscape – soft surfaces for grassed surfaces.

- Landscape – plants for plants and planting.

- Trees supply for the supply of new trees.

- Irrigation for irrigation.

- Landscape – furniture and fixtures for installed equipment associated with landscaped works.

- Segmental pavers – mortar bed for external stone, ceramic and cementitious paving.

- Segmental pavers – sand bed for external unit paving.

- Concrete formwork, Concrete reinforcing and Concrete in situ for footings and other concrete structures.

- Brick and block construction for masonry walls.

- Tensioned membrane structures for shade structures.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

- Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

Worksection cross-referencing

Other worksections that cross reference this worksection are:

- None.

Documenting this and related work

You may document this and related work as follows:

- Show on the drawings the locations and depths of soils to be removed and soils to be provided.

- If the extent of soils to be removed is uncertain, consider allowing for provisional volumes here.

## General

### Submissions

#### Suppliers

The suppliers of topsoil and mulch should generally be approved.

Obtain statements from suppliers of plant materials, giving the following:

- Particulars of the supplier’s experience in the required type of work.

- Lead times for delivery of the material to the site.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following:

- lawns prepared before seeding

- plant holes excavated and prepared for planting

- setout of gravel paths prepared for filling

## Products

### Topsoil

#### Source

Import topsoil unless the topsoil type can be provided from material recovered from the site.

#### Additives

Use additives to raise topsoil to the required standard approved by the Employers Representative.

### Compost and fertiliser

#### Compost

Provide well rotted vegetative material or animal manure, free from harmful chemicals, grass and weed growth.

Type: e.g. spent mushroom compost, if available. Mushroom compost may be alkaline and not generally recommended for use with native plants in the eastern states. Consider the supply conditions.

Application rate: e.g. 4:1 topsoil:compost, by volume.

#### Fertiliser

Provide proprietary fertilisers, delivered to the site in sealed bags marked to show manufacturer or vendor, weight, fertiliser type, recommended uses and application rates.

### gRAVEL PATHS

Provide paths constructed with consolidated small gravel chippings and concrete edging pavers where shown on plans.

N:P:K ratios and application rates vary greatly depending on conditions of use. Obtain specialist advice. Proprietary fertilizers meeting the requirements may be named if desired. Consult manufacturers for application rates suitable for individual plantings. If nothing is stated in the schedule, the General requirements worksection requires the contractor to follow the manufacturers’ recommendations.

## Execution

### Preparation

#### Vegetative spoil

Remove vegetative spoil from site. Do not burn.

Burning destroys soil bacteria. Avoid if possible. Check with local authority regarding burning before permitting it. If vegetative spoil is not to be chipped and reused it should be removed from the site.

### Rockwork

#### Rock work

General: Place rocks while ground formation work is being carried out. Provide site rock, otherwise provide imported rock. Bury rock two thirds by volume, with weathered faces exposed. Protect the weathered faces from damage.

Site rock: Stockpile for future placement and accessibility for lifting. Dispose of other rock off site.

Imported rock: Provide rock which has been selected before delivery.

Storage and handling procedures may need special consideration, particularly for very large rocks and rocks from distant sources.

Source: e.g. quarry or stone supplier (proprietary item).

Rock type and description: May be shown on the drawings. Show or specify the size, especially if the item is unusually large. Quarried rock is unweathered and generally cheaper and more readily available than weathered rock.

### Subsoil

#### Ripping

Rip parallel to the final contours wherever possible. Do not rip when the subsoil is wet or plastic. Do not rip within the dripline of trees and shrubs to be retained.

Ripping depths: Rip the subsoil to the following typical depths:

- Compacted subsoil: 300 mm.

- Heavily compacted clay subsoil: 450 mm.

e.g. 450 mm. This depth may result in bringing subsoil to the surface, and requires a fairly heavy machine. Consider resolving depth on site.

#### Planting beds

Excavated: Excavate to bring the subsoil to at least 300 mm below finished design levels. Shape the subsoil to fall to subsoil drains where applicable. Break up the subsoil to a further depth of 100 mm.

Unexcavated: Remove weeds, roots, builder’s rubbish and other debris. Bring the planting bed to 75 mm below finished design levels.

#### Cultivation

Minimum depth: 100 mm.

The 100 mm minimum depth  assumes that the subsoil is of sufficiently good quality and it therefore is not necessary to excavate and to import topsoil. This depth may not be achievable on rocky or steep areas. Vary if required.

Services and roots: Do not disturb services or tree roots; if necessary cultivate these areas by hand.

Cultivation: Thoroughly mix in materials required to be incorporated into the subsoil. Cultivate manually within 300 mm of paths or structures. Remove stones exceeding 50 mm, and weeds, rubbish or other deleterious material brought to the surface during cultivation. Trim the surface to design levels after cultivation.

#### Additives

Apply additives after ripping or cultivation and incorporate into the upper 100 mm layer of the subsoil. Refer to the Soil additives schedule.

Gypsum may be used where the site topsoil is heavy clay, provided planting is not lime-sensitive, or where the subsoil is heavy clay, to improve drainage. Other additives include lime or dolomite to reduce acidity in highly acidic soils, and sulfur to reduce alkalinity in highly alkaline soils. Test the subsoil by chemical and physical analysis to determine appropriate remedial methods.

### Topsoil

#### Placing topsoil

Preferably show on the drawings the areas to be topsoiled and the topsoil depths. Typically the areas would be those required to be grassed, turfed, or mass planted.

Spread the topsoil on the prepared subsoil and grade evenly. Ensure that grassed areas may be finished flush with adjacent hard surfaces such as kerbs, paths and mowing strips.

Contamination: Where diesel oil, cement or other toxic material has been spilt on the subsoil or topsoil, excavate the contaminated soil, dispose of it off the site, and replace it with site soil or imported topsoil.

Finishing: Feather edges into adjoining undisturbed ground.

#### Consolidation

Compact lightly and uniformly in 150 mm layers. Produce a finished topsoil surface which has the following characteristics:

- Smooth and free from stones or lumps of soil.

- Graded evenly into adjoining ground surfaces.

- Ready for planting.

#### Topsoil depths

Vary if the typical depths specified in the Template are not acceptable.

Spread topsoil to the following typical depths:

- Excavated planting areas: If using organic mulch, 200 mm.

- Irrigated grassed areas generally: 150 mm.

- Non-irrigated grass areas: 100 mm.

Soil key, texture, use and depth may be shown on the drawings. The examples given below are by way of illustration only; vary as necessary. Obtain specialist advice.

Depth: Delete if Template defaults are acceptable.

#### Surplus topsoil

Spread surplus topsoil on designated areas on site, if any; otherwise, dispose off site.

Designated areas to be determined by the Employers Representative.

### Grass seeding

See the Commentary on grass seeding.

#### Preparation

Prepare the areas to be sown. Spread the fertiliser evenly over the cultivated bed within 48 hours before sowing, and rake lightly into the surface. If a prepared area becomes compacted from any cause before sowing can begin, rework the ground surface before sowing.

#### Sowing

Do not sow if frost is likely before the plant has reached an established state, or in periods of extreme heat, cold or wet, or when wind velocities exceed 8 km/h. Provide even distribution. Lightly rake the surface to cover the seed.

Use this schedule to identify and describe each of the grass seed mixes to be used in the project, whether for sowing, hydroseeding, temporary grassing, etc. A separate schedule for each mix designation may be necessary.

Mix designation: e.g. Type A. See the Commentary.

Seed species: and Application rates: Set out as in the typical examples. Identify and describe each of the grass seed mixes to be used in the project, whether for sowing, hydroseeding, temporary grassing, etc.

Sowing method: Specify any particular requirements additional to those in the Template, or delete this column. The method may comprise broadcast sowing in two transverse directions, or row sowing, etc., depending on the equipment used and the conditions.

Mowing height: Different for non-irrigated and irrigated grasses. 30 mm for stoloniferous grasses, 75-150 mm for species with coarse growth, such as Oats or Rye. Mowing may not be possible on steep banks and may not be desirable where seed mix includes species with coarse growth.

#### Rolling

Roll the seed bed immediately after sowing.

Roller weight (maximum):

- Clay and packing (heavy) soils: 90 kg/m width.

- Sandy and light soils: 300 kg/m width.

#### Watering

Before germination: Water the seeded area with a fine spray until the topsoil is moistened to its full depth. Continue watering until germination to keep the surface damp and the topsoil moist but not waterlogged.

After germination: Water to maintain a healthy condition, progressively hardened off to the natural climatic conditions.

#### Germination

Maintain sown areas until healthy grass covers the whole of the seeded area.

Reseeding: If germination has not been attained within one month, reseed the sown areas.

e.g. the original seed mixture or a modified mixture.

#### Weeding

Identify special requirements for areas such as steep embankments and broad scale sites where germination is to be by rainfall only.

Remove weeds that occur in sown areas. Where necessary spray with a selective weedkiller for broad leafed weeds. Do not spray grass seeded areas within 3 months of germination.

#### Protection

Protect the newly sown areas against traffic until well established. Protection method to be approved by the Employers representative.

e.g. temporary fencing.

Vary the type and timing of the post-sowing application as necessary. Sulphate of ammonia may be required more frequently in warm wet conditions. On the other hand, it does not suit clover.

#### Mowing

Mow to maintain the grass height within the required range. Do not remove more than one third of the grass height at any one time. Carry out the last mowing within 7 days before the end of the planting establishment period. Remove grass clippings from the site after each mowing.

### Plants

#### Plants

Characteristics: Provide plants with the following characteristics:

- Large healthy root systems.

- Vigorous, well established, free from disease and pests.

- Suitable for planting in the natural climatic conditions prevailing at the site.

Replacement: Replace damaged or failed plants with plants of the same type and size.

#### Plant containers

Supply plants in weed-free containers of the required size.

Open rooted stock: If trees are to be supplied as open rooted stock, ensure this is appropriate to the species, variety, size, and time of year for planting.

Potting-on is the transfer of plants into larger soil-filled containers, to prevent them from becoming root-bound. If a need for potting-on (i.e. long delays) is considered to be likely, tenderers should be required to nominate actual contract rates at the time of tender. In any case, agree the rates before authorising potting-on, if the principal/proprietor is to bear the cost.

Refer to the Plant Schedule.

Delete if the information is shown on the drawings.

Plant species: Depending on location and species, plants may be open-rooted or balled. Specify accordingly.

Roots: Depending on location and species, plants may be open-rooted or balled. Specify accordingly. Check availability of bare or open-rooted plants - not for all species, not all year round, not all sizes.

Minimum trunk caliper: For some species the trunk caliper may not relate to height due to growing characteristics of the species.

#### Labelling

Label at least one plant of each species or variety in a batch with a durable, readable tag.

#### Storage

Deliver plant material to the site on a day to day basis, and plant immediately after delivery.

### Planting

#### Individual plantings in grassed areas

Excavate a hole to twice the diameter of the root ball and at least 100 mm deeper than the root ball. Break up the base of the hole to a further depth of 100 mm, and loosen compacted sides of the hole to prevent confinement of root growth.

Developments in tubestock planting using purpose-made machinery show this to be an economically viable tool for broadscale revegetation. Specification will depend on machinery and site conditions. Backfilling with topsoil may not be necessary.

#### Locations

If it appears necessary to vary plant locations and spacings to avoid service lines, or to cover the area uniformly, or for other reasons, obtain directions from the Employers Representative.

#### Planting conditions

Do not plant in unsuitable weather conditions such as extreme heat, cold, wind or rain. In other than sandy soils, suspend excavation when the soil is wet, or during frost periods.

#### Watering

Thoroughly water the plants before planting, immediately after planting, and as required to maintain growth rates free of stress.

#### Placing

Remove the plant from the container with minimum disturbance to the root ball, ensure that the root ball is moist and place it in its final position, in the centre of the hole.

If individual locations are not nominated or required show those areas on the drawings.

#### Fertilising

In planting beds and individual plantings, place fertiliser pellets around the plants at the time of planting.

Nominate a rate if manufacturer’s recommendations are not acceptable.

#### Watering basins for plants in grass

Except in irrigated grassed areas and normally moist areas, construct a watering basin around the base of each individual plant, consisting of a raised ring of soil capable of holding at least 10 L.

### Stakes and ties

#### Stakes

Use Hardwood stakes, straight, free from knots or twists, pointed at one end.

Drive stakes into the ground at least one third of their length, avoiding damage to the root system.

Stake sizes:

- For plants  2.5 m high: Three 50 x 50 x 2400 mm stakes per plant.

- For plants 1 – 2.5 m high: Two 50 x 50 x 1800 mm stakes per plant.

#### Ties

Provide ties fixed securely to the stakes, one tie at half the height of the main stem, others as necessary to stabilise the plant.

Tie types for plants < 2.5 m high: 50 mm sack webbing stapled to the stake.

### gravel paths

#### Pavement

Use small size gravel in layers not exceeding 150mm thick to form paths where shown on drawings. Colour and type of gravel to approval of Employers representative. Retain sides of path with either:

- Precast decorative concrete paving edge strips, colour to approval of Employers Representative.

- Concrete kerbs

### Planting establishment

#### Period

If planting is a separable part it may not achieve practical completion at the same time as the rest of the works, in which case the planting establishment period commences at the date of practical completion of the planting works. The Template assumes that the planting works are not a separable part. Modify the text if this is not the case. Separable parts must also be scheduled in Preliminaries.

The planting establishment period commences at the date of substantial completion and finishes at the date of final certificate.

Specify a number of weeks, preferably equal to and concurrent with the defects liability period (but consider location, climate, season, costs, etc.). This subparagraph applies if the landscape work is a separable part.

Practical completion of the planting works includes, but is not limited to, the germination of grassed areas, establishment of turfed areas, and replacement of plants which have failed, been damaged or been stolen during the works.

#### Existing planting and grass

Where existing grass or planting is within the landscape contract area, maintain it as for the corresponding classifications of new grass or planting.

#### Recurrent works

Throughout the planting establishment period, carry out maintenance work including, watering, mowing, weeding, rubbish removal, reseeding, staking and tying, replanting, cultivating, and keeping the site neat and tidy.

Complete the schedule. Refer to “Soil texture classifications” AS 4419 Appendix I Table I1.

Refer to “Guidance for the selection and use of soils” of AS 4419 Appendix K. Generally, these soil texture classifications are considered unsuitable for general landscaping purposes, according to this Appendix.