# Landscape – wallS AND FENCES

Worksection application

This worksection is applicable to retaining and free standing walls up to 750 mm high.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. See for example:

- Site preparation.

- Earthwork for excavation and filling.

- Landscape – fences and barriers for lightweight fences and barriers.

- Landscape – soils for soils.

- Landscape – soft surfaces for grassed surfaces.

- Landscape – plants for plants and planting.

- Trees supply for the supply of new trees.

- Irrigation for irrigation.

- Landscape – furniture and fixtures for installed equipment associated with landscaped works.

- Segmental pavers – mortar bed for external stone, ceramic and cementitious paving.

- Segmental pavers – sand bed for external unit paving.

- Concrete formwork, Concrete reinforcement and Concrete in situ for footings and other concrete structures.

- Brick and block construction for masonry walls.

- Tensioned membrane structures for shade structures.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

Worksection cross-referencing

Other worksections that cross reference this worksection are:

- None.

Documenting this and related work

You may document this and related work as follows:

- This worksection relies on fully detailed landscape drawings including location and type of walls, dimensions, pattern or bond and size of units.

- Retaining walls over 750 mm high. Obtain structural engineering advice. See also the guidance given in Clay Brick and Paver Institute Technical Notes 21A and 21B.

- Import the appropriate building specification worksections for landscape structures.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes, do not repeat them here.). You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

### Inspection

#### Notice

Give sufficient notice so inspection may be made of the following:

- Setting out before commencement of construction.

- Filter fabric and subsurface drainage in place before backfilling.

Hold points, if required, should be inserted here.

## Products

### Timber

Select the appropriate hazard class. AS 1604 1 identifies 6 hazard classes (H1 – H6).Table

#### Hardwood

All hardwood in timber fences is to be without any rot, significant knots, twists, or other defects which may affect its strength and to be as per Employers Representative’s approval.

Nominate Durability class 1 unless a service life of less than 25 years is appropriate. Alternatively, using Hazard class, nominate a preservative treatment.

Preservative treatment: Provide only timbers with preservative treatment painted on the timbers surface where the timber is in the ground, or ensure that all timber is highly resistant to rot.

### Steel

#### Steel tubes and channels

All steel tubes and channels used for posts, rails, stays are to be painted or galvanised to ensure the maximum lifetime for the item without significant maintenance.

Posts are available shop painted but may be site painted.

#### Wire

Chainwire, cable wire, tie wire and barbed wire are to be galvanised or other suitable metallic finish for maximum lifetime.

Organic coatings include PVC (black or green), PE (polyethylene or polythene), plastic and powder. Barbed wire may be supplied with a metallic coating or with a plastic or a thin organic coating. See AS 2423 clause 3.3.7.

### Concrete walls

#### General

- Concrete walls and concrete foundations are to be constructed as shown on the drawings.

For Engineer designed concrete structures delete this clause and provide appropriate concrete worksections from Building Standard or Basic packages.

### stone walls

Preferably show location, wall dimensions (height, thickness), pattern or bond, stone sizes and shapes, coursing, copings, face finish, etc., on the drawings.

#### Walling stone

Weathered stone is more expensive than quarried, and generally not as readily available.

Natural stone: Stone of uniform quality, sound and free from defects liable to affect its strength, appearance or durability.

Field stone: Local weathered uncut random sized natural stones.

Quarried stone: Cut or uncut random or regular size stone.

Dimensions, volume, or both.

### Crib walls

#### General

Type: Proprietary system of interlocking precast concrete units with selected backfill placed and compacted progressively to form a retaining wall.

### Gabion walls

#### General

Type: Proprietary system of rock filled wire baskets.

### brick walls

#### General

Brick walls on stone or concrete foundations are to be constructed as shown on the drawings.

### earth block walls

#### General

Earth block walls on stone or concrete foundations are to be constructed as shown on the drawings.

### Filter fabric

Consult manufacturers for information about available fabrics, including their properties and recommended applications. See AS 3704 for a glossary of terms used for geotextiles.

#### General

Type: Polymeric fabric formed from a plastic yarn composed of at least 85% by weight of propylene, ethylene, amide or vinyledenechloride and containing stabilisers or inhibitors to make the filaments resistant to deterioration due to ultraviolet light.

#### Protection

Provide heavy duty protective covering. Store clear of the ground and out of direct sunlight. During installation do not expose the filter fabric to sunlight for more than 14 days.

## Execution

### General

#### Set out

General: Set out the wall and fence lines and mark the positions of posts, gates and bracing panels.

#### Clearing

Extent: Except trees or shrubs to be retained, clear vegetation within 1 m of the landscape walls. Grub out stumps and roots of removed trees or shrubs and trim the grass to ground level, but do not remove the topsoil.

#### Excavation

Excavate for foundations and footings.

#### Earth footings

Backfill with earth around posts, compacting firmly by hand or machine in 150 mm deep layers.

#### Concrete footings

In ground: Place mass concrete around posts and finish with a weathered top falling 25 mm from the post to ground level.

On slabs: Provide welded and drilled post flanges and fix with 3 masonry anchors per post.

Strip footings: Place mass concrete or reinforced concrete footings for walls. Refer to drawings for details.

### Gates

#### Types

Gates are to be constructed with minimum 30 x 30mm steel tube frames for rigidity. Infill panels can be steel sheet, steel mesh, timber boards or other material as identified on the drawings.

#### Hardware

Provide the following:

- Drop bolt and ferrule to each leaf of double gates.

- Latch to one leaf of double gates.

- Provision for locking by padlock.

- Holding lugs for security bars to inside face of double gates with vehicle access.

- Minimum of 2 hinges for gates 1.2m high. 3 hinges for gates 1.2 to 2.1m high. 4 hinges for gates greater than 2.1m high.

Schedule any particular requirements here e.g. signage, ball bearing lubricated hinges, spring closer, motor drive.

#### Hand access

General: Where required, provide hand holes to give access from outside to reach locking provision.

### Timber fencing

#### Timber picket fence

There is no standard for timber fencing.

Height (mm):As shown on drawings

Maximum post spacing: 2400 mm.

Member sizes (dressed):

- Posts: 90 x 90 mm.

- Rails: 70 x 40 mm.

- Pickets: 70 x 19 mm.

Picket spacing: 125 mm maximum.

Footing type: Earth.

Footing size: 200 mm diameter x 600 mm depth.

Picket pattern and Picket spacing: e.g. “gothic”, “pointed”, “rounded”, “colonial” pattern and vary the spacing if required.

#### Installation

General: Mortice posts, taper splice rails and nail twice in mortices. Set pickets and palings clear of the ground.

Or bolt fix to housings.

Picket fence: Nail twice to each rail.

Or substitute a galvanized steel tube frame if you prefer it for durability and stability.

### Chainwire barriers

Chainwire fencing of the type described here may be used with no rails or with one or two rails and with or without barbed extensions.

#### Fence dimensions

Maximum post spacing: 3000 mm.

#### Component sizes

Intermediate posts: 42.4 mm diameter, 2.6 mm wall thickness.

End, corner and gate posts: 60.3 mm diameter, 2.9 mm wall thickness.

Chainwire: 3.15 mm diameter wire woven to form uniform mesh.

- Mesh generally: 50 mm.

Tie wire: 2 mm diameter.

Post and rail barriers:

- Rails and gooseneck stay: 33.7 mm diameter, 2.6 mm wall thickness.

Railless barriers:

- Struts: 42.4 mm diameter, 2.6 mm wall thickness.

- Cable wires:

- Two strands: 3.15 mm diameter wire.

- One strand: 4 mm helicoil wire.

Security barriers:

- Chainwire selvedges: Twisted and barbed.

- Barbed wire to security fencing post extensions: Barbs at 95 mm maximum centres.

#### Installation

Posts: Do not splice members except in posts when splice is embedded at least 150 mm into concrete. Fit tightly fitting steel caps to posts, except where fixed to overhead structure.

Chainwire: Lace chainwire to end and gate posts. Tie chainwire twice around members at 250 mm maximum intervals. Twist ends twice and cut off neatly.

Cable wire: Tension cable wire(s) to support chainwire after at least 24 hour curing of concrete footings.

Footing type: Concrete.

- Footing size:

- Intermediate and end posts: 225 mm diameter x 600 mm depth.

- Corner posts and gate: 225 mm diameter x 900 mm depth.

Post and rail barriers:

- Rails: Connect rail(s) to posts using bolted split pipe fittings and purpose-made caps and brackets with rail apertures.

- Continuous rail type fences: Join the rails together in long lengths using purpose-made sleeves or socketed connections, and pass them through the apertures of caps and brackets on intermediate posts.

Railless barriers:

- Struts: Provide struts at ends, corners and gates.

Security barriers:

- Security fencing: Strain barbed wire between post extensions.

#### Gates

The gate post sizes and gate details described here should be increased if a pair of gates wider than 3600 mm is required. In addition sliding gates and motorised gates are available. Draw from the standard manufacturer’s specification if you require more detail.

Frame tubes: 33.7 mm diameter, 2 mm wall thickness.

Chainwire: Match fence.

Maximum width: 3600 mm.

Security barriers:

- Barbed wire security gate extension supports: 26.9 mm diameter, 2 mm wall thickness.

- Barbed wire: Match fence.

### stone walls

#### Construction

Select the stones for their locations and lay them in the wall with the minimum of stonecutting.

e.g. “Natural weathered face”, “Quarry faced”.

Footings: Select the largest, flattest and most regular stones for footings, and set them in concrete blinding in accordance with drawings.

Copings: Select stones of reasonably uniform size and finish the top of the wall to a level line or cap with precast concrete sections.

#### Retaining walls

Construction: Where dry stone walls act as retaining walls, construct the stonework to be free draining through the wall. Batter back the wall face 50 – 70 mm for every 300 mm in height. Cap the top of the wall. Backfill progressively, with a layer at least 300 mm thick of porous material, such as coarse aggregate or crushed rock in the size range 20 – 40 mm. Install filter fabric to stop movement of silt into porous material.

Minimum thickness: 450 mm.

Where stone walls are mortared, batter back the wall face 50 – 70 mm for every 300 mm in height. Cap the top of the wall. Backfill progressively, with a layer at least 300 mm thick of porous material, such as coarse aggregate or crushed rock in the size range 20 – 40 mm. Install filter fabric to stop movement of silt into porous material. Install a slotted pipe drain at the bottom of the wall backfill to ensure all water is drained away from the wall face.

Minimum thickness: 450 mm.

### Crib walls

#### Construction

Construct walls in conformance with the manufacturer’s written requirements or specific design included in the drawings.

Specify and detail to the recommendations of the product supplier.

### Gabion walls

#### Assembly

Construction: Assemble the baskets and join them together by wiring along edges both horizontally and vertically before placing the rock fill. Fix the top of the basket by wiring to both the sides and the diaphragms.

### Brick, earth block walls

#### Construction

Construct walls in conformance with the specific design included in the drawings. Construction of brickwork and earth blockwork to be in accordance with the relevant specification sections.

Specify and detail to the recommendations of the product supplier.