# Service trenching

Worksection application

This worksection is applicable to trenching for underground services.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Site preparation for work near trees.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection. For example:

- This worksection does not include any particular laying and bedding requirements such as clearance under pipe collars, mortar bedding and the like, and any special requirements for trench widths, depths, gradients.

- Worksection cross referencing

- Other worksections that cross reference this worksection are:

- Hydraulic design and install.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Hydraulic general requirements.

- Electrical general requirements.

- Mechanical general requirements.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made at the following stages:

- Service trenches excavated before laying the service.

- Services laid in trenches and ready for backfilling.

Add Hold points, if required.

## Execution

### Excavating

#### Excavation

Excavate for underground services, to required levels and grades. Generally make the trenches straight between inspection points and junctions, with vertical sides and uniform grades.

#### Trench widths

General: Keep trench widths to the minimum required for laying and bedding of the relevant service and construction of pits.

#### Trench depths

If excavation is necessary below the zone of influence of the underside of adjacent footings, give notice, and provide support for the footings as instructed.

#### Obstructions

Clear trenches of sharp projections. Cut back roots encountered in trenches to at least 600 mm clear of services. Remove other obstructions including stumps and boulders which may interfere with services or bedding.

See also Work near trees in the Site preparation worksection, regarding cutting of roots. If Site preparation is not included in the project specification import the clause here.

#### Dewatering

Keep trenches free of water. Place bedding material, services and backfilling on firm ground free of surface water.

#### Excess excavation

If trench excavation exceeds the correct depth, reinstate to the correct depth and bearing value using compacted bedding material or sand stabilised with 1 part of cement to 20 parts of sand by weight.

### Backfilling

#### General

Backfill service trenches as soon as possible after the service has been laid and bedded, if possible on the same working day. Place the backfill in layers maximum150 mm thick and compact to approval of Employers representative.

#### Backfill material

General fill with no stones greater than 25 mm occurring within 150 mm of the service, or other materials as required for particular services or locations.

Under roads and paved areas and within 4 m of building: Coarse sand, controlled low strength material or fine crushed rock.

In topsoil areas: Complete the backfilling with topsoil for at least the top 100 mm.

### Reinstatement of surfaces

#### General

Reinstate existing surfaces removed or disturbed by trench excavations to match existing and adjacent work.