# Precast concrete

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to precast concrete members such as floor and wall panels, beams and columns. The worksection permits contractor design.

The worksection does not apply to precast concrete elements that are cast horizontally off or on site, and which are initially lifted by rotation about an edge and temporarily braced (designated tilt-up).

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Tilt-up concrete.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

- Concrete formwork.

- Concrete reinforcement.

- Concrete post-tensioned.

- Concrete finishes.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

- Documenting this and related work

- You may document this and related work as follows:

- It is important that the structural drawings either define performance criteria (loading, deflection, exposure) or detail the member sizes and the connection details. If the former is adopted, independent certification by a professional engineer of the design and documentation, and of the erected elements, are appropriate.

- If precast is not by contractor design:

.	-	Precast concrete members should be detailed on the drawings.

.	-	Handling requirements, including lifting and transportation should be set out on the drawings.

The core reference for precast concrete is CIA Z48. For tilt-up precast concrete it is AS 3850.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

CIA Z48 is a joint handbook by the National Precast Concrete Association Australia and the Concrete Institute of Australia entitled Precast Concrete and is available from Standards Australia.

### Definitions

#### Definitions

For the purposes of this worksection the following definition applies:

- Precast units: Concrete elements manufactured in other than their final position including elements manufactured on site but excluding tilt-up panels.

The Definitions subclause can be augmented to suit the project or deleted if not required.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following:

- Formwork dimensions and stability.

- Panel edge details and penetrations.

- Connection materials, reinforcement and inserts in place.

Delete as appropriate, if contractor is responsible for design and is to be permitted to proceed without review.

Hold points, if required, should be inserted here.

### Submissions

#### Subcontractors

Submit name and contact details of proposed manufacturer of precast concrete units.

#### Design

Veneered fabrication: If veneered fabrication is proposed submit proposals to the Employers Representative.

Veneered construction should only be approved after due consideration has been given to all aspects, including the porosity of the veneer facing, the location of the reinforcement, the method of casting, the experience of the manufacturer, and the question of a warranty.

Contractor design: Provide verification by a professional engineer of compliance of the design with project documents.

Delete if the works are fully documented.

#### Shop drawings

See also CIA Z48 clause 12.6.2.

Submit shop drawings of precast units showing the proposed details for their design, manufacture, assembly, transport and installation, including the following:

- Project title and manufacturer's name.

- Shape or profile drawings (submit these before fabrication of moulds and tooling).

- Concrete mix and type of cement if special-class concrete.

- Veneer details, if applicable.

- Surface finish class and surface treatment, if applicable.

- Curing and protection methods.

- Marking plan.

- Equipment and methods for handling, transport and installation, including lifting inserts and pick-up points.

- Calculated maximum loadings on lifting and bracing inserts and attachments.

- Evidence of load capacity of lifting and bracing inserts and attachments in the form of test reports or calculations.

With contractor design, shop details are provided for review, rather than approval.

#### Lifting

Specify here the testing requirements, if any, for lifting inserts and attachments for precast units.

Early lifting: If it is proposed to lift the units by their designated lifting points before 28 day strength has been achieved, submit evidence to demonstrate that the unit has adequate strength to carry its own weight without damage or residual cracking or deflection on removal of the lifting device.

Attachments for handling purposes: If it is proposed to locate lifting attachments, holes and other temporary fixings for handling purposes on visible faces of units, submit proposals.

Lifting units: If it is proposed to lift or support units at other than specified points, submit proposals.

Delete these paragraphs for contractor design.

### Prototypes

#### General

Provide prototypes in accordance with the Precast concrete prototypes schedule.

Maintain prototypes on site, undamaged and protected from discolouration for comparison with manufactured precast units.

See AS 3600 Appendix B. AS 3600 is cited in the BCA for structural sufficiency and fire resistance.

You may waive the requirement for structural testing if satisfied with the calculations for performance requirements submitted with the precasting shop drawings.

#### Test panels

Make separate test panels for surface finish, colour, or both, in conformance with the Precast concrete prototypes schedule.

Specify the test panels required, or preferably show them on the drawings. Incorporated features may include tie bolts, construction joints and surface patterns.

AS 3600 clause B4.2 requires a minimum of 2 prototypes.

## Products

### Precast units

#### Marking

Identification: Identify units by marks which are as follows:

- Remain legible until after the unit has been fixed in place.

- Are not visible in the completed structure.

- Show the date of casting.

- Show the correct orientation of the unit.

- On other than units manufactured as a standard product, indicate the locations within the structure in accordance with the marking plan.

Delete if all the requirements are shown on the drawings.

#### Tolerances

Fixings and embedded items in precast units: To be maximum of 5mm from design location unless agreed otherwise with the Employers Representative.

Tolerances necessary for structural integrity are specified in AS 3600 clause 19.5, but are generally less stringent than those required for serviceability, fit of components, or aesthetics. AS 3610 clause 3.4 covers both in situ and precast construction.

Show on the drawings the required tolerance on dimensions of joints between precast units.

#### Lifting devices

Capacity: Design each lifting device for a working load at least 1.65 times the maximum calculated static load at that point and an ultimate load  4 times the maximum static load.

Include this clause only for contractor design. On these and other embedded items, see AS 3850 clause 2.4.2. On their placement tolerances, see AS 3610 Table 3.4.3.

#### Attachments

Sealing: Recess lifting attachments such as ferrules, or other types of cast-in fixings, and provide plugs for sealing.

See WTIA TN11 for a commentary on the AS/NZS 1554 series.

### Veneered construction

#### General

Use a method which ensures that delamination of the veneer will not occur. Obtain approval from the Employers Representative prior to construction commencing.

See Durability of Precast Veneer Panels, a paper presented at the 20th Biennial Conference of the Concrete Institute of Australia in September 2001 and Data Sheet 4 Veneer Construction, both available from the National Precast Concrete Association Australia.

## Execution

### Handling

#### Precautions

Lift or support units only at designated or other approved points. Use handling methods which do not overstress, warp or damage the units. Protect the units against staining, discolouration and other damage until they are installed in their final location.

#### Attachments

Remove temporary attachments after erection. Seal or otherwise make good residual recesses.

### Installation

#### General

Fixing: Fix the units securely and accurately in their final positions.

Ancillaries: Provide components and materials, including fasteners, braces, shims, jointing strips, sealant, flashings, grout and mortar, necessary for the installation of the units.

Show on the drawings the method of connection of the units to the frame, or call for the connections to be shown on the precasting shop drawings. Detail the method of jointing panels on the drawings.