# Light steelwork

There is a Product Partner for this worksection. Click the logo to be redirected to their page.

Worksection application

This worksection is applicable to framing for structures generally comprising cold formed metallic-coated or aluminium/zinc-coated steel sections where particular project specific requirements need to be met.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- There are no directly related NATSPEC worksections.

Worksection(s) on which this worksection relies

The following worksections must be included, with this worksection, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection. For example:

- Proprietary components or systems installed in accordance with the manufacturers’ published requirements.

- Worksection cross referencing

- Other worksections that cross reference this worksection are:

- Structural steel.

- Joinery.

- Documenting this and related work

- You may document this and related work as follows:

- Where cold formed steel sections are being used for structural framing, it is important that the structural drawings either define performance criteria (loading, deflection, exposure) or detail the member sizes and the connection details. If the former is adopted, independent certification by a professional engineer of the design and documentation, and of the erected framing, are appropriate.

- Cold-formed sections used in non-load bearing partitions which are specified in the Partitions – framed and lined worksection.

- Cold-formed purlins and girts used in conjunction with structural steel framing which are specified in the Structural steel worksection.

## General

AS 3623 is a performance-based design standard for the framing of domestic buildings of up to two storeys in height and roof pitches of up to 35°. It is a deemed-to-satisfy standard in the BCA (Housing Provisions). Include if the contractor is responsible for design.

### Inspection

#### Notice

Give notice so that inspection may be made of steel framing erected before lining or cladding.

If independent certification is being required, delete this clause.

Add Hold points, if required.

### Submissions

#### Design

The Contractor is to confirm that all proposed member sizes are available for the project in accordance with the drawings and BOQ. If selected sizes are not available, seek alternatives and obtain approval from the Employer’s representative.

## Execution

### Construction generally

#### Fabrication

Length: Cut members accurately to length so that they fit firmly against abutting members.

Service holes: Form holes by drilling or punching if needed.

- Bushes: Provide plastic bushes or grommets to site cut holes.

- Swarf: Remove swarf and other debris from cold-formed steel framing immediately.

Site work: Do not fabricate on site where welded connections are required.

#### Fastening

Select from the following:

- Bolting.

- Self-drilling, self-tapping screws.

- Blind rivets.

#### Welding

Burning: Avoid procedures that result in greater than localised “burning” of the sheets or framing members. Protect other adjoining materials from damage during welding activities.

Other workers: Protect other workers on site from welding flash, sparks and other potential injuries during welding activities.

#### Prefabricated frames

Protect frames from damage or distortion during storage, transport and erection.

Timber noggings or supports can be used to isolate the service pipes and accessories from the steel framing.

#### Unseasoned timber

Do not fix in contact with framing without fully painting the timber and/or the steel to avoid future rusting of the steel.

#### Earthing

Permanent earthing: Required.

Temporary earthing: Provide temporary earthing during erection until the permanent earthing is installed.

The local authority may have requirements for permanent earthing of the frame.

Refer to AS/NZS 3000 Section 5 for earthing arrangements and earthing conductors.

#### Protection

Coatings which have been damaged by welding or other causes shall be restored. Thoroughly clean affected areas to base metal and coat with zinc rich organic primer.

### Trusses

#### Fabrication

Factory assemble trusses and transport to site where possible. Obtain approval from the Employer’s representative if it is required to fabricate trusses on site.

#### Marking

Permanently mark each truss to show:

- Manufacturer.

- Tag or number.

- Location.

- Support points.

#### Installation

Fix to support structures, plumb to within H/200, where H is the height at the apex.

### Completion

#### Cleaning

On completion of framing remove debris from any gaps between members.