# Partitions – systems

Worksection application

This worksection is applicable to complete proprietary partition systems.

Related material located elsewhere in NATSPEC

Related material may be found in other worksections. For example:

- Doors and hatches and Door hardware for partition door leafs and hardware.

- Partitions – framed and lined for framed and lined (drywall) partitions built in situ.

- Partitions – glazed for framed and glazed partitions and frameless glass partitions.

- Cubicle systems or Terrazzo precast for toilet or shower partitions.

- Room dividers for room dividers.

- Applied wall finishes for decorative claddings to partitions.

NATSPEC worksection(s) on which this worksection relies

The following worksections must be included, if this worksection is used, to provide a complete specification:

- General requirements.

Material not included in NATSPEC

Some projects may include items not covered by NATSPEC. For these you may need to create new text, modify this text or use an applicable worksection.

Worksection cross-referencing

Other worksections that reference this worksection are:

- None.

Documenting this and related work

You may document this and related work as follows:

- Indicate the location and a scheduled description of partitions on drawings to your office documentation policy.

## General

List worksections cross referenced by this worksection. (General requirements references the other general worksections e.g. Metals and prefinishes do not repeat them here.) You may also wish to direct the contractor to other worksections where there may be work that is closely associated with this work.

- Windows.

- Doors and hatches.

- Door hardware.

- Glazing.

- Partitions – framed and lined.

- Cubicle systems or Terrazzo precast.

- Room dividers.

- Applied wall finishes.

Delete if not required.

### INTERPRETATION

#### Definitions

For the purposes of this worksection the definitions given below apply.

- Partition – fully demountable: A partition system in which any component may be demounted without damage, using only small hand tools, and subsequently reassembled without cutting, trimming or refinishing.

- Partition – semi demountable: A partition system in which  the major components are designed to be removed and reused but panels or linings, which are likely to be damaged during removal, are not.

- Partition – non demountable: A partition system in which major components, such as panels or linings, are likely to be damaged during removal and may require cutting, trimming or structural repair before reuse.

- Cool room panel system: A partition system fabricated to suit specific thermal conditions in which the insulated panels are designed to be removed and reused but panels or trims are likely to be damaged during removal.

### Inspection

#### Notice

Give sufficient notice so that inspection may be made of the following stages:

- Installation of framing / fixings before they are enclosed.

Hold points, if required, should be inserted here.

### Submissions

#### Samples

Submit samples as follows:

- A sample, at least 300 x 300mm, of each panel type.

- Floor and ceiling fixings and adjustments.

- Samples at least 100 mm long of each structural section, including posts, sills, transoms, door frames, ceiling channels and metal channel headrails, and each moulding, cover strip and bead.

- Skirting, skirting duct, skirting duct stop ends, returns and removable covers.

If prototypes are specified, separate samples of visible components may not be necessary.

### Tolerances

#### General

Deviation (from true grid lines and planes): up to 3 mm maximum in a 1500mm length.

Misalignment (of adjoining surfaces at grid junctions): 3 mm maximum.

Flatness, twist and bow: Maximum 3 mm deviation from a 1500mm straightedge placed in any position.

## Products

### Partitions

#### General

Provide proprietary non-load-bearing partition wall framing and lining comprising cold formed steel or extruded aluminium members, or both, in conformance with the Partition schedule.

#### Building movements

Provide clearances or movement joints so that partitions are not damaged by structural building movements such as long term slab deflection. Where fire resistance or acoustic properties are specified provide a resilient foam or mastic seal having properties equal to those required for the partition.

#### Control joints

General: Provide for control joints in sheet finishes where required by the structural frame.

Select materials and detail baffles to maintain the weighted sound reduction index nominated for the partition

## Execution

### Preparation

#### General

Prepare the base to receive the partitions. Ensure that all surfaces are flat without lumps or hollows that could affect the performance of the partition system.

#### Set out

Set out the partitions so that the partition grid, as expressed in panel joints and centrelines of frame members, coincides with the ceiling grid and the building grid, if applicable.

### Erection

#### Partition erection

Install the partitions so they:

- Are plumb, level, on their correct alignments, and firmly fixed.

- Have adequate top support by fixing the top plate to the ceiling structure or slab soffit, or are stabilised by lapping and fastening intersecting or butting plates together.

- Have bottom plates fixed at 600 mm maximum centres generally, and 100 mm maximum from ends.

- Install coolroom panel systems to manufacturers recommendations and standard details where feasible. Ensure that all seal strips, cover strips, accessories, fixings needed to satisfactorily carry out the installation are provided and installed to the approval of the Employers representative.

Edit to suit partition system.

#### Fixing

Conceal fixings. For demountable items provide fixings capable of being repeatedly removed and replaced without damage to finishes.

Fixing to masonry: Provide masonry anchors of expansion or chemical grout type. Do not provide explosive-driven fastenings.

Fixing to suspended ceilings: Provide adequate top support to the partition without damage to the ceiling components.

#### Protection

Protect existing work from damage during the installation and make good any damage. Provide temporary coverings if necessary.

#### Sound properties

Preserve the sound reduction properties of partitions by sealing flanking sound transmission paths during installation, including junctions between partitions and other building surfaces, air gaps around doorsets, recesses, such as pelmets and blind boxes and cut-outs for services. Avoid cut-outs next to or back-to-back with each other.

Sealing methods: Use appropriate sealing methods, such as durable resilient gaskets or closed cell foam strips. Provide solid resilient materials in preference to foamed materials whenever possible.

Preferably show on the drawings the actual methods and materials for sealing junctions with the building structure.

Details of partition junctions with window mullions may have to accommodate deflection while maintaining acoustic properties.

#### Support

Provide additional support in the form of framing for fixing hardware, fixtures and fittings.

### Services

#### Services access

Conceal associated building services, either within cavities in the partition structure, or within ducted skirtings supplied as part of the partition system, or both. Provide removable or demountable components of the partition system, for access to services concealed within partition cavities.

Show sufficient details of the services on the drawings (e.g. diagrammatic service runs, cable, pipe or duct sizes, outlets, etc.). The clause Submissions requires the method of reticulation to be shown on the shop drawings. Consider access for repairs, etc. Show access panels on the drawings.

Demountability: E.g. fully demountable, semi demountable or non demountable.