OTI-09 - Laparotomy box

Description

Complete set of instruments for major laparotomy

Reference Pictures at the end of this document as part of Annex 1.

| Brand / Marque |  |  |  |
| --- | --- | --- | --- |
| Model / Modèle |  |  |  |
| Model code as per manufacturer classification / Numéro de référence du produit fournisseur |  |  |  |
| Manufacturer name / Nom du Fabricant |  |  |  |
| Manufacturing site / Site du Fabricant |  |  |  |
| Country of manufacturing / Pays de fabrication |  |  |  |
| Country of origin / Pays d'origine |  |  |  |

|  | UNOPS Minimum technical requirements // Exigences techniques minimales de l'UNOPS | Complies (Yes/No) // Conforme (Oui/Non) | Comments/ Technical data // Commentaires/Données techniques |
| --- | --- | --- | --- |
| A | Functionality and Performance / Fonctionnalité et performances |  |  |
| 1 | Made of high grade medical Stainless Steel AISI 316 grade, clearly identifying the grade of steel used in the manufacture of the instruments, in accordance with ISO 7153-1: 2016, DIN 58298, ASTM F899 or an equivalent standard. |  |  |
| 2 | Non-sterile, reusable instruments. |  |  |
| 3 | Can be autoclaved at 134ºC |  |  |
| 4 | Please note that the ranges and dimensions shown below are approximate. They may vary from brand to brand. |  |  |
| 5 | Complete set of surgery instruments for cesarean section contains all the items contained in the Annex 1 (Description must be included in column “Comments/ Technical data // Commentaires/Données techniques” for each item) |  |  |
| B | Certifications and post sales included in the offered price |  |  |
| 1 | The Supplier, included in the offered price, will provide the post sales services described in the document “OT-PS Type I post sales requirements for Medical Devices”. |  |  |
| C | Supplied with |  |  |
| 1 | Detailed step-by-step instructions for cleaning and disinfection, preferably using pictograms and Lubricating agents where necessary for maintenance.. |  |  |
| D | ANY DEVIATIONS OR SUPERIOR CHARACTERISTICS MUST BE INDICATED BELOW // TOUTE DÉVIATION OU CARACTÉRISTIQUE SUPÉRIEURE DOIT ÊTRE INDIQUÉE CI-DESSOUS* : |  |  |

Annex 1 - Box composition

- Sterilization container: for sterilization of surgical instruments, with perforated lid, perforated button and clip lock. Reference dimension (Length x width x height): 470X274X135 mm. Size 3/4.

- Surgery Instrumentation:

| Item | Description | Detail | Reference Dimensions | Nr of units | Complies (Yes/No) // Conforme (Oui/Non) | Comments/ Technical data // Commentaires/Données techniques |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1 | Bengolea Forceps | Straight, 1x2 Teeth | 18cm | 3 |  |  |
| 6.2 | Bengolea Forceps | Curved, Toothless | 16 cm | 3 |  |  |
| 6.3 | O’Shaugnessy Dissecting Forceps | - | 23cm | 1 |  |  |
| 6.4 | Kelly Forceps | Straight | 16 cm | 3 |  |  |
| 6.5 | Kelly Forceps | Curved | 16 cm | 3 |  |  |
| 6.6 | Dissecting Forceps | 1X2 teeth | 18 cm | 1 |  |  |
| 6.7 | Dissecting Forceps | straight, toothless | 18 cm | 1 |  |  |
| 6.8 | Dissecting Forceps | 1X2 teeth | 25 cm | 1 |  |  |
| 6.9 | Dissecting Forceps | straight, toothless | 25 cm | 1 |  |  |
| 6.10 | Mayo Scissors | Straight | 18 cm | 1 |  |  |
| 6.11 | Mayo Scissors | Curved | 18 cm | 1 |  |  |
| 6.12 | Metzenbaum Scissors | Curved | 26 cm | 1 |  |  |
| 6.13 | Mayo-Hegar Needle Holder | - | 20 cm | 1 |  |  |
| 6.14 | Kocher Forceps | Straight, 1x2 Teeth | 14 cm | 6 |  |  |
| 6.15 | Kelly Retractor | Blade 190x38 mm |  | 1 |  |  |
| 6.16 | Backhaus Towel Clamp | - | 14 cm | 6 |  |  |
| 6.17 | Dartigues Elevating Forceps | - | 25,5 cm | 1 |  |  |
| 6.18 | Gosset Abdominal Retractor | Set Complete |  | 1 |  |  |
| 6.19 | Farabeuf Retractor | Pair | 15 cm | 1 |  |  |
| 6.20 | Allis Forceps | 5x6 Teeth | 19 cm | 2 |  |  |
| 6.21 | Scalpel Handle | #4 | 13,5 cm | 1 |  |  |
| 6.22 | Scalpel Handle | #3 | 12,5 cm | 1 |  |  |
| 6.23 | Holding ring handle instruments | - | 18 cm | 2 |  |  |
| 6.24 | Metal Cup | - | 80 ml | 1 |  |  |
| 6.25 | Metal Cup | - | 500 ml | 1 |  |  |
| 6.26 | Cotte Self-retaining retractor | - | - | 1 |  |  |
| 6.27 | Rezzano muscle forceps | Atraumatic | - | 1 |  |  |
| 6.28 | Jean-Louis Faure Forceps | Curved, 1x2 teeth | 22 cm | 1 |  |  |

Reference Picture:

Note: The images below are intended solely as a guiding support and should be considered as purely indicative and not restrictive of the expected item characteristics

Item 5:

Item 6.1:

Item 6.2:

Item 6.3:

Item 6.4

Item 6.5

Item 6.6 & 6.8

Item 6.7 & 6.9

Item 6.10

Item 6.11

Item 6.12

Item 6.13

Item 6.14

Item 6.15

Item 6.16

Item 6.17

Item 6.18

Item 6.19

Item 6.20

Item 6.21 & 6.22

Item 6.23

Item 6.24 & 6.25

Item 6.26

Item 6.27

Item 6.28

Supplied with:

Detailed step-by-step instructions for cleaning and disinfection, preferably using pictograms and Lubricating agents where necessary for maintenance.

Post sales services requirement

The Supplier, included in the offered price, will provide the post sales services described in the document “OT-PS Type I post sales requirements for Medical Devices”.