Section II: Schedule of Requirements

| IUS-AF2-UWS- HSWN -003 Supply, Deliver, Testing and Training of welding machines (HDPE Pipe Butt Fusion Jointing Machine) with silent mobile Diesel Generator compatible for operation. To Be Delivered to National Water & Sanitation Authority – HASWIN Branch - NWSA- HASWIN’s Warehouses, HASWIN District - Al Maharah Governorate |  |  |  |
| --- | --- | --- | --- |
| No. | Description | Unit | Qty. |
| 1 | HDPE Pipe Butt Fusion Welding Machine |  |  |
| 1.1 | Supply, Testing and Deliver of Welding Machines (HDPE Pipe Butt Fusion Jointing Machine) and all necessary accessories as per required and according to Technical Specifications | Set | 1 |
| 2 | (9 – 10) KVA Quiet Diesel Mobile Generator |  |  |
| 2.1 | Supply, Testing and Deliver of (9 – 10) KVA Quiet Diesel Mobile Generator, according to the Technical Specifications. | Set | 1 |

SCOPE OF WORK

The sub-project is to Providing Spare Parts for The Maintenance of Water Transmission Pipelines and Distribution Networks That Were Damaged by Hurricane Tej in HASWIN City with All Accessories for NWSA – HASWIN Branch.

- GENERAL

- The goods to be supplied must be new and unused.

- Delivery and Test of supplied goods of supplied goods will be made for National Water & Sanitation Authority – HASWIN Branch - NWSA- HASWIN’s Warehouses, HASWIN District - Al Maharah Governorate

- The material to be supplied shall be new, unused and of the latest design (refurbished, used Material or the stock for more than 3 years will not be considered)

- Documentary evidence to prove that the material offered comply with the Technical Specifications given in (Section II and Section III Form C) must be provided.

- The supplied goods and materials shall conform to the authoritative standards appropriate to the Goods country of origin. Such standards shall be the latest issued by the institution concerned.

- Technical Data/Brochures: bidders Must provide relevant technical data detailing the technical data of the material and original manufacturer brochures, where there is more than one model on a brochure, the bidder, must highlight which they are quoting for in this tender.

- All materials shall be complying with specified International Standards and shall be supplied from reputable and approved manufactures and country of origin.

- Markings:  Unless otherwise specified in the relevant standard, products are to have the following legible marks as appropriate, cast, stamped or indelibly painted:

-  Manufacturer’s name, initials and identification mark.

-  Class designation.

-  Date of manufacture.

-  Initials and number of relevant standards.

- Transport, handle and store all products and materials in accordance with the manufacturer’s recommendations and in a manner that prevents damage or deterioration or excessive distortion.

- The employer UNOPS, UWSSP / NWSA- HASWIN Branch shall have the right to reject any damaged or defective product material.

- Warranty & training,

- Recommended Preventive Maintenance Schedule for the equipment shall be included.

- Final test and Inspection of supplied goods shall be carried out including: (Visual Inspection, Load test, testing, Material test certificate, field testing, etc) shall be done in the destination place (HASWIN city/ Al-Mahara Government).

- One-week operation and maintenance training for the technical crew and operators of the unit shall be conducted upon delivery at supplier`s cost.

- One-year warranty for the whole unit. Warranty must be approved from the Bidder and the Manufacturer.

- Warranty of after sell service support for the unit shall be available through authorized services centre in the Yemen.

- Climate:

- All materials and equipment incorporated in the work shall be capable of withstanding shade temperatures of 45°C and if exposed to direct sunlight shall be capable of withstanding the additional heat gain up to 70°C , approx. 90% humidity and attitude 300 m above sea level. All materials and equipment shall take into account the generally tropical conditions with occasional dust storms, intense rain storms.

Technical specifications:

IUS-AF2-UWS- HSWN -003- :Welding Machines (HDPE Pipe Butt Fusion Jointing Machine)

Product Description

- For welding PE / PP / PVDF

- Consists of basic frame, hydraulic unit, planning tool, heating plate, support stand for planning tool & heating plate, and optional parts.

- Removable PTFE coated heating plate with high accurate temperature control system

- Electrical planning tool.

- Be made of lightweight and high strength material, simple structure, sturdy & user friendly.

- Low starting pressure ensures reliable welding quality of small pipes.

- Suitable to weld fittings

Product Specification

| - Welding Type: Butt Fusion |  |
| --- | --- |
| - Power Consumption: 5 Kw |  |
| - Phase: Single Phase |  |
| - Usage/Application: Welding Machine |  |
| - Type: Automatic |  |
| - Temperature Deviation in Surface: +/- 5 % | ` |
| - Temperature Range: 50 To 300 Degree C |  |
| - Weight: < 200 kg |  |
| - Voltage: 220 V |  |
| - Frequency: 50 Hz |  |
| - Clamping: 4 |  |
| - Suitable for Pipe Size: 160, 180, 200, 225, 250, 280 & 315 mm |  |
| - Driven Type: Electric Motor |  |

IUS-AF2-UWS- HSWN -003- (9 – 10) KVA Quiet Diesel Mobile Generator

Technical Specification

- High quality, reliable and complete power unit

- Compact design

- Easy start and maintenance

- Fully tested for load, controls and safety shutdown functions

- Rated Power: (9 – 10) KVA

- Voltage: 220 Single-Phase or 400/230v 50 Hz

- Frequency: 50 Hertz

- Speed (RPM) 1500 rpm

- Auto Voltage Regulation

- Governing Type: Mechanical

- Fuel injection pump: Individual type

- Diesel Engine

- Water cooled

- Alternator: Brushless

- Insulation system:  Class H

- Integral 5A Battery Charger

Metering and Alarm indications:

- Generator frequency

- Under speed, Overspeed

- Generator volts (L-L, L-N)

- Generator current

- Engine oil pressure

- Engine coolant temperature

- Hours run counter

- Battery volts

- Fail to start/stop

- Emergency stop

- Charge fails

- CAN diagnostics and CAN fail/error

Delivery requirements and Comparative Data Table:

| UNOPS Requirements |  | Is quotation compliant? Bidder to complete | Details Bidder to complete |
| --- | --- | --- | --- |
| Delivery time | The Bidder shall deliver the goods within 6 months after the Contract signature. | ☐ Yes ☐ No | Insert details |
| Delivery place and Incoterms rules | DAP as per Incoterms 2020 at National Water & Sanitation Authority – HASWIN Branch - NWSA- HASWIN’s Warehouses, HASWIN District - Al Maharah Governorate. | ☐ Yes ☐ No | Insert details |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- [20%] , without any change in the unit prices or other terms and conditions of the RFQ. | ☐ Yes ☐ No | Insert details of goods offered, including specifications, picture, and brand/model offered if applicable |