Section II: Schedule of Requirements

| IUS-AF2-UWS-HSWN-003: Supply and Delivery of Equipment and Tools for the purpose of maintaining and operating the water network system. To Be Delivered to National Water & Sanitation Authority – HASWIN Branch - NWSA- HASWIN’s Warehouses, HASWIN District - Al Maharah Governorate |  |  |  |
| --- | --- | --- | --- |
| No. | Description | Unit | Qty. |
| Item # 1: 12" Petrol Disc Cutter machine (portable), according to the Technical Specifications |  |  |  |
| 1.1 | 12" Petrol Disc Cutter machine (portable), according to the Technical Specifications. | Set | 1 |
| Item # 2: Angle Grinder, according to the Technical Specifications |  |  |  |
| 2.1 | Angle Grinder, according to the Technical Specifications. | Set | 1 |
| Item # 3: Mechanics Tool Set - 68 Piece, according to the Technical Specifications. |  |  |  |
| 3.1 | Mechanics Tool Set - 68 Piece, according to the Technical Specifications. | Set | 2 |
| Item # 4: VDE Insulated Electrician's Tool Set - 41 Piece, according to the Technical Specifications. |  |  |  |
| 4.1 | VDE Insulated Electrician's Tool Set - 41 Piece, according to the Technical Specifications | Set | 1 |
| Item # 5: Electrical measuring instruments and devices, according to the Technical Specifications |  |  |  |
| 5.1 | Supply, Testing and Deliver of High Voltage & PV Insulation Resistance Tester, With all accessories, according to the Technical Specifications. | No. | 1 |
| 5.2 | Supply, Testing and Deliver of Digital Clamp Meter for measure the electrical current,With all accessories, according to the Technical Specifications. | No. | 2 |
| 5.3 | Supply, Testing and Deliver of Digital Multimeter Tester,With all accessories, according to the Technical Specifications. | No. | 3 |
| Item # 6: Plumber / Pipe Fitter Tool, according to the Technical Specifications |  |  |  |
| 6.1 | Supply, Testing and Deliver of (25 BAR) Pressure Gauge Stainless Steel Case, NS 100 [4"], according to the Technical Specifications. | No. | 4 |
| 6.2 | Supply, Testing and Deliver of Heavy-Duty Pipe Wrench, according to the Technical Specifications. With the following sizes: |  |  |
| 6.2.1 | Pipe Wrench 6” | No. | 5 |
| 6.2.2 | Pipe Wrench 8” | No. | 5 |
| 6.2.3 | Pipe Wrench 10” | No. | 5 |
| 6.2.4 | Pipe Wrench 12” | No. | 5 |
| 6.2.5 | Pipe Wrench 14” | No. | 5 |
| 6.2.6 | Pipe Wrench 18” | No. | 4 |
| 6.2.7 | Pipe Wrench 24” | No. | 4 |
| 6.2.8 | Pipe Wrench 36” | No. | 2 |
| 6.3 | Supply, Testing and Deliver of Heavy-Duty Adjustable Wrenches, according to the Technical Specifications. With the following sizes: |  |  |
| 6.3.1 | Adjustable Wrench 4” | No. | 5 |
| 6.3.2 | Adjustable Wrench 6” | No. | 5 |
| 6.3.3 | Adjustable Wrench 8” | No. | 5 |
| 6.3.4 | Adjustable Wrench 10” | No. | 5 |
| 6.3.5 | Adjustable Wrench 12” | No. | 5 |
| 6.3.6 | Adjustable Wrench 15” | No. | 2 |
| 6.3.7 | Adjustable Wrench 18” | No. | 2 |
| 6.4 | Supply, Testing and Deliver of Heavy-Duty Hacksaw Frame, according to the Technical Specifications. With the following sizes: |  |  |
| 6.4.1 | Heavy-Duty Hacksaw Frame (Size:300mm/12"), Including 6-pcs 65Mn saw blade for for each Hacksaw Frame | No. | 10 |
| 6.5 | Supply, Testing and Deliver of Heavy-Duty Toolbox 3 Levels Empty, according to the Technical Specifications. With the following sizes: |  |  |
| 6.5.1 | Toolbox 3 Levels Empty (Size:495x200x295mm) | No. | 10 |
| Item # 7: Safety Supplies, according to the Technical Specifications. |  |  |  |
| 7.1 | Supply, Testing and Deliver of First AID Kit, according to the Technical Specifications. | Kit | 4 |
| 7.2 | Supply, Testing and Deliver of 5.0 Kg CO2 Fire Extinguisher, according to the Technical Specifications. | No. | 4 |
| 7.3 | Supply, Testing and Deliver of Personal Protective Equipment (PPE), according to the Technical Specifications. |  |  |
| 7.3.1 | Supply, Testing and Deliver of Safety Shoes, according to the Technical Specifications. With the following sizes: 40, 41, 42, 43, 44 | No. | 10 |
| 7.3.2 | Supply, Testing and Deliver of Goggles, according to the Technical Specifications. | No. | 20 |
| 7.3.3 | Supply, Testing and Deliver of Safety helmet for workers, according to the Technical Specifications. With the following Colors: |  |  |
| 7.3.3.1 | White color safety helmets | No. | 10 |
| 7.3.3.2 | Yellow color safety helmets | No. | 10 |
| 7.3.3.3 | Blue color safety helmets | No. | 5 |
| 7.3.3.4 | Red color safety helmets | No. | 5 |
| 7.3.4 | Supply, Testing and Deliver of Leather Gloves, according to the Technical Specifications. (Pair of Gloves) | No. | 25 |
| 7.3.5 | Supply, Testing and Deliver of Work overalls, according to the Technical Specifications. With the following sizes: S, M, L, XL, XXL | No. | 20 |

SCOPE OF WORK

The sub-project is to Providing of Equipment and Tools for the purpose of maintaining and operating the water network system, to be delivered to National Water & Sanitation Authority – HASWIN Branch’s Warehouse

- GENERAL

- The goods to be supplied must be new and unused.

- Delivery and Test of supplied goods will be made for National Water & Sanitation Authority – HASWIN Branch - HASWIN’s Warehouses, HASWIN District - Al Maharah Governorate

- The material to be supplied shall be new, unused and of the latest design (refurbished, used Material or the stock for more than 3 years will not be considered)

- Documentary evidence to prove that the material offered comply with the Technical Specifications given in (Section II and Section III Form C) must be provided.

- The supplied goods and materials shall conform to the authoritative standards appropriate to the Goods country of origin. Such standards shall be the latest issued by the institution concerned.

- Technical Data/Brochures: bidders Must provide relevant technical data detailing the technical data of the material and original manufacturer brochures, where there is more than one model on a brochure, the bidder, must highlight which they are quoting for in this tender.

- All materials shall be complying with specified International Standards and shall be supplied from reputable and approved manufactures and country of origin.

- Markings:  Unless otherwise specified in the relevant standard, products are to have the following legible marks as appropriate, cast, stamped or indelibly painted:

-  Manufacturer’s name, initials and identification mark.

-  Class designation.

-  Date of manufacture.

-  Initials and number of relevant standards.

- Transport, handle and store all products and materials in accordance with the manufacturer’s recommendations and in a manner that prevents damage or deterioration or excessive distortion.

- The employer UNOPS, UWSSP / NWSA- HASWIN Branch shall have the right to reject any damaged or defective product material.

- Warranty

- Recommended Preventive Maintenance Schedule for the equipment shall be included.

- Final test and Inspection of supplied goods shall be carried out including: (Visual Inspection, Load test, testing, Material test certificate, field testing, etc) shall be done in the destination place (HASWIN city/ Al-Mahara Government).

- One-year warranty for the whole unit. Warranty must be approved from the Bidder and the Manufacturer.

- Warranty of after sell service support for the unit shall be available through authorized services centre in the Yemen.

- Climate:

- All materials and equipment incorporated in the work shall be capable of withstanding shade temperatures of 45°C and if exposed to direct sunlight shall be capable of withstanding the additional heat gain up to 70°C , approx. 90% humidity and attitude 300 m above sea level. All materials and equipment shall take into account the generally tropical conditions with occasional dust storms, intense rain storms.

Technical specifications:

### ITEM #1: 12" Petrol Disc Cutter machine (portable)

Description

- High efficiency, speed, and safety in cutting all types of pipes

- Rapid start pull force required to start the machine is drastically reduced with spring-assisted recoil starter

- Touch and stop switch

- Fuel primer for easy starting

- High quality driving belt does not require tensioning once the proper tension is adjusted

- Enhanced air filtration

- Developed in compliance with all known exhaust emission regulations

- Lightweight design

- Water supply with intake filter and fine adjustment for adequate dust protection

The machine with:

- Tool kit

- Dust suppression kit

- Diamond Blade

Specification:

| - Max. Cutting Depth: 110 mm |  |
| --- | --- |
| - Noise sound pressure: 97 dB(A) |  |
| - Noise sound power: 113 dB(A) |  |
| - Engine: 2-Stroke |  |
| - Arbour: 20 mm |  |
| - Max Wheel Speed: 10,000 rpm |  |
| - Wheel diameter: 305 mm |  |
| - Power Rating: 4.3 hp |  |
| - Fuel Tank Capacity: 0.7 litres |  |
| - Fuel Type: Petrol |  |
| - Displacement: 61 cm |  |
| - Power Rating: 3.2 kW |  |
| - Vibration no load: 3.7 m/sec |  |
| - Net weight: Less than 9kg |  |

### ITEM #2: Angle Grinder

Specification:

| - Voltage: 220-240V~50/60Hz |  |
| --- | --- |
| - Input power: 1100W |  |
| - No-load speed: 5000-12000rpm (variable speed) |  |
| - Disc diameter: 125mm |  |
| - Spindle thread: M14 |  |
| - With 1pcs auxiliary handle |  |

The Grinder with:

- Tool kit

- 3 Extra Discs

- Standard Spare Parts

### ITEM #3: Mechanics Tool Set - 68 Piece

Details

1/4" & 3/8" Drive 6- & 12-Point Standard & Deep SAE/Metric Mechanics Tool Set - 68 Piece.

Features

- 84 Tooth Full Polish Chrome Thin Proﬁle Ratchets

- Knurled Full Polish Chrome Extensions

- Includes Storage Case

- Full Polish Chrome Sockets with Large Hard Stamp Size Markings

- Mets or Exceeds ASME B107.1, B107.5M, B107.10

Speciﬁcations

| - Set Piece Count: 68 |  |
| --- | --- |
| - Set Included Storage: Blow Mold Case |  |
| - Set Size Range Min: 3/16", 4mm |  |
| - Set Size Range Max: 3/4", 17mm |  |
| - SAE/Metric/Torx: SAE/Metric |  |
| - ASME Speciﬁcation: B107.1, B107.5M, B1007.10 |  |
| - Drive Type: 6 & 12 Point |  |
| - Drive Tang Size: Multi Drive |  |
| - Length Format: Standard |  |
| - Finish: Full Polish Chrome |  |
| - Material: Alloy Steel |  |

### ITEM #4: VDE Insulated Electrician's Tool Set - 41 Piece

Description

Insulated tools must comply with the NF EN 60900 and IEC 60900 standards. To meet these standards, insulated tools undergo 10,000 volts through electrical testing and many mechanical tests, including impact tests, tearing tests, and puncture tests. For working live at nominal voltage up to 1000 V AC and 1500 V DC. (1000V,VDE/GS Certification is necessary)

Speciﬁcations

| - Material: Premium Chrome Vanadium steel |  |
| --- | --- |
| - The entire tools set come pack in a dedicated BMC certified box. |  |
| - Set Piece Count: 41 |  |
| - Set Include: |  |
| - 1 pc Insulated Long Nose Pliers: 160mm (6″) - 1 pc Insulated Combination Pliers: 200mm (8″) - 3 pcs Insulated Slotted Screwdriver: SL3.0*75mm, SL4.0*100mm, SL5.5*125mm - 2 pcs Insulated Phillips Screwdriver: PH1*80mm, PH2*100mm - 7 pcs Insulated Open End Spanners: 10mm, 11mm, 12mm, 13mm, 14mm, 17mm, 19m - 1 pc Voltage Tester: 3mm * 70mm - 1 pc Cable Knife: 50 * 180mm - 1 pc Insulated Electrician’s Scissors: 160mm (6″) - 1 pc Insulated Adjustable Wrench: 250mm (10″) - 1 pc Vinyl Electrical Tape - 11 pcs Insulated Hexagon Sockets (1/2″): 12mm, 13mm, 14mm, 16mm, 17mm, 19mm, 22mm, 24mm, 27mm, 30mm, 32mm - 4 pcs Insulated Bit Socket for Hexagon Screws (1/2″): 4mm, 5mm, 6mm, 8mm - 3 pcs Insulated Ring Wrench: 14mm, 17mm, 19mm - 2 pcs Insulated Extension Bars (1/2″): 125mm, 250mm - 1 pc 1/2″ Insulated Ratchet Wrench - 1 pc 1/2″ Insulated T-handle Wrench |  |

### ITEM #5: Electrical measuring instruments and devices

#### High Voltage & PV Insulation Resistance Tester

Technical Specifications

| General Specifications |  |
| --- | --- |
| - Measurement parameters: - Insulation resistance - PV insulation resistance - Leakage current - Voltage - Capacitance (DD function) |  |
| - Storage temperature and humidity range: -25°C to 65°C (-13°F to 149°F), less than 80% RH |  |
| - Operating temperature range: -20°C to 50 °C |  |
| - Operating environment: Indoors, up to 2,000 m |  |
| - Dustproof/waterproof: - IP40 (with protector attached, excluding terminals) - IP65 (CARRYING CASE) |  |
| - Others: Battery charge indicator, live circuit indicator, automatic power save, automatic discharge, backlight, buzzer, manual recording, logging recording, temperature and humidity input, elapsed time display, clock, filter, hardware filter, data-hold, system reset, USB communication, wireless communication, comparator, resistance gauge display, switching of insulation diagnosis function, breakdown cut-off, negative voltage notification. |  |
| Basic Specifications (input, output, measurement) |  |
| - Measurement items: - Insulation resistance, PV insulation resistance, leakage current, voltage, capacitance (DD function) |  |
| - Max. rated voltage to terminals: 1000 V AC, 2000 V DC |  |
| - Max. rated voltage to ground: 1000 V (CAT IV), 600 V (CAT III) |  |
| PV insulation resistance |  |
| - Test voltage range: DC 250 V to 2.00 kV |  |
| - Test voltage presets: 500 V, 1 kV, 1.5 kV |  |
| - User-set test voltage: 250 V to 1 kV: in 10 V increments; 1 kV to 2 kV: in 25 V increments |  |
| - Rated current: [Test voltage] / [20 MΩ] |  |
| - Open-circuit voltage: From 0% to 10% of test voltage |  |
| - Short-circuit current: 2 mA or less |  |
| - Possible number of measurements: 1000 times or more on one set of fully-charged batteries |  |
| Insulation resistance measurement |  |
| - Test voltage range: DC 250 V to 5.20 kV DC |  |
| - Test voltage presets: 250 V,500 V, 1 kV, 2.5 kV, 5 kV |  |
| - User-set test voltage: 250 V to 1 kV: in 10 V increments; 1 kV to 5 kV: in 25 V increments |  |
| - Rated current: 1 mA to 1.2 mA |  |
| - Short-circuit current: 2 mA or less |  |
| - Possible number of measurements: 200 times or more on one set of fully-charged batteries |  |
| Leakage current measurement |  |
| - Measurement range (auto): 10 nA, 100 nA, 1000 nA, 10 μA, 100 μA, 1 mA |  |
| - Display range: 0.00 nA to 9.99 nA, 9.0 nA to 99.9 nA, 90 nA to 999 nA, 0.90 μA to 9.99 μA, 9.0 μA to 99.9 μA, 90 μA to 999 μA (*4), 0.90 mA to 3.00 mA (*5) |  |
| - Measurement range (auto): 10 nA, 100 nA, 1000 nA, 10 μA, 100 μA, 1 mA |  |
| - Resolution: 0.01 nA, 0.1 nA, 1 nA, 0.01 μA, 0.1 μA, *4: 1 μA, *5: 0.01 mA |  |
| Voltage measurement |  |
| - Guaranteed accuracy range: 30 V to 1,000 V AC (45 Hz to 65 Hz), ±10 V to ±2,000 V DC |  |
| - Accuracy: ±3% rdg. ±3 dgt. |  |
| - Input resistance: 500 kΩ or more (DC, 45 Hz to 65 Hz) |  |
| - Overload protection: - 1100 V AC (1 min., between +/- terminals) - 2200 V DC (1 min., between +/- terminals) |  |
| Capacitance measurement |  |
| - Accuracy: ±10% rdg. ±5 nF (guaranteed accuracy range: 10.0 nF to 25.0 µF) |  |
| - Measurement range: 100 nF, 1000 nF, 10 μF |  |
| - Display range: 0.0 nF to 99.9 nF, 100 nF to 999 nF, 1.00 μF to 9.99 μF (*6), 10.0 μF to 25.0 μF (*7) |  |
| - Resolution: 0.1 nF, 1 nF, *6: 0.01 μF, *7: 0.1 μF |  |
| Necessary accessories |  |
| - TEST LEAD: (red), (black), (blue), each 3 m (9.84 ft.) - TEST LEAD: (red), (black), (blue), each 10 m (32.81 ft.) - ALLIGATOR CLIP: (red), (black), (blue) - TEST PIN SET: (red and black) - CARRYING CASE - WIRELESS ADAPTER - COMMUNICATION PACKAGE (USB) |  |

#### Digital Clamp Meter for measure the electrical current.

Technical Specifications

| - Functions: The clamp meter must be combines the functionality of a power meter, harmonics meter, and data logger in one easy-to-use package that meets the requirements of BSEN 61010-1 CAT IV 600 V for supply side applications. It has a large, backlit LCD screen that clearly displays all measurements that can be selected at the touch of a button or when rotating the selection dial. It also features auto-ranging and has three-phase rotation indication and automatically switches off to save battery consumption. |  |
| --- | --- |
| - AC/DC voltage to 1000 V |  |
| - AC/DC current to 1000 A |  |
| - True RMS |  |
| - Resistance 100 kΩ |  |
| - Bluetooth link to the Megger Link app |  |
| - Phase rotation indication |  |
| - Continuity Beeper |  |
| - Data Hold/Min-Max/Peak Hold |  |
| - Bright LED torch |  |
| - CAT IV 600 V Rating |  |
| - Supplied with carry case and Carrying holster/pouch |  |
| - Maximum Conductor Size: 40 mm |  |
| - Maximum Busbar Size:61 x 12mm |  |
| - Power Requirement: AAA Size Battery 1.5V x 6 |  |
| - Temperature: -50°C – 1000°C |  |
| - Included: Digital Clamp Meter; Carrying Case; Instructions; Test Leads; Thermocouple; Batteries; with all accessories. |  |

#### Digital Multimeter Tester.

Technical Specifications

| - Functions: Measures AC AC/DC Voltage; Resistance; Continuity; Battery Test; Finds Breaker to Corresponding Receptacle; Shows Amount of Time it Takes to Trip GFCI; Detects and Identifies Common Wiring Problems in GFCI |  |
| --- | --- |
| - Features: Data Hold; Auto-Range with Option for Manual-Range; Min/Max Values; Backlight |  |
| - Material: ABS and Electrical Components |  |
| - Batteries: 2 x AAA |  |
| - Display: Digital, 4000 Count Backlit LCD |  |
| - Voltage Maximum range: 1000V AC/DC |  |
| - Voltage Maximum resolution: 0.1 mV AC/DC |  |
| - Current Maximum range: 10 A AC/DC |  |
| - Current Maximum resolution: 0.01 mA AC/DC |  |
| - Frequency: 50 Hz to 60 kHz |  |
| - Ohms (Resistance): 50 MΩ |  |
| - Standard: CAT III 600V, Class 2, Double insulation |  |
| - Temperature Range: -20 °C / 300 °C |  |
| - IP54 environmental protection |  |
| - Included: Digital Multimeter; Right-Angle Test Leads; Receptacle Test Cord; Batteries; User Manual; Storage Pouch; with all accessories. |  |

### ITEM #6: Plumber / Pipe Fitter Toolkits

#### Pressure Gauge Stainless Steel Case, NS 100 [4"].

| - Standard: EN 837-1; ASME B40.100 |  |
| --- | --- |
| - Nominal size (NS): Ø 100 mm [4"] |  |
| - Connection location: Lower mount (radial) |  |
| - Material: Window: Laminated safety glass or Polycarbonate Case: Stainless steel Ring: Bayonet bezel, stainless steel |  |
| - Display range min.: 0 bar |  |
| - Display range max.: 25 bar |  |
| - Process connection: G 1/2 B |  |
| - Ambient temperature min.: -20 °C |  |
| - Ambient temperature max.: 60 °C |  |
| - Medium temperature: 100 °C |  |
| - Accuracy class: 1 |  |
| - Liquid filling: Yes |  |
| - Material of the process connection: Stainless steel 316L |  |
| - Protection rating: IP65 |  |

#### Heavy Duty Pipe Wrench

Technical Specifications

| - Functions: Pipe Wrenches are hand tools designed to turn pipe and tighten and loosen threaded pipe connections with round threaded fittings. |  |
| --- | --- |
| - Sturdy ductile iron housing for long lasting performance |  |
| - I-beam handle with full floating forged hook jaw |  |
| - Self-cleaning threads with replaceable hook and heel jaws |  |
| - Complies with federal specifications GGG-W65IE, Type II, Class A |  |

#### Adjustable Wrenches

Technical Specifications

| - Material: - High-performance alloy steel forged in single piece - Chrome plated, precision-hardened and anticorrosion treated |  |
| --- | --- |
| - Parallel jaws placed at a 15° angle provide a firm grip and best reach |  |
| - Wide handle for comfort in use |  |
| - Accurate jaw movement |  |
| - Laser scale in mm for pre-adjusting and accurate measurement |  |
| - Standards: ISO 6787, DIN 3117, ASME B107.8-2003 and BS 6333 |  |

#### Hacksaw Frame

Technical Specifications

| - Heavy-duty steel frame |  |
| --- | --- |
| - Aluminum + ABS + TPR handle, Strong and Comfortable |  |
| - Adjustable tension |  |
| - Heavy-duty steel frame |  |
| - Size:300mm/12" |  |
| - Including 6-pcs 65Mn saw blade |  |

#### Toolbox 3 Levels Empty

Technical Specifications

| - Size:495x200x295mm |  |
| --- | --- |
| - 3 layers |  |
| - Steel thickness：0.6~0.8mm |  |
| - Heavy-duty steel |  |

### ITEM #7: Safety supplies

#### First AID Kit

Technical Specifications

| - First Aid Supplies includes (Mandatory Content): First aid box, Sterile adhesive bandages in assorted sizes, Assorted sizes of safety pins, Cleansing agent/soap, Latex gloves (2 pairs), 2-inch sterile gauze pads (4-6), 4-inch sterile gauze pads (4-6), Triangular bandages (3), Non-prescription drugs, 2-inch sterile roller bandages (3 rolls), 3-inch sterile roller bandages (3 rolls), Scissors, Tweezers, Needle, Moistened towelettes, Antiseptic, Thermometer, Tongue depressor blades (2), Tube of petroleum jelly or other lubricant.. |  |
| --- | --- |

#### 5.0KG CO2 Fire Extinguisher

Technical Specifications

| - APPLICATION: - Suitable for B Class Fires (flammable liquid fires such as oils, paints and solvents) - Suitable for E Class Fires (energized electrical fires such as switchboards, electrical motors and electronic equipment) - Specifications: - CAPACITY: 5.0 Kg - FIRE RATING: 5B: E - AGENT: CO2 - DISCHARGE TIME: 14 Seconds - PRODUCT DIMENSIONS: (H X CYL Ø) 670 Mm X 150 Mm - OVERALL MASS: 12.4 Kg - CYLINDER: Lightweight Aluminum, UV Powder Coat - HANDLE: Stainless Steel - VALVE: Chrome Plated Brass - HOSE: EPDM, Alloy Hose Retaining Clip |  |
| --- | --- |

#### Personal Protective Equipment (PPE)

Technical Specifications

| Safety Shoes: - Specifications - PU artificial leather upper, PU dual density sole. non-steel toe - Collar: PU padded with heavy duty sponge. Lining: perfect soft mesh fabric lining - Height: Middle Cut - Sizes: 40,41,42,43,44 - Product Information: - Water resistant, - Slip resistant, - Antistatic - Penetration resistant - Oil resistant Outsole - Composite Midsole with Safety Toe cap |  |
| --- | --- |
| Goggles - Specifications: - Weight: Should weigh approx 1.8 ozs - Widest lens: Should have ellipsoidal lens that extends the viewing range further, even peripherally. - Style: Should not be bulky and should have a contoured, low-profile design. - Comfort: Should have a soft PVC shroud that gently hugs the face and is indirect vented to allow cooling air flow and help prevent fogging. - Adjustable: Should be a pantascoptic goggle with three lens angle adjustments. |  |
| Safety helmet for workers - Product Information: - Shell should be dome shaped, no metallic components are to be used. Brim diameter should be 6mm, peak 50mm.Diameter of the hole for ventilation should not exceed 6mm and maintained at a distance not less than 15mm from adjacent hole. Number of holes should be more than 3 but the average area of all holes should not exceed 300mm2. The headband should be more than 30mm in width. Cradle should be secured at minimum of four anchoring points. Straps should be anti- concussion and width not less than 19 mm. Straps should be fitted such that minimum clearance be at least 30 mm and maximum clearance more than 80 mm. - Chin strap should have minimum width of 19 mm and directly attached to shell. Nape strip should have minimum depth of 115 mm. - The mass of helmet without attachments should be 400 g. If it is to exceed 400 g, with nearing 35 g, it should be labelled. |  |
| Leather Gloves - Product Information: - Good quality chrome tanned cow/young buffalo split leather as per IS - shall be used for palm, finger's back and front cuff. - Leather for back cuff and reinforcement pieces shall be dyed in brown colour and grain printed. - The leather used for palm, finger's back and the front cuff shall be of natural colour - The thick ness of the leather shall be from 1.5 to 1.8 mm. - The nylon thread pf sewing cotton 60 S/6 (Variety No. 31 of IS) shall be used. - The gloves shall be machine stitched and the number of stitches shall be per cm - It shall have the double line stitching at reinforcement. |  |
| Work overalls - Product Information: - Product Type: Overalls - Material: Cotton - Wear Type: Not Reflective - Gender: Men - Size: All Sizes, Large, Small, Medium - Color: Navy Blue - Sleeves: Full or Half sleeve - Specification: - 100 % Cotton 245 GSM - 2 chest pockets with one flap. - Front heavy duty YKK metal zipper 2 way. - Metal Button on hand Cuff. - Tool Pockets. - Two open pockets at the back. - Elastic at the back. - Hammer loop optional. |  |

Delivery requirements and Comparative Data Table:

| UNOPS Requirements |  | Is quotation compliant? Bidder to complete | Details Bidder to complete |
| --- | --- | --- | --- |
| Delivery time | The Bidder shall deliver the goods within 6 months after the Contract signature. | ☐ Yes ☐ No | Insert details |
| Delivery place and Incoterms rules | DAP as per Incoterms 2020 at National Water & Sanitation Authority – HASWIN Branch - NWSA- HASWIN’s Warehouses, HASWIN District - Al Maharah Governorate. | ☐ Yes ☐ No | Insert details |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- [20%] , without any change in the unit prices or other terms and conditions of the RFQ. | ☐ Yes ☐ No | Insert details of goods offered, including specifications, picture, and brand/model offered if applicable |