Section II: Schedule of Requirements

| IUS-AF2-UWS-GAYD-006- PROVIDING SPARE PARTS FOR THE MAINTENANCE OF WATER TRANSMISSION PIPELINES AND DISTRIBUTION NETWORKS THAT WERE DAMAGED BY HURRICANE TEJ IN AL GHAYDAH CITY - AL MAHARAH GOVERNORATE |  |  |  |
| --- | --- | --- | --- |
| Supply, Deliver, Testing and Training of welding machines (HDPE Pipe Butt Fusion Jointing Machine) with silent mobile Diesel Generator compatible for operation. To Be Delivered to National Water & Sanitation Authority – Al Ghaydah Branch - NWSA- AL-GHAYDAH’s Warehouses, Al Ghaydah District - Al Maharah Governorate |  |  |  |
| No. | Description | Unit | Qty. |
| 1 | HDPE Pipe Butt Fusion Welding Machine |  |  |
| 1.1 | Supply, Testing and Deliver of Welding Machines (HDPE Pipe Butt Fusion Jointing Machine) and all necessary accessories as per required and according to Technical Specifications | Set | 1 |
| 2 | (9 – 10) KVA Quiet Diesel Mobile Generator |  |  |
| 2.1 | Supply, Testing and Deliver of (9 – 10) KVA Quiet Diesel Mobile Generator, according to the Technical Specifications. | Set | 1 |

SCOPE OF WORK

The sub-project is to Providing Spare Parts for The Maintenance of Water Transmission Pipelines and Distribution Networks That Were Damaged by Hurricane Tej in Al Ghaydah City with All Accessories for NWSA – Al Ghaydah Branch.

- GENERAL

- The goods to be supplied must be new and unused.

- Delivery and Test of supplied goods of supplied goods will be made for National Water & Sanitation Authority – Al Ghaydah Branch (NWSA-AL GHAYDAH) - NWSA- AL-GHAYDAH’s Warehouses, Al Ghaydah District - Al Maharah Governorate

- The material to be supplied shall be new, unused and of the latest design (refurbished, used Material or the stock for more than 3 years will not be considered)

- Documentary evidence to prove that the material offered comply with the Technical Specifications given in (Section II and Section III Form C) must be provided.

- The supplied goods and materials shall conform to the authoritative standards appropriate to the Goods country of origin. Such standards shall be the latest issued by the institution concerned.

- Technical Data/Brochures: bidders Must provide relevant technical data detailing the technical data of the material and original manufacturer brochures, where there is more than one model on a brochure, the bidder, must highlight which they are quoting for in this tender.

- All materials shall be complying with specified International Standards and shall be supplied from reputable and approved manufactures and country of origin.

- Markings:  Unless otherwise specified in the relevant standard, products are to have the following legible marks as appropriate, cast, stamped or indelibly painted:

-  Manufacturer’s name, initials and identification mark.

-  Class designation.

-  Date of manufacture.

-  Initials and number of relevant standards.

- Transport, handle and store all products and materials in accordance with the manufacturer’s recommendations and in a manner that prevents damage or deterioration or excessive distortion.

- The employer UNOPS, UWSSP / NWSA- Al Ghaydah Branch shall have the right to reject any damaged or defective product material.

- Warranty & training,

- Recommended Preventive Maintenance Schedule for the equipment shall be included.

- Final test and Inspection of supplied goods shall be carried out including: (Visual Inspection, Load test, testing, Material test certificate, field testing, etc) shall be done in the destination place (Al Ghaydah city/ Al-Mahara Government).

- One-week operation and maintenance training for the technical crew and operators of the unit shall be conducted upon delivery at supplier`s cost.

- One-year warranty for the whole unit. Warranty must be approved from the Bidder and the Manufacturer.

- Warranty of after sell service support for the unit shall be available through authorized services centre in the Yemen.

- Climate:

- All materials and equipment incorporated in the work shall be capable of withstanding shade temperatures of 45°C and if exposed to direct sunlight shall be capable of withstanding the additional heat gain up to 70°C , approx. 90% humidity and attitude 300 m above sea level. All materials and equipment shall take into account the generally tropical conditions with occasional dust storms, intense rain storms.

Technical specifications:

A.: Welding Machines (HDPE Pipe Butt Fusion Jointing Machine)

Product Description

- For welding PE / PP / PVDF

- Consists of basic frame, hydraulic unit, planning tool, heating plate, support stand for planning tool & heating plate, and optional parts.

- Removable PTFE coated heating plate with high accurate temperature control system

- Electrical planning tool.

- Be made of lightweight and high strength material, simple structure, sturdy & user friendly.

- Low starting pressure ensures reliable welding quality of small pipes.

- Suitable to weld fittings

Product Specification

| - Welding Type: Butt Fusion |  |
| --- | --- |
| - Power Consumption: 5 Kw |  |
| - Phase: Single Phase |  |
| - Usage/Application: Welding Machine |  |
| - Type: Automatic |  |
| - Temperature Deviation in Surface: +/- 5 % | ` |
| - Temperature Range: 50 To 300 Degree C |  |
| - Weight: < 200 kg |  |
| - Voltage: 220 V |  |
| - Frequency: 50 Hz |  |
| - Clamping: 4 |  |
| - Suitable for Pipe Size: 160, 180, 200, 225, 250, 280 & 315 mm |  |
| - Driven Type: Electric Motor |  |

B.: (9 – 10) KVA Quiet Diesel Mobile Generator

Technical Specification

- High quality, reliable and complete power unit

- Compact design

- Easy start and maintenance

- Fully tested for load, controls and safety shutdown functions

- Rated Power: (9 – 10) KVA

- Voltage: 220 Single-Phase or 400/230v 50 Hz

- Frequency: 50 Hertz

- Speed (RPM) 1500 rpm

- Auto Voltage Regulation

- Governing Type: Mechanical

- Fuel injection pump: Individual type

- Diesel Engine

- Water cooled

- Alternator: Brushless

- Insulation system:  Class H

- Integral 5A Battery Charger

Metering and Alarm indications:

- Generator frequency

- Under speed, Overspeed

- Generator volts (L-L, L-N)

- Generator current

- Engine oil pressure

- Engine coolant temperature

- Hours run counter

- Battery volts

- Fail to start/stop

- Emergency stop

- Charge fails

- CAN diagnostics and CAN fail/error

Delivery requirements and Comparative Data Table:

| UNOPS Requirements |  | Is quotation compliant? Bidder to complete | Details Bidder to complete |
| --- | --- | --- | --- |
| Delivery time | The Bidder shall deliver the goods within 6 months after the Contract signature. | ☐ Yes ☐ No | Insert details |
| Delivery place and Incoterms rules | DAP as per Incoterms 2020 at National Water & Sanitation Authority – Al Ghaydah Branch - NWSA- AL-GHAYDAH’s Warehouses, Al Ghaydah District - Al Maharah Governorate. | ☐ Yes ☐ No | Insert details |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- [20%] , without any change in the unit prices or other terms and conditions of the RFQ. | ☐ Yes ☐ No | Insert details of goods offered, including specifications, picture, and brand/model offered if applicable |