Section II: Schedule of Requirements

| IUS-AF2-UWS-GAYD-006- PROVIDING SPARE PARTS FOR THE MAINTENANCE OF WATER TRANSMISSION PIPELINES AND DISTRIBUTION NETWORKS THAT WERE DAMAGED BY HURRICANE TEJ IN AL GHAYDAH CITY - AL MAHARAH GOVERNORATE |  |  |  |
| --- | --- | --- | --- |
| Supply, Deliver and Testing of HDPE Pipes, Fittings and Valves of various diameters, complete with all required connections, accessories, to be delivered to National Water & Sanitation Authority – Al Ghaydah Branch’s Warehouse. |  |  |  |
| No. | ITEMS | Unit | Qty. |
| 1 | SUPPLY, DELIVER AND TESTING OF HDPE PIPES, FITTINGS AND VALVES OF VARIOUS DIAMETERS, COMPLETE WITH ALL REQUIRED CONNECTIONS, ACCESSORIES |  |  |
| 1.1 | HDPE Pipes according to Technical Specifications and according to the following dimensions: |  |  |
| 1.1.1 | Ø 400mm, PE100, SDR 11, PN16, (12m Pipe Length) | m | 120 |
| 1.1.2 | Ø 315mm, PE100, SDR 11, PN16, (12m Pipe Length) | m | 120 |
| 1.1.3 | Ø 250mm, PE100, SDR 11, PN16, (12m Pipe Length) | m | 120 |
| 1.1.4 | Ø 200mm, PE100, SDR 11, PN16, (12m Pipe Length) | m | 120 |
| 1.1.5 | Ø 160mm, PE100, SDR 11, PN16, (12m Pipe Length) | m | 240 |
| 1.1.6 | Ø 110mm, PE100, SDR 11, PN16, (100m roll Pipes Length) | m | 400 |
| 1.1.7 | Ø 90mm, PE100, SDR 11, PN16, (100m roll Pipes Length) | m | 400 |
| 1.1.8 | Ø 63mm, PE100, SDR 11, PN16, (100m roll Pipes Length) | m | 400 |
| 1.1.9 | Ø 32mm, PE100, SDR 11, PN16, (100m roll Pipes Length) | m | 400 |
| 1.1.10 | Ø 25mm, PE100, SDR 11, PN16, (100m roll Pipes Length) | m | 400 |
| 1.2 | Compression Fittings for HDPE Pipes according to Technical Specifications |  |  |
| 1.2.1 | Coupling, Compression Fittings, PN16, according to the following dimensions: |  |  |
| 1.2.1.1 | DN 110mm | No. | 20 |
| 1.2.1.2 | DN 90mm | No. | 20 |
| 1.2.1.3 | DN 63mm | No. | 100 |
| 1.2.1.4 | DN 32mm | No. | 250 |
| 1.2.1.4 | DN 25mm | No. | 250 |
| 1.2.2 | Reducing Coupling, Compression Fittings, PN16, according to the following dimensions: |  |  |
| 1.2.2.1 | DN 110mm x DN 90mm | No. | 20 |
| 1.2.2.2 | DN 90mm x DN 63mm | No. | 20 |
| 1.2.2.3 | DN 63mm x DN 32mm | No. | 50 |
| 1.2.2.4 | DN 63mm x DN 25mm | No. | 50 |
| 1.2.2.5 | DN 32mm x DN 25mm | No. | 100 |
| 1.2.3 | Male Adaptor (mm x inch) BSP male, Compression Fittings, PN16, according to the following dimensions: |  |  |
| 1.2.3.1 | DN 63mm x 2” | No. | 20 |
| 1.2.3.2 | DN 32mm x 1” | No. | 100 |
| 1.2.3.3 | DN 25mm x ¾” | No. | 100 |
| 1.2.4 | Equal Tee, Compression Fittings, PN16, according to the following dimensions: |  |  |
| 1.2.4.1 | DN 100mm | No. | 10 |
| 1.2.4.2 | DN 90mm | No. | 10 |
| 1.2.4.3 | DN 63mm | No. | 50 |
| 1.2.4.4 | DN 32mm | No. | 100 |
| 1.2.4.5 | DN 25mm | No. | 100 |
| 1.3 | HDPE Electrofusion Fittings for HDPE Pipes according to Technical Specifications |  |  |
| 1.3.1 | HDPE Electrofusion Coupling, PE100, SDR11, PN16, according to the following dimensions: |  |  |
| 1.3.1.1 | DN 400mm | No. | 10 |
| 1.3.1.2 | DN 315mm | No. | 10 |
| 1.3.1.3 | DN 250mm | No. | 10 |
| 1.3.1.4 | DN 200mm | No. | 10 |
| 1.3.1.5 | DN 160mm | No. | 10 |
| 1.4 | Restraint Coupling & Flange Adaptor (used for HDPE) according to Technical Specifications |  |  |
| 1.4.1 | Restraint Coupling, according to the following dimensions: |  |  |
| 1.4.1.1 | DN 300mm | No. | 10 |
| 1.4.1.2 | DN 250mm | No. | 10 |
| 1.4.1.3 | DN 200mm | No. | 10 |
| 1.4.1.4 | DN 150mm | No. | 20 |
| 1.4.1.5 | DN 100mm | No. | 20 |
| 1.4.1.6 | DN 80mm | No. | 20 |
| 1.4.2 | Restraint Flange Adaptor, according to the following dimensions: |  |  |
| 1.4.2.1 | DN 300mm | No. | 10 |
| 1.4.2.2 | DN 250mm | No. | 10 |
| 1.4.2.3 | DN 200mm | No. | 10 |
| 1.4.2.4 | DN 150mm | No. | 20 |
| 1.4.2.5 | DN 100mm | No. | 20 |
| 1.4.2.6 | DN 80mm | No. | 20 |
| 1.5 | Flanged Gate Valve according to Technical Specifications |  |  |
| 1.5.1 | Flanged Gate Valve, according to the following dimensions: |  |  |
| 1.5.1.1 | DN 400mm | No. | 4 |
| 1.5.1.2 | DN 300mm | No. | 6 |
| 1.5.1.3 | DN 250mm | No. | 6 |
| 1.5.1.4 | DN 200mm | No. | 6 |
| 1.5.1.5 | DN 150mm | No. | 10 |
| 1.5.1.6 | DN 100mm | No. | 10 |
| 1.5.1.7 | DN 80mm | No. | 10 |

SCOPE OF WORK

Supply, delivery and Test of 3 Submersible pumps units (bowel assembly, Motors, Cables, Control panels, etc.) complete with all required connections, gaskets, bolts and accessories, to be delivered to National Water & Sanitation Authority – Al Ghaydah Branch- NWSA- AL-GHAYDAH’s Warehouses, Al Ghaydah District - Al Maharah Governorate, for the purpose of maintaining submersible pumping units for a number of water wells in Al-Jazaa water well field

- GENERAL

- The goods to be supplied must be new and unused.

- Delivery and Test of supplied goods of supplied goods will be made for National Water & Sanitation Authority – Al Ghaydah Branch (NWSA-AL GHAYDAH) - NWSA- AL-GHAYDAH’s Warehouses, Al Ghaydah District - Al Maharah Governorate

- The material to be supplied shall be new, unused and of the latest design (refurbished, used Material or the stock for more than 3 years will not be considered)

- Documentary evidence to prove that the material offered comply with the Technical Specifications given in (Section II and Section III Form C) must be provided.

- The supplied goods and materials shall conform to the authoritative standards appropriate to the Goods country of origin. Such standards shall be the latest issued by the institution concerned.

- Technical Data/Brochures: bidders Must provide relevant technical data detailing the technical data of the material and original manufacturer brochures, where there is more than one model on a brochure, the bidder, must highlight which they are quoting for in this tender.

- All materials shall be complying with specified International Standards and shall be supplied from reputable and approved manufactures and country of origin.

- Markings:  Unless otherwise specified in the relevant standard, products are to have the following legible marks as appropriate, cast, stamped or indelibly painted:

-  Manufacturer’s name, initials and identification mark.

-  Class designation.

-  Date of manufacture.

-  Initials and number of relevant standards.

- Transport, handle and store all products and materials in accordance with the manufacturer’s recommendations and in a manner that prevents damage or deterioration or excessive distortion.

- The employer UNOPS, UWSSP / NWSA- Al Ghaydah Branch shall have the right to reject any damaged or defective product material.

- Warranty

- Recommended Preventive Maintenance Schedule for the equipment shall be included.

- Final test and Inspection of supplied goods shall be carried out including: (Visual Inspection, Load test, testing, Material test certificate, field testing, etc) shall be done in the destination place (Al Ghaydah city/ Al-Mahara Government).

- One-year warranty for the whole unit. Warranty must be approved from the Bidder and the Manufacturer.

- Warranty of after sell service support for the unit shall be available through authorized services centre in the Yemen.

- Climate:

- All materials and equipment incorporated in the work shall be capable of withstanding shade temperatures of 45°C and if exposed to direct sunlight shall be capable of withstanding the additional heat gain up to 70°C , approx. 90% humidity and attitude 300 m above sea level. All materials and equipment shall take into account the generally tropical conditions with occasional dust storms, intense rain storms.

- Markings:

Unless otherwise specified in the relevant standard, products are to have the following legible marks as appropriate, cast, stamped or indelibly painted:

- Manufacturer’s name, initials and identification mark.

- Class designation.

- Date of manufacture.

- Initials and number of relevant standards.

Technical specifications:

1.1.:  HDPE Pipes and Fittings.

HDPE pipes and Fittings should conform to:

- DIN EN ISO 178 – (or equivalent international standards) Determination of the Flexural Properties of Plastics

- DIN EN ISO 527-2 – (or equivalent international standards) Determination of tensile properties of plastics – Test Conditions for Moulding and Extrusion Plastics

- DIN 7728–1 – Plastics – (or equivalent international standards) Symbols for Polymers and Their Special Characteristics

- DIN 8074/ ISO 4427 - (or equivalent international standards) Polyethylene (PE) Pipes Dimensions

- DIN 8075/ ISO 4427 - (or equivalent international standards) Polyethylene (PE) Pipes General Quality Requirements and Testing

- DIN EN ISO 12162 – (or equivalent international standards) Thermoplastic Materials for pipes and Fittings for Pressure Applications – Classification, Designation and Overall Service (design) Coefficient

- DIN 16963 – (or equivalent international standards) Pipe Joint Assemblies and Fittings for High – Density Polyethylene (PE-HD) Pressure Pipes

- DIN 53752 – (or equivalent international standards) Determination of Coefficient of Linear Thermal Expansion of Plastics

- HDPE pipes shall be: - Pressure Rating: PN 16

- HDPE Pipes shall be grades (PE. 100)

- HDPE Pipes Minimum Required Strength (MRS) 10 Mpa.

- The density (PE 100) should not be less 0.95 ̴ 0.965 g/cm

Testing of Pipes and Fittings:

  - The pipe and fitting manufacturer shall have an established quality control program responsible for inspecting incoming and outgoing materials.  Incoming polyethylene materials shall be inspected for density, melt flow rate, and contamination.  The cell classification properties of the material shall be certified by the supplier.  Incoming materials shall be approved by Quality Control before processing into finished goods.

  - Outgoing products shall be tested as required in DIN Standards.

  - The Supplier shall inspect all pipe and accessories for shortages, loss, or damage upon receipt of the shipped material at the time of unloading, recording this information directly on the waybill received from the carrier.

  - The pipe shall contain no recycled compound.

Marking for Pipes and Fittings:

Generally marking the pipes have to be marked according to DIN 8074 No.8.  Intervals not exceeding 4m

| Item | Description | NOTE |
| --- | --- | --- |
|  | Pipes |  |
| 1 | Manufacturer's name or trade marks |  |
| 2 | Material designation Standard PE code designation (e.g PE100) |  |
| 3 | Applicable Standards |  |
| 4 | Dimensions, mm (nominal diameter (DN), wall thickness) |  |
| 5 | date of manufacture |  |
| 6 | PN, SDR values |  |
|  | Fittings |  |
| 7 | Manufacturer's name or trade marks |  |
| 8 | Material designation Standard PE code designation (e.g PE100) |  |
| 9 | Applicable Standards |  |
| 10 | Dimensions, mm (nominal diameter (DN), wall thickness) |  |
| 11 | date of manufacture |  |
| 12 | PN, SDR values |  |
| 13 | Angle (Degrees) | just for Bend |

The pipes and fittings shall be rejected, if   the   above   information   for marking is not shown clearly.

1.2.:  P.P. Compression Fittings.

Description

Compression Fittings must be designed specifically for connecting polyethylene pipes with an outside diameter from 16 to 110mm. They are compatible with PELD, PEHD, PE40, PE80 and PE100 pipes complying with AS / NZS 4130 Series 1.  They are normally used to convey drinking water and fluids at pressures up to 16 bar for generic applications. The quality of the materials used makes these fittings resistant to etching by numerous chemical substances, and to UV-rays.

Technical specifications

- Used: Suitable for use with all types of polyethylene pipes PE 80, PE 100, PEX and PVC-U.

- Standards:

|  | For PE Pipes: | DIN EN 12201; ISO 11922-1; DIN 8074; EN 12201-1/2. |
| --- | --- | --- |
|  | Threads: | DIN EN 10226 (ISO 7-1); BS 21; AS 1722.1. |
|  | Flanges: | DIN EN1092-1; DIN 2501.1. |
|  | Test Standards: | ISO 17885; DIN 8076-3; AS/NZS 4129. |

- Suitable for Drinking Water.

- Nominal Pressure:	PN16.

- Materials of Compression Fittings:

| 1. | Body: | Polypropylene, high-stability copolymer (PP-B) with UV protection |  |  |
| --- | --- | --- | --- | --- |
| 2. | Seal (lip gasket): | Food-safe EPDM rubber suitable for potable water. |  |  |
| 3. | Bush Ring: | Polypropylene, high-stability copolymer (PP-B) with UV protection. |  |  |
| 4. | Clamping Ring: | Acetalic resin (POM) |  |  |
| 5. | Nut: | Polypropylene, high-stability copolymer (PP-B) with UV protection. |  |  |
| *The threaded end (for female thread) is reinforced with a Stainless-steel ring (AISI 430) |  |  |  |

1.3.:  HDPE Electrofusion Fittings

Description

The HDPE Electrofusion fittings have a resistive wire inside which is connected to the outside cable terminals.

When voltage is applied, this resistance generates the heat needed to melt polyethylene. Energy is directly transmitted at the interface between the fitting and the pipe causing heat welding of the parts. When it cools down, the joint is even, strong, safe and reliable.

Technical specifications

- Used: Suitable for use with all types of polyethylene pipes PE 100

- Electrofusion Fittings are manufactured from a PE 4710/PE100

- Standards:

|  | Test Standards: | EN ISO 1043-1 and EN ISO 1043-2 EN ISO 1872-1 and EN ISO 1872-2 EN ISO 4613-1 and EN ISO 4613-2 EN ISO 1133-1 and EN ISO 1133-2 EN ISO 178 EN ISO 527-1 and EN ISO 527-2 EN ISO 16770 EN ISO 12086 EN ISO 1872-1 and EN ISO 1872-2 EN 12201-1 to EN 12201-5 (for polyethylene piping systems) |
| --- | --- | --- |

| - Suitable for Drinking Water. - Nominal Pressure: PN16. - Pin Connectors: 4 mm - Marking: The Marking and labeling All components are embossed with a permanent identify cation during the production process to ensure full traceability: - Brand name - Material - Dimensions - Article number - Pressure rating |  |
| --- | --- |

1.4.:  Restraint Coupling & Flange Adaptor.

Description

Restraint Coupling & Flange Adaptor are suitable for use in potable water and Wastwater pipelines, the Restraint Coupling & Flange Adaptor are designed to provide full restraint connectivity for polyethylene pressure pipes.

Technical specifications

- Technical Data

|  | Standards: | EN 12842 |
| --- | --- | --- |
|  | Working Pressure: | PN16 |
|  | Flanges: | EN1092-2; 2 dimensions AS 4087 PN 16 drill pattern |
|  | Angular Deflection: | Not Less Than 1.5 deg. |

- Materials:

| 1. | End Ring: | Ductile Iron, FBE Coated |  |
| --- | --- | --- | --- |
| 2. | Grip Ring: | Copper Alloy |  |
| 3. | Bolts: | 316 SSS – Delta Seal Coating |  |
| 4. | Elastomeric Seals: | EPDM |  |
| 5. | Barrel: | Ductile Iron, FBE Coated |  |

1.5.: Flanged Gate Valve.

Description

Flanged gate valve for drinking water and neutral liquids to max. 70°C

Technical specifications

- Technical Data

|  | Standards: | Designed according to EN 1074 part 1 & 2 |
| --- | --- | --- |
|  | Working Pressure: | PN16 |
|  | Flanges: | EN1092-2 (ISO 7005-2), PN10/16 |
|  | Face-to-face dimension: | EN 558 Table 2 Basic Series 3. |

- Materials:

| 1. | Wiper ring | NBR rubber |  |
| --- | --- | --- | --- |
| 2. | O-ring | NBR rubber |  |
| 3. | O-ring | NBR rubber |  |
| 4. | Bushing | Polyamide |  |
| 5. | Thrust collar | Brass, DZR CW602N |  |
| 6. | Stem seal | EPDM rubber |  |
| 7. | Stop ring | Stainless steel 316L |  |
| 8. | Stem | Stainless steel 316L |  |
| 9. | Bonnet bolt | Stainless steel A4 |  |
| 10. | Bonnet | Ductile iron GJS-500-7 (GGG-50) |  |
| 11. | Bonnet gasket | EPDM rubber |  |
| 12. | Wedge nut | Brass, DZR CW626N |  |
| 13. | Wedge rubber | EPDM rubber |  |
| 14. | Body | Ductile iron GJS-500-7 (GGG-50) |  |
| 15. | Wedge core | Ductile iron GJS-500-7 (GGG-50) |  |
| 16. | Wedge shoe | Polyamide |  |

Delivery requirements and Comparative Data Table:

| UNOPS Requirements |  | Is quotation compliant? Bidder to complete | Details Bidder to complete |
| --- | --- | --- | --- |
| Delivery time | The Bidder shall deliver the goods within 6 months after the Contract signature. | ☐ Yes ☐ No | Insert details |
| Delivery place and Incoterms rules | DAP as per Incoterms 2020 at National Water & Sanitation Authority – Al Ghaydah Branch - NWSA- AL-GHAYDAH’s Warehouses, Al Ghaydah District - Al Maharah Governorate. | ☐ Yes ☐ No | Insert details |
| UNOPS Right to vary requirements | At the time the Contract is awarded, UNOPS reserves the right to vary the quantity of the goods and associated services specified above, provided this does not exceed +/- [20%] , without any change in the unit prices or other terms and conditions of the RFQ. | ☐ Yes ☐ No | Insert details of goods offered, including specifications, picture, and brand/model offered if applicable |