| Item | Descriptions | Unit | Quantity | Unit Price | Total |
| --- | --- | --- | --- | --- | --- |
| 1 | PV modules 105.4kwp Supply and install for all materials, connect and operate monocrystalline photovoltaic solar modules with all material needed to have complete job ready for installing high quality PV modules with total array capacity of (circa) 100kwp at the UNICEF offices in Dar es Salaam. Example or equivalent Solar panels 310w Mono Crystaline technology 25 year linear warranty / 10 year product warranty Dimensions 991cm x 1650cm | 310w | 336pcs |  |  |
| 2 | 25,000w Solar inverters that syncronise energy to existing grid Including Remote monitoring kit inc 5 year fully inclusive warranty “Zero feed in” equipment (to not feed power into Generator or TANESCO) | 25kw | 5pcs |  |  |
| 3 | Solar mounting structure | Unit | 1pc |  |  |
| 4 | Earthing for solar mounting structure | Unit | 1pc |  |  |
| 5 | DC cabling from panels to inverters | Meters | 200 |  |  |
| 6 | DC cable MC4 connectors, fixings, cable ties, 1 inch conduits (per string) etc | Unit | 1 |  |  |
| 7 | Cable combiner box 5pcs 16mm 4C + earth into 1pc 70mm 4C + earth including outdoor enclosure | Unit | 1 |  |  |
| 8 | AC armoured copper cabling from inverters to combiner 16mm | No. | 100m |  |  |
| 9 | AC earth copper cable from inverters to combiner 16mm | No. | 150m |  |  |
| 10 | AC armored copper cable from combiner to energy meter (between transformer and gen ATS) 70mm 4C | No. | 150m |  |  |
| 11 | AC earth copper cable from combiner to energy meter 70mm | No. | 150m |  |  |
| 12 | Cable trays 150 * 100mm and materials to fix from PV array to solar inverters | No. | 150m |  |  |
| 13 | Bus bar chamber for 70mm cable to connect to existing Tanesco - Generator ATS cable including outdoor enclosure | Unit | 1 |  |  |
|  | TOTAL BILL (US $) |  |  |  |  |